3GPP TSG RAN4 Meeting #117							R4-2520370

Dallas, USA, Nov. 17th-21st November 2025

Agenda item:		8.3

Source:	SES, MITRE, JSAT, Thales, Lockheed Martin, Gatehouse, Gilat, Airbus

Title:	Multi-Orbit NTN Solution for 6G

Document for:	Discussion and Decisions

# Introduction

In this contribution we would like to introduce the concept of Multi-Orbit Connectivity Non-Terrestrial Networks (NTN) solution, as part of the 6G Study

Since 3GPP Release 17 Non-Terrestrial Networks (NTN) have been established as supplementary network solutions for UEs and NB-IoT devices and together with the conventional Terrestrial Networks (TN) they offer a ubiquitous connectivity solution over land and over water, added resiliency to network failures, and offer best-of-both service quality.

In our view this ubiquitous coverage can be further extended to provide seamless connectivity between Terrestrial and Non-Terrestrial network.

In our view, the next evolution in the NTN connectivity solution would be the Multi-Orbit NTN solution, whereby a UE or a VSAT can connect to various satellites in various orbits, as illustrated in the following Figure 1.  Figure 1 illustrates GEO, MEO and LEO orbits.

Figure 1 Multi-Orbit NTN Connectivity

By leveraging the strengths of satellites at various orbital altitudes, this solution can potentially create a robust and versatile solution that enhances overall performance, enabling high-speed internet access, improved coverage, enhanced communication, and resilient services.

With the growing expansion of the communication services and ever-growing needs for data connectivity, Multi-Orbit NTN solution is becoming essential to remain competitive. In addition to the technological enhancements, the Multi-Orbit NTN solution has commercial advantages for operators who operate NGSO and GSO constellations.

In this concept, delay sensitive applications can be routed to LEO constellations, while non-delay sensitive applications can be routed to GEO; essentially load balancing the application data traffic using multi-orbit constellations.

Additionally, with this multi-orbit approach, legacy satellites can be used for fall-back resiliency, which may also include non-3GPP satellite systems. Multi-Orbit NTN solution also offers services resilience as even if one satellite fails, the network can switch services through another satellite.

Furthermore, the Multi-Orbit NTN solution would offer a type of Relay or backhaul between the LEO constellation for the Direct-to-Device services.

In our view the work previously completed for NTN-TN and vice versa can be leveraged to extend the solution for multi-orbit connectivity by way of new signalling procedures and possibly new SIB(s).

Although at first glance the Multi-Orbit NTN solution would involve devices, such as the VSATs etc., nevertheless in our view we need to have a holistic view of the solution, as the solution could involve architectural enhancements to accommodate the Multi-Orbit NTN solution.

Observations 1: Multi-Orbit NTN solution would contribute to a ubiquitous and robust connectivity.

# Topics to be Considered for Multi-Obit NTN Solution

As part of the 6G Study, the mobility solution needs to be extended to accommodate not only Terrestrial to Non-Terrestrial Networks mobility, but also handover and mobility between various satellites, namely NTN-to-NTN mobility, in various orbits, see following Figure 2

Figure 2 Mobility and Handover Scenarios in a Multi-Orbit NTN Solution

## Mobility and Handover Scenarios

With the multi-orbit scenario, we envisage the following mobility scenarios such as

- Scenario 1 Terrestrial Cell to/from LEO Cell

- Scenario 2 Terrestrial Cell to/from GEO Cell

- Scenario 3 LEO Cell to/from GEO Cell

- Scenario 4 GEO Cell t/from GEO Cell

- Scenario 5 LEO Cell to/from LEO Cell

In Figure 2. Scenarios 3-5 would require further analysis.

Although not shown MEO cells could also be part of the Mobility Scenarios. Other handover scenarios could be possible.

Additionally for the NTN-NTN mobility solution, the connected mode mobility would require further analysis.

It should be noted that in realistic deployment scenarios, the networks involved in these mobility scenarios (terrestrial networks, GEO and LEO constellations) may be operated by different operators. The handover procedures and signalling mechanisms would need to accommodate such multi-operator scenarios.

Observation 2: For the Multi-Orbit NTN solution, new mobility and handover scenarios need to be considered, and solution sought.

## Mobility Triggers

Several new mobility triggering is to be analysed for the Multi-Orbit NTN solution,

For the VSAT to be handed from one cell to another certain network condition must occur for the VSAT to transition to another cell, these conditions are either RF/Signal based or are set by the network. These triggers are as follow:

Signal Level Based

The VSAT handover from weak to strong cell based on RSRP/RSRQ values. Hand-out and hand-in solution was completed in Rel17 and Rel18 respectively.

Location Based

The VSAT handover is triggered by VSAT’s location on the ground. This solution is GPS based, and it is currently being developed in Rel19 by RAN2.

See following Figure 6

Figure 3 Handover Based on Signal Strength and Location Based

Time Based Trigger

The handover trigger is based on the length of time a VSAT has remained in a particular cell.

Load Balancing Trigger

The VSAT is pushed to another less-congested cell due to cell congestion.

Service Based Trigger

Service Based: The VSAT is pushed to another cell due to service requirements.

User Priority Trigger

Mission-critical users have higher priority.

Emergency service users have higher priority over regular users.

Gold customers have priority over regular users,

Regulatory Triggers

VSAT is triggered to handover to a new cell due to Regional Regulatory Requirements

Elevation Angle Based Triggers

NGSO satellite elevation above the ground triggers handover to another cell

Observation 3: With the Multi-Orbit NTN solution, additional handover triggering mechanism would need to be considered.

## Large Number of Satellite Cells

With the advent of the High Throughput Satellites and GEO Satellites with Narrow Beams, the number of satellite cells on the ground would increase.

Furthermore, by considering the MEO and LEO Cells, the number of satellite cells on the ground and seen by the VSAT would increase with the Multi-Orbit NTN solution.

See following Figure 4

Figure 4 Large Number of Satellite Cells Available to the VSAT

Observations 4: with the Multi-Orbit NTN solution, large number of satellite cells would be in VSAT’s neighbouring list.

## Service Anchor

In the mobility solutions that have been developed in Rel17 and Rel18 Specifications, whereby the UE is handed out and handed back in, the service anchor has been the Terrestrial Network, and all the signalling was ‘anchored’ in the terrestrial 5G core. Essentially the mobility signalling was managed by the terrestrial 5G Core. In this “legacy” solution the Non-Terrestrial Network is serving as a complementary service to the Terrestrial Network/

Considering a practical maritime situation where the UE or VSAT is switched on in the middle of the ocean, away from any terrestrial coverage, it would require registering to a GEO or NGSO constellation for the initial service access; in this scenario the 5G Core associated with the satellite constellation would act as the service anchor and all signalling would be routed vis  the satel5G Core.

With the Multi-Orbit NTN solution, this service anchor would be in the satellite’s 5G Core Network whereby the UE/VSAT is handed between satellite constellations and terrestrial network.

Observation 5: A Non-Terrestrial Service Anchor need to be considered for a Multi-Orbit NTN solution.

## Non-Terrestrial Service Anchor (NTSA) for Multi-Orbit Operation

In Multi-Orbit NTN solutions, it is necessary to consider configurations where the service anchor resides in the Non-Terrestrial Network itself. In such configurations, one or more satellite 5G cores act as the service anchor to support service continuity when UEs/VSATs operate outside terrestrial coverage.

Further study could be necessary in RAN2 to analyse RRC and mobility signalling procedures related to:

· Initial access and registration when the service anchor is purely non-terrestrial.

· Mobility handling between orbits when the anchor resides in the NTN core, including scenarios of inter-NTN core handover.

· Coordination with terrestrial anchors for hybrid coverage situations.

· Procedures to ensure session continuity and stable QoS when a UE transitions from terrestrial to NTN service anchoring, or between NTN anchors.

Note that these studies above may require collaboration with RAN3 and SA2 for RAN interface(s) impact and core aspects.

# Areas of Further Study

The study would focus on two VSAT connectivity scenarios:

- Scenario 1: VSAT with single connectivity

- Scenario 2: VSAT with two connectivity

To progress with the Multi-Orbit NTN solution, we envisage the following topics to be studied by various working groups:

RAN2 Topics

- Additional signalling for cell selection and re-selection between GEO, MEO and LEO cells (GEO + LEO, GEO + MEO, GEO + GEO)

- New information in SIB(s) to assist the NTN-NTN handover (between different orbit)

- How to address a potential large number of neighbouring/overlapping Cells

- Measurement time to detect larger number of neighbouring cells.

- Signalling enhancements to minimize handover interruption time and enable RACH-less handover.

- Develop signalling for new handover scenarios and triggers (see section 2.2 )

- Study of seamless inter-orbit switching mechanisms for service continuity across GEO, MEO, and LEO systems. This includes mobility management procedures enabling Make-Before-Break (MBB) or conditional handover solutions between orbits while maintaining minimal service interruption.

- Study of simultaneous multi-orbit connectivity where a UE/VSAT maintains concurrent RRC connections with multiple satellite layers.

  - It may build on existing Dual Connectivity (DC) principles, defining Master and Secondary Nodes across orbits, and may require new RRC configurations for activation/deactivation, measurements, and coordination procedures. But other mechanisms are not excluded.

- Study and clarify assumptions and requirements for whether the Multi-Orbit NTN solution operates on a single-connectivity or multi-connectivity basis, in coordination with RAN4 for Doppler, frequency, and other RF alignment aspects.

- Consider NTSA. RAN2 to do it in coordination with RAN3 and SA2

RAN4 Topics

- Provisioning for seamless and uninterrupted handover between GEO to MEO/LEO, Dopler and Timing Alignment

- Frequency offset tracking.

- Doppler Tracking

- Differential Dopler Compensation for VSATs at beam edge

- Time and Frequency synchronisation

Othe features to study:

- Simultaneous Connectivity between GEO, MEO and LEO satellite access links during handover

- Multi-operator deployment scenarios and associated handover procedures

# Observations

Observations 1: Multi-Orbit NTN solution would offer ubiquitous and robust connectivity.

Observation 2: For the Multi-Orbit NTN solution, new mobility and handover scenarios need to be considered, and solution sought.

Observation 3: With the Multi-Orbit NTN solution, additional handover triggering mechanism would need to be considered.

Observations 4: with the Multi-Orbit NTN solution, large number of satellite cells would be in VSAT’s neighbouring list.

Observation 5: A Non-Terrestrial Service Anchor need to be considered for a Multi-Orbit NTN solution.

# Conclusion and Proposal

As part of technological evolution towards 6G, NTN has been identified as the one of the key components to provide ubiquitous connectivity. It follows that to enhance this ubiquitous connectivity; Multi-Orbit NTN solution is of interest for enhanced network resiliency.

To date, mobility and handover solution have been provided between TN and NTN networks, and the solution is transferable to the Multi-Orbit NTN solution. However, to realise the complete Multi-Orbit NTN solutions further work, and analysis would be required in 3GPP.

Proposal: RAN4 to consider General RF aspects of the “Multi-Orbit” NTN solution as part of the 6G Study

# References

- RP-251318