3GPP TSG-RAN WG4 Meeting # 117											R4-2520833

Dallas, USA, 17 – 21 November, 2025

Agenda item:			6.12.1

Source:	Moderator (Samsung)

Title:	Topic summary for [117][208] NR_MIMO_Ph5_RRM_Part1

Document for:	Information

Introduction

This topic summary covers the contributions submitted under the following AI for part of Rel-19 RRM for NR MIMO Phase 5:

- 4.18	NR MIMO Phase 5

- 4.18.2 RRM core requirements

- 6.12	NR MIMO Phase 5

- 6.12.2 RRM performance requirements

  - 6.12.2.1 Enhancement for UE-initiated/event-driven beam management

Recommended issues to online discussion:

Issue 2-1

Issue 1-2

Issue 1-1

Issue 2-2

Topic #1: RRM core requirements maintenance

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520471 | Nokia | Observation 1: Current specification does not define starting and ending point for timer-based reporting. Observation 2: If report is triggered by a restarted timer, event condition may have become met already during the previous timer, in which case the agreed timer-based reporting delay may not be sufficient to cover the time from event condition becoming met until the first PUCCH. Proposal 1: When eventDetectionTimeWindowLength-r19 is configured, the event triggered L1-RSRP measurement reporting delay is defined as the time between an event that will trigger a measurement report and the point when the UE transmits first PUCCH over the air interface. a.If a timer is already running at the time of the event that will trigger a measurement report, the time until start of the timer during which the report is triggered is added in the event triggered measurement reporting delay Observation 3: timeRestrictionForChannelMeasurements in CSI-ReportConfig applies to the RS/beam measurements, i.e., either SSB or CSI-RS measurements, that are reported by the UE. Observation 4: In the current version of the specifications, timeRestrictionForChannelMeasurements in a UEIBM CSI-ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR: - applies to the new beams, i.e., to the RSs/beams included in high layer parameter resourcesForChannelMeasurement; - does not always apply to the current beam. Proposal 2: Capture as proposed in our companion CR that the value of M determined for the new beams by timeRestrictionForChannelMeasurements in a UEIBM ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR, is also used for the current beam. |
| R4-2521914 | Ericsson | Proposal 1: RAN4 to agree on one of the following. a.If current beam is configured in multiple ReportConfig, then UE shall select the lowest M value for the measurement delay of the event triggered reporting. |

Open issues summary

Sub-topic 1-1

- Issue 1-1: Further update reporting delay requirements for the window-based case?

- Proposals

  - Option 1: (Nokia)

    - (CR R4-2520472). If the reporting condition becomes met after restarting the timer for the beam to be reported, the reporting delay is the time from when the event condition is met for the first time till the PUCCH transmission, which also includes windows during which the reporting condition was not fulfilled.

  - Option 2: (Huawei)

    - (CR R4-2522320). Add low boundary of the measurement reporting delay.

- Recommended WF

  - TBA

- Issue 1-2: Whether to add explicit restriction for current beam and new beams for “timeRestrictionForChannelMeasurement” in RAN4 spec?

- Proposals

  - Option 1: (Nokia)

    - (CR R4-2520472) Yes. The value of M determined for the new beams by timeRestrictionForChannelMeasurements in a UEIBM ReportConfig, i.e., a CSI-ReportConfig including high layer parameter eventTypeUE-IBR, is also used for the current beam.

  - Option 2: (Ericsson)

    - (CR R4-2521915) If current beam is configured in multiple ReportConfig, then UE shall select the lowest M value for the measurement delay of the event triggered reporting.

- Recommended WF

  - TBA

Sub-topic 1-2: Draft CRs

| T-doc number | Company |  |
| --- | --- | --- |
| R4-2520472 | Nokia | DraftCR for UEIBM reporting delay |
| R4-2520627 | Apple | CR on UEIBR Requirements |
| R4-2522320 | Huawei, HiSilicon | CR on the core part requirement of MIMO UE-initiated/event-driven beam management for Rel-19 |
| R4-2521095 (Resubmission; endorsed in RAN4#116bis) | vivo | CR on core requirements maintenance for Rel-19 MIMO UEIEBM |
| R4-2521915 | Ericsson | Draft CR for event-triggered reporting for MIMO |
| R4-2522096 (Resubmission; endorsed in RAN4#116bis) | Nokia | CR MTTD requirements for UL only TRP |
| R4-2522097 (Resubmission; endorsed in RAN4#116bis) | Nokia | CR on transmit timing requirements for asymmetric DL sTRP/UL mTRP deployment |

Topic #2: RRM performance requirements for UE-initiated/event-driven beam management

Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520475 | Nokia | Proposal 1: Introduce test cases for FR1 and FR2, Event-2, Intra-cell SSB with window time based and eventInstanceCount-r19 > 1. |
| R4-2520946 | Huawei, HiSilicon | Observation 1: The core part defined the requirement that the measurement report is limited to no larger than the length of the window. Nothing mentioned that UE is forced to maintain the measurement during the whole window. Proposal 1: Revising the core part requirement as if eventDetectionTimeWindowLength-r19 is provided, the event-triggered measurement reporting delay shall be no larger than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel and shall be no shorter than (eventInstanceCount-r19 + L) * TL1-meas_basic + Tfirst UL channel. Proposal 2: If the minimum measurement report delay has been introduced, test case 8: Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 is able to be defined. |
| R4-2521916 | Ericsson | Proposal 1: For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state). Proposal 2: For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state). Proposal 3: RAN4 to specify the periodic first PUCCH resource in the test configuration as follows. |
| R4-2522023 | Samsung | Proposal 1: For two FFS test cases, RAN4 to only add one test case for TC8. |

Open issues summary

Sub-topic 2-1: Performance requirements for UE-initiated/event-driven beam management

[Background]: In last meeting, there are FFS for two test cases.

| TC index | TC name | Company name |
| --- | --- | --- |
| TC1 | Event-2, FR2, Inter-cell SSB based, no window time based, mode-A | Samsung |
| TC2 | Event-2, FR2, CSI-RS based, no window time based, mode-A, with ”QCL-info” for the SSBs with the same cell ID | vivo |
| TC3 | Event-2, FR2, Intra-cell SSB based, no window time based, mode-A | Ericsson |
| FFS on TC4 | Event-2, FR2, FFS Intra-cell SSB or CSI-RS based, with window time based, and eventInstanceCount-r19 > 1 |  |
| TC5 | Event-2, FR1, Inter-cell SSB based, no window time based, mode-A | CMCC |
| TC7 | Event-2, FR1, Intra-cell SSB based, no window time based, mode-A | Nokia |
| FFS on TC8 | Event-2, FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1 |  |

- Issue 2-1: Whether to add test case(s) for window time based case?

- Proposals

  - Option 1: (Nokia)

    - Add two TCs for:

      - FR2, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1

      - FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1

  - Option 2: (Huawei (only if core req. update), Samsung)

    - Add one TC for FR1, Intra-cell SSB based, with window time based, and eventInstanceCount-r19 > 1

  - Option 3:

    - No test cases.

- Recommended WF

  - TBA

- Issue 2-2: Test configuration

- Proposals

  - Option 1: (Ericsson)

    - For the SSB based event triggered reporting, RAN4 to define the test cases by considering the Scheme-2 (RS for the current beam is the SSB which is QCLed with the QCL RS in the indicated TCI state).

    - For CSI-RS based test case design, RAN4 to define the test cases by considering the Scheme-1 (RS for the current beam is the QCL RS in the indicated TCI state).

    - RAN4 to specify the periodic first PUCCH resource in the test configuration as follows

| Parameter |  | Value |
| --- | --- | --- |
| BWP ID |  | 0 |
| PUCCH-Cell |  | SpCell |
| periodicityAndOffset |  | Sl5 |
| PUCCH resource ID |  | 0 |
| PUCCH resource | Starting PRB | To be determined by RAN5 |
|  | intraSlotFrequencyHopping | disabled |
|  | Format | format 2 |
|  | nrofPRBs | 2 |
|  | nrofSymbols | 1 |
|  | startingSymbolIndex | 0 |

- Recommended WF

  - TBA

Sub-topic 2-2: Draft CRs

| T-doc number | Company |  |
| --- | --- | --- |
| R4-2520204 | Samsung | Draft CR on test case of Event-2, FR2, Inter-cell SSB based, no window time based, mode-A |
| R4-2520464 | CMCC | DraftCR on TC Event-2, FR1, Inter-cell SSB based, no window time based, mode-A (TC5) |
| R4-2520476 | Nokia | DraftCR TC for Intra-cell SSB-based UEIBM reporting in FR1 |
| R4-2521096 | vivo | DraftCR on TC2 for Rel-19 MIMO UEIEBM CSI-RS based event-2 |
| R4-2521917 | Ericsson | TC on Event-2, FR2, Intra-cell SSB based, no window time based, mode-A |