3GPP TSG-RAN WG4 Meeting #116-bis	R4-2522226

Dallas, USA, 17th – 21st November 2025

Agenda item:			7.1.1

Source:	Moderator (Nokia)

Title:	Topic summary for [117][316] NR_UE_RF_Ph5

Document for:	Information

# Introduction

This topic summary covers the discussion for Rel-20 NR_UE_RF_Ph5, approved under WID [1]. The following agenda items covered 7.1.2, and 7.1.4, i.e., general aspects and HPUE requirements. The approved workplan is presented in [2].

The submissions focus on discussions and initial views regarding High Power UE (HPUE) requirements, including RF enhancements, PC1.5 implementations, and multi-antenna (4Tx/2Tx) configurations for NR and FDD technologies. Multiple vendors contributed insights on refining HPUE parameters and addressing specific RF challenges in evolving network scenarios.

# Topic #1: 4Tx with up to 32dBm MOP

PC1 4Tx for following TDD bands n41, n77/n78, n79, and n104 and PC1.5 2Tx for following FDD bands n1 and n25.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520090 | CATT | Proposal 1: the nominal output power for 4Tx TDD bands with 4x26dBm PA should be 32dBm. In addition, since the exemplary bands n41, n77 and n78 can supported PC1 31dBm in the latest specification, it may cause some NBC issue or ambiguity if RAN4 just simply reuse the existing ‘PC1’ indication. Proposal 2: 2dB upper tolerance is proposed for 32dBm nominal output power. Proposal 3: As 32dBm+2dB tolerance with 4x26dBm PA can achieve 34dBm output power, RAN4 can request RAN2 to extend the range of P-Max without any NBC issue. |
| R4-2521142 | China telecom | Proposal1: 4Tx total output power should be 32dBm with +2/-3 tolerance. We can define large MPR value for 4Tx if the 32dBm simulation result is negative. Proposal2: For the wider application of 4x26dBm PA UE, UE should both support ULFPTx mode1 and mode2. Proposal3: Discuss whether and how introducing full power transmission mode reporting corresponding to current PC for 4Tx UE in Rel-20. |
| R4-2522170 | Ericsson | Observation 1: the (band-specific) modified MPR behaviour can be used to distinguish PC1 support for the example bands of the 4 x 26 dBm CPE devices with their relaxed ACLR requirement and associated MPR from legacy PC1 devices. Observation 2: ULFPTx ‘Mode-0’ is not feasible for 32 dBm power class. Observation 3: ULFPTx ‘Mode-1’ is feasible for 32 dBm power class for both single- and two-layers cases. Observation 4: ULFPTx ‘Mode-2’ is not feasible for 32 dBm power class. Observation 5: It should be discussed how to capture the ULFPTx ‘Mode-1’ only support in the specifications. Observation 6: ‘Mode-full power’ can be introduced for PC2 under 4 x 26 dBm PA configuration assumption. It should be discussed whether it can be introduced in this WI. Based on the discussion in the previous sections we propose the following: Proposal 1: Specify ACLR of 33 dBc for 32 dBm nominal maximum output power. Proposal 2: a new power class should not be introduced - PC1 should be used with power boosting capability of 1 dB. Proposal 3: Specify only ULFPTx ‘Mode-1’ for 32 dBm power class. |
| R4-2522135 | Huawei | Observation 1: Existing MPR requirements for PC1.5 with 4Tx are >0dB. It does not prevent a practical UE from reaching the MOP (29dBm). Observation 2: MPR for 2Tx could be smaller than that for 4Tx as seen from PC1.5. It’s possible that 2x29dBm architecture (from 2026Q4) could have 0dB MPR relative to 32dBm MOP. Proposal 1: The MOP for 4x26dBm is defined as 32dBm, and there’s no need to wait for the exact ACLR number. Proposal 2: It’s better to define a new power class for 4x26dBm with new set of RF requirements including MOP, ACLR, MPR and etc. Proposal 3: Follow PC1.5 with 4Tx, study and define MPR requirements for both 10dB and 20dB antenna isolation for 4x26dBm architecture. Proposal 4: Follow Rel-18 agreement that supporting 4Tx is release independent from Rel-18. |
| R4-2520823 | MediaTek | Proposal 1: If RAN4 define different power class with 32dBm FWA device, then PC1bis or other designated power class would be considered to distinguish different maximum output power level between 31dBm and 32dBm. Proposal 2: To derive MPR requirements for 4x26dBm (up to 32dBm) FWA device, RAN4 can define delta value on top of the existing MPR requirements in Table 6.2D.2-5. RAN4 can consider single antenna isolation with 20dBm for both CPE and FWA. Proposal 3: For A-MPR requirements for 4x26dBm (up to 32dBm) FWA device in n41, RAN4 can define delta value on top of the existing A-MPR requirements for PC1 in clause 6.2.3.2. For the other example NR bands, RAN4 do not need to define A-MPR requirements. Proposal 4: If RAN4 consider 31dBm with PC1 for (4x26dBm) FWA device, then the delta value is [1dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41. If RAN4 consider 32dBm with PC1BIS for (4x26dBm) FWA device, then the delta value is [1.5dB] on top of the existing A-MPR requirements of PC1.5 UE for NS_04 and NS_47 for n41. Proposal 5: For configured Tx power, RAN4 can further discuss how to treat the SAR regulatory limits for FWA device. Other Tx requirements can be reused the existing Tx RF requirements except ACLR requirements for FWA 32dBm device. Proposal 6: For ACLR requirements for FWA 4x26dBm (up to 32dBm) output power device, RAN4 will decide the ACLR requirements based on SLS coexistence evaluation results to protect legacy systems. Proposal 7: For FWA REFSENS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 can reuse the existing 2Rx REFSENS requirements. RAN4 also will support 4Rx and 8Rx FWA REFSENS requirements with the additional delta values in Table 7.3.2-2 and Table 7.3.2-2a. Proposal 8: For ACS requirements for FWA 4x26dBm (up to 32dBm) device, RAN4 will decide the ACS requirements based on SLS coexistence evaluation results to protect legacy systems. Other Rx requirements can be reused for FWA 32dBm device. |
| R4-2521403 | Nokia | Proposal 1: If Average throughput loss and 5%-ile throughput loss should be less than 5% method for co-ex is chosen then define new power class 1Bis for 32 dBm. Proposal 2: Wait for further agreements on power class and need for additional signalling before deciding release independence. |
| R4-2520263 | Qualcomm | Observation 2-1: From a power control perspective, the network does not need to distinguish between a 31 dBm UE and a 32 dBm UE of equivalent configuration. Observation 2-2: A 4x26 dBm UE poses no new UE configuration challenge compared to legacy 5G UEs. Observation 2-3: From a network management perspective, a 4x26 dBm does not need a new power class to be defined for it. Proposal 2-1: No new power class is needed for the 4x26 dBm UE, PC1 can be re-used. Observation 2-4: Any potentially elevated power expectation (like 32 dBm) from the 4x26 dBm UE can be captured in D-suffix requirements. Observation 2-5: A 4x26 dBm UE would need to be specified with a reduced max. output expectation of 3 dB for t2rN and 6 dB for t1rn SRS transmissions. Proposal 2-2: The target 4x26 dBm UE is specified with PPowerClass of 3 and 6 dB for SRS-TxSwitch capability of ‘t2rN’ and ‘t1rN’ respectively for SRS transmissions with usage ‘antennaSwitching’. Proposal 2-3: The Rel-19 TRxSRS framework is re-used for the target 4x26 dBm UE. Specification for additional SRS-TxSwitch capabilities is not precluded. Observation 2-6: The target 4x26 dBm UEs must be enabled for ‘power class requirement fall back’ behavior to enable the use of legacy PC2 PAs. Proposal 2-4: A new capability like ‘maxUplinkDutyCycle-PC2-FR1’ is defined for the 4x26 dBm UE to enable use of legacy PC2 PAs and to allow ‘power class requirement fallback’ behavior when indicated capability is exceeded by scheduling. Proposal 2-5: RAN4 to request RAN2 to enhance the CA power class list to include the power class chosen for the target 4x26 dBm UE. |
| R4-2520732 | Vivo | Proposal 1: It is suggested to reuse PC1 with or without general power boosting scheme to achieve up to 32 dBm in Rel-20. Proposal 2：It is suggested to only consider Mode-1 for 4Tx with 4*26dBm PA configuration for uplink full power transmission. Observation 1：No rational manufacturer would design hardware capable of PC1 certification, deliberately downgrade it to PC2. Proposal 3: Do not consider mode-full power mode for PC2 with 4x26dBm PA configurations. Proposal 4: Follow the WID and only consider the exemplary TDD bands for 4Tx PC1. |
| R4-2520770 | ZTE | Observations: Observation 1: 4Tx PC1 NTN HPUE has been introduced in Rel-19 and 34dB ACLR was specified. Observation 2: A new power class with 32dBm maximum output power will impact current RAN2 specification. Observation 3: UE is not allowed to always transmit with maximum output power because of power backoff and network configuration. 4Tx with 4x26dBm PA configuration will cause more serious reverse IMD compared to legacy UE, which may further limit UE’s output power. Proposals: Proposal 1: Define 4Tx CPE/FWA UE with 4x26dBm PA configuration as PC1 UE with 31dB maximum output power and +2/-3dB tolerance. Proposal 2 : If 32dBm output power is feasible, treat the potential 1dB more output power as power boosting. Proposal 3: Output power dynamic, Transmit signal quality and Output RF spectrum emissions for UL MIMO and Tx diversity can keep unchanged for HPUE with up to 32dBm MOP. Proposal 4: MPR/A-MPR requirements for UL MIMO and Tx Diversity can be evaluated once the power class is decided. Proposal 5: 4Tx HPUE with 4x26dBm PA configuration could indicate ULFPTx Mode-1 to achieve 31/32dBm MOP. ULFPTx Mode-2 can be further considered if other PA configurations are introduced in the future. Proposal 6: There is no impact on Rx requirements when introducing HPUE with up to 32dBm MOP. |
| R4-2520290 | Skyworks | Proposal on 4x26dBm PA power class: - Assuming ≥15dB antenna isolation and up to a 34dB ACLR target, the 31dBm PC1 power class can be reused with a single MPR table. - PC1 4Tx can be merged for TN and NTN - To account for smaller inner that may reach 32dBm but avoid specifying a new power class only for a limited number of allocations: - Either a 1dB boosting or 1dB extra margin at the top of the power class range can be added. - To avoid significant work in defining the allocations that can be boosted, the latter is preferred - Inner waveforms are 0dB MPR for DFT-s-OFDM and 1.5dB MPR for CP-OFDM - Based on our measurements for full allocation waveforms, with the above assumptions, outer waveforms need 2.5dB MPR for DFT-s-OFDM and 4.5dB MPR for CP-OFDM - Edge allocations are dominated by the digital filtering/windowing, thus, the edge allocation definition and value from 1Tx PC1 can be reused. - If 37dB ACLR is required, all MPR should be increased by 1dB. |

## Transmitter power

Sub-topic description: With 4 26 dBm output power PAs, the total output power can be 32 dBm. Current PC1 output power is 31 dBm with +2/-3 tolerance.

Issue 1-2-1: Maximum output power

- Proposals

  - Option 1: 31 dBm +2/-3 tolerance (ZTE)

  - Option 2: 32 dBm +2/-3 tolerance (CATT, China Telecom, Ericsson, Huawei, Nokia)

  - Option 3: 31 dBm +3/-3 tolerance (Skyworks)

- Recommended WF

  - TBD.

Issue 1-2-2: Power class

- Proposals

  - Option 1: Use PC1 (Qualcomm, Vivo, ZTE, Skyworks)

  - Option 2: PC1 with 1dB power boosting (Ericsson)

  - Option 3: PC1Bis (CATT, Huawei, MediaTek, Nokia)

- Recommended WF

  - TBD.

Issue 1-2-3: PPowerClass

- Proposals

  - Option 1: 3 for ‘t2rN’ and 6 dB for ‘t1rN’ in SRS transmission with ‘antennaSwitching’ (Qualcomm)

- Recommended WF

  - TBD.

Issue 1-2-4: TRxSRS

- Proposals

  - Option 1: Reuse the R19 framework (Qualcomm)

- Recommended WF

  - TBD.

Issue 1-2-5: SAR regulatory limits

- Proposals

  - Option 1: Discuss how to handle SAR regulatory limits (MediaTek)

  - Option 2: A new capability for uplink duty cycling is needed (Qualcomm)

- Recommended WF

  - TBD.

## Emissions

Issue 1-3-1: ACLR value

- Proposals

  - Option 1: 33 dB (Ericsson)

  - Option 2: Derive based on co-existence simulations (MediaTek)

  - Option 3: 34 dB (Skyworks)

- Recommended WF

  - Discuss in the co-existence session.

## MPR/A-MPR

Sub-topic description: MPR and A-MPR assumptions need to be agreed before stating the simulation work.

Issue 1-4-1: Antenna isolation

Moderator comment: Below is the relevant agreement from RAN4#116bis

Agreement: - Evaluate assuming following antenna isolation assumptions - For CPE, assuming both 10dB and 20dB, - For FWA, assuming 20dB. Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results.

Currently for PC1.5 with 4/2 Tx, MPR is defined with two different isolation assumptions, 10dB and 20dB.

- Proposals

  - Option 1: Adopt both 10 dB and 20 dB isolation assumption (Huawei)

    - Option 1-1: Capture in the specification via modifiedMPR-Behavior bits (Ericsson).

  - Option 2: Adopt 20db for both FWA/CPE. (MediaTek)

  - Option 3: ≥15dB (Skyworks)

- Recommended WF

  - TBD.

Issue 1-4-2: MPR

Moderator comment: Currently for PC1.5 with 4/2 Tx, MPR is defined with two different isolation assumptions, 10dB and 20dB.

- Proposals

  - Option 1: No need to simulate, have a single MPR requirement by adding a delta on top of PC1.5 4 Tx MPR requirements (Table 6.2D.2-5) (MediaTek)

  - Option 2: If 34 dB ACLR and ≥15dB antenna isolation, then following (Skyworks)

    - Inner waveforms are 0dB MPR for DFT-s-OFDM and 1.5dB MPR for CP-OFDM

    - For full allocation waveforms, 2.5 dB and 4.5 for outer waveforms for DFT-s-OFDM and CP-OFDM, respectively.

    - Edge allocation can reuse 1Tx PC1 MPR

  - Option 3: If 37 dB ACLR and ≥15dB antenna isolation, then proposed MPR values in Option 2 are increased by 1dB (Skyworks)

- Recommended WF

  - TBD.

Issue 1-4-3: A-MPR requirement for NS_04 and NS_47

- Proposals

  - Option 1: Add a delta on top of existing PC1.5 A-MPR for n41 (MediaTek)

    - Option 1-1: 1 dB for if MOP is agreed to be 31 dBm

    - Option 1-2: 1.5 dB if MOP is agreed to be 32 dBm

  - Option 2: Simulate instead of delta addition.

- Recommended WF

  - TBD.

## ULFPTx

Issue 1-5-1: Mode-1 and Mode-2 feasibility

- Proposals

  - Option 1: Both Mode-1 and Mode-2 are feasible (China Telecom)

  - Option 2: Only Mode-1 is feasible for both single and two layers (Ericsson, Vivo, ZTE)

- Recommended WF

  - TBD.

Issue 1-5-2: Mode-full power feasibility

- Proposals

  - Option 1: Not feasible irrespective of PC1/PC1bis decision (Ericsson, ZTE)

- Recommended WF

  - Agree that Mode-full power is not feasible.

## Rx requirements

Issue 1-6-1: REFSENS with 2Rx

- Proposals

  - Option 1: Reuse existing REFSENS requirement (MediaTek)

- Recommended WF

  - TBD.

Issue 1-6-2: REFSENS with 4Rx and 8Rx

- Proposals

  - Option 1: On top of 2Rx REFSENS, existing delta values for 4Rx and 8Rx are considered (MediaTek)

  - Option 2: No new delta values are needed (ZTE).

- Recommended WF

  - TBD.

Issue 1-6-3: ACS requirement

- Proposals

  - Option 1: Decide based on co-existence (MediaTek)

- Recommended WF

  - TBD.

Issue 1-6-4: Other Rx requirements

- Proposals

  - Option 1: No additional requirements need to be specified (MediaTek)

- Recommended WF

  - TBD.

## Other WGs

Issue 1-7-1: LS to RAN2

- Proposals

  - Option 1: Extend P-Max range to 34 (CATT)

  - Option 2: Extend CA power class list to include power class used for the target UE (Qualcomm)

- Recommended WF

  - TBD.

## Release independence

Issue 1-8-1: Release independence

- Proposals

  - Option 1: 4Tx is release independent from Rel-18 (Huawei)

  - Option 2: Wait for additional agreements before deciding (Nokia)

- Recommended WF

  - TBD.

## 4Tx WID applicability

Sub-topic description: WID does not say anything regarding PC2 or FDD bands support 4Tx for CPE/FWA UE with 4x26dBm PA configurations. The exemplary bands for this objective are TDD.

Issue 1-9-1: Mode-full power feasibility for PC2

- Proposals

  - Option 1: Mode-full power is feasible for PC2. Discuss whether it can be introduced in this WI. (Ericsson)

  - Option 2: No need to discuss Mode-full power for PC2. (Vivo)

- Recommended WF

  - TBA

Issue 1-9-2: 4Tx PC1/PC1bis requirements to be band agnostic

- Proposals

  - Option 1: Requirements only for the exemplary TDD bands (Vivo)

- Recommended WF

  - TBD.

# Topic #2: PC1.5 with 2Tx for FDD

PC1.5 2Tx for following FDD bands n1 and n25.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2520580 | Apple | Observation 1: For n1 and n25 the legacy antenna isolation assumption can be reused. Low frequency FDD bands require re-evaluation if enabled for PC1.5 in the future. If RAN4 adds PC1.5 to low frequency FDD bands in the future, the antenna isolation assumption should be carefully discussed. Observation 2: To define the RSD values due to PC3 PC1.5, RAN4 should leverage on the existing PC3PC2 RSD specifications so that consistency is maintained. Proposal 1: Capture this aspect in the specification by modifying the title of Table 6.2D.2-2 and add the requirement that it is only applicable to frequency bands above 1.7 GHz. Proposal 2: Consider the following methodology for deriving the values of RSD for PC3PC1.5: - Derive the interfering Tx noise from the existing PC2 RSD specifications - Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5 - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5 Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5 Proposal 3: Adopt the RSD requirements for n25 from PC3 to PC1.5 with 2Tx as shown in the Table below. Operating Band; 3 MHz (dB); 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n25; 3.6; 3.6; 3.7; 3.7; 3.9; 8.7; 8.9; 9.4; 9.8. |
| R4-2520090 | CATT | P4-1: RAN4 can specify AMPR for NS_03, NS_05 for PC1.5. After that, RAN4 can check whether the same PC1.5 AMPR for NS_03, NS_05 can be applicable to the NS_03U, NS_05U. P4-2: RAN4 can check whether AMPR requirements for NS_100 in Table 6.2.3.1-2 are applicable to PC1.5. Proposal 5: For 2Tx PC1.5 band n1, the RSD is zero for all the supported CBWs from PC3 to PC1.5 for UE supporting Tx Diversity. |
| R4-2522170 | Ericsson | Proposal 4: For the MPR requirements, reuse the current MPR requirements for PC1.5 with 2 Tx derived for TDD bands. Both tables should be included: one targeting handheld UEs and the other targeting UEs with large FWA form factor with 20 dB or above antenna isolation. Proposal 5: Adopt 0 dB as PC1.5 RSD requirement for band n1 due to a large duplexing distance. |
| R4-2522135 | Huawei | Proposal 6: It’s feasible to reuse the existing PC1.5 MPR requirements for 10dB and 20dB antenna isolation. Proposal 7: For UTRA ACLR, the same requirement for PC2 can be reused if no concerns from operators. Table 6.5.2.4.2-1: UTRA ACLR requirement Power class 1.5; Power class 2; Power class 3. UTRAACLR1; 33 dB; 33 dB; 33 dB. UTRAACLR2; 36 dB; 36 dB; 36 dB. Proposal 8: The 2Tx PC1.5 RSD for n25 can be defined as 2Tx PC2 RSD + 3dB. |
| R4-2521517 | LG Electronics | Proposal 1: Consider duplexer isolation factor for specifying the PC1.5 2Tx FDD RSD requirement as shown below table. CBW≤20 MHz; CBW> 20 MHz. n1; 50 dB; 50 dB. n25; 50 dB; 40 or 35 dB. Proposal 2: No need to consider antenna isolation factor for 2t-2r architecture assuming dominant interference from duplexer. |
| R4-2520823 | MediaTek | Proposal 9: RAN4 can reuse the existing MPR requirements in clause 6.2D.2 for the PC1.5 (2x26dBm) HHUE and FWA in TN FDD bands. Proposal 10: RAN4 can consider how to simplify the A-MPR requirements with delta value on top of the existing A-MPR requirements of PC2 UE for PC1.5 UE in Rel-20. Proposal 11: RAN4 need to define UTRA ACLR for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signalled. The UTRA ACLR level can be verified by coexistence evaluation in Rel-20. Proposal 12: For FDD PC1.5 (2x26dBm) UE, RAN4 shall further discuss how to comply the SAR regulatory limits when P-MPR solution is not applicable case (e.g., conducted test mode) considering with FDD PC2 maintenance progress. Proposal 13: RAN4 can consider that the RSD is 0.0 dB from PC3 to PC1.5 2Tx in n1. Table 2. Reference Sensitivity Degradation from PC3 to PC1.5 for n25 FDD band for UE supporting Tx Diversity Operating Band; 3 MHz (dB); 5 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n25; 1.5 +2; 1.5+2; 1.6+2; 1.6+2; 1.7+2; 6.0+2.5; 6.2+2.5; 6.7+2.5; 7.1+2.5. Proposal 14: RAN4 can consider the RSD levels in Table 2 from PC3 to PC1.5 2Tx in n25. |
| R4-2520224 | Murata | Observation 1: RAN4 spent much time and efforts to determine PC2 RSD and it is expected that many FDD bands supporting PC1.5 are to be introduced later Observation 2: There might be some inconsistency between the existing PC2 1Tx RSD and newly specified PC1.5 2Tx RSD if RAN4 follows the traditional way of determining PC1.5 2Tx RSD due to different inputs from different companies. Proposal 1: It is highly recommended to have a general methodology for deriving the PC1.5 2Tx RSD not only for RAN4 working efficiency but also for specification consistency. Observation 3: The general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 can be used not only for PC1.5 2Tx RSD calculation procedure simplification but also for consistency of spec Proposal 2: RAN4 to use the general PC1.5 2Tx RSD calculation methodology in Table 1 and 2 for PC1.5 2Tx RSD calculation Proposal 3: Use Table 3 for PC1.5 2Tx RSD of n25. Additionally, RAN4 to discuss further on the more general methodology, e.g. LUT basis. |
| R2-2521403 | Nokia | Observation 1: PC1.5 with 2Tx for TDD feature is already specified and those emission requirements can be extended into FDD operation. Observation 2: PC1.5 with 2Tx for TDD MPR maybe be extendedable for FDD operation. Proposal 3: 20 dB or larger antenna isolation should be used for FWA devices. Proposal 4: RAN4 shall discuss if 45 MHz needs to be studied for NS_03 A-MPR. Observation 3: Initial A-MPR results for NS_03 and NS_05 are presented in R4-2521305. |
| R4-2520263 | Qualcomm | Observation 3-1: Currently no agreement that 2Tx PC1.5 will not be deployed in the same regions as EUTRA in bands n1 and n25. Observation 3-2: Extra workload in supporting 2Tx PC1.5 for NS_03U and NS_05U is considerable Proposal 3-1: Companies should further study whether support of NS_03U and NS_05U and NS_100 is required for 2Tx PC1.5. Observation 3-3: UTRA ACLR requirement is applicable for NS labels NS_03U, NS_05U and NS_100. Proposal 3-2: If it is decided that NS_03U, NS_05U and NS_100 will be supported, evaluate whether new UTRA ACLR is required for PC1.5 with 2 Tx when NS labels NS_03U, NS_05U and NS_100 are signaled. Proposal 3-3: Analyze the applicability of existing MPR table for PC 1.5 with 2 Tx for a FDD handheld UE. Proposal 3-4: Analyze the applicability of existing MPR table for PC1.5 with 2 Tx for a FDD FWA UE. Observation 3-4: For a PC2 UE with Tx Diversity additional relaxation is provided in Note 7 of Table 3-5 (Table 6.2.3.1-1 in 38.101-1). Proposal 3-5: Evaluate whether further relaxation in A-MPR for PC1.5 with 2 Tx FDD architecture is required for a UE indicating Tx Diversity. Proposal 3-6: Update the modified MPR behavior table L.1-1 for bands n1 and n25 to support PC1.5 with 2 Tx for FDD handheld and FWA UEs. |
| R4-2520732 | Vivo | Observation 2：The general methodology for deriving RSD needs to consider interference coupling paths, e.g. the isolation of the duplexer, spatial coupling gain, Tx non-linear components. |
| R4-2520770 | ZTE | Proposal 7: The MPR requirements of PC1.5 2Tx requirements for TDD bands can be applied to FDD bands for PC1.5 UE with 2Tx. Proposal 8: It is proposed that RSD=0dB from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity in band n1. Proposal 9: To consider the the RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity as shown in Table 2. Table 2 RSD from PC3 to PC1.5 for PC1.5 UE supporting Tx Diversity Operating Band; 3 MHz (dB); 5 MHz (dB); 7 MHz (dB); 10 MHz (dB); 15 MHz (dB); 20 MHz (dB); 25 MHz (dB); 30 MHz (dB); 35 MHz (dB); 40 MHz (dB); 45 MHz (dB); 50 MHz (dB). n25; 2.4; 2.5; 2.5; 2.6; 2.8; 8.8; 9.1; 9.8; 10.2. Proposal 10: It is proposed to study a general approach to defined the RSD requirements from PC3 to PC1.5 for all PC1.5 2Tx FDD bands. |
| R4-2520290 | Skyworks | - The current 2Tx TDD PC1.5 MPR tables are reused for FDD bands where the UL is >1.6GHz - Unless antenna isolation <8dB or ACLR >31dB should be assumed for FDD band <1.6GHz, the current 2Tx TDD PC1.5 MPR tables are reused for all FDD bands. Proposal for 2Tx PC1.5 FDD RSD: - Only 2Tx RSD is specified for PC1.5 - RSD simplification is investigated in a similar way than for MSD simplification based on: - For PC3 reference, a 6dB main to diversity imbalance is assumed and based on smallest CBW ideal REFSENS scaling, from which the equivalent de-sense and UL interference is calculated - The UL interference is increased by 3dB compared to PC3 1Tx and applied on main and diversity branches for PC1.5 2Tx - i.e. , being a 2Tx case, the REFSENS is recalculated with a main to diversity imbalance of 0dB - As de-sense is indirect, there is no possibility for an LUT from PC3 REFSENS to HPUE RSD but de-sense can be calculated from the above assumptions and an PC1.5 LUT applied to derive the RSD. - FFS if a similar method should be applied to derive PC2 1Tx and 2Tx RSDs. - Another option is to provide an LUT for “additional RSD” from 2Tx PC2 to 2Tx PC1.5 - Given the large duplex distance, PC1.5 RSD should be minimal for Band n1 but important for Band n25 that has a significant de-sense for CBW>25MHz (both from REFSENS and PC2 2Tx RSD). |

## Leveraging TDD requirements

Sub-topic description: PC1.5 is already specified for TDD bands and for this objective the exemplary bands are FDD bands n1 and n25.

Issue 2-2-1: Reusing MPR for 2Tx PC1.5 from TDD bands

Moderator comment: There are two MPR tables for PC1.5 with 2Tx, assuming different isolation and form factor. One of those tables (6.2D.2-2) still has values in square brackets in version 19.3.0 of TS 38.101-1

- Proposals

  - Option 1: Yes (Ericsson, Huawei, MediaTek, Nokia, ZTE)

  - Option 2: Yes for FDD bands with UL > 1.6 GHz (Skyworks)

- Recommended WF

  - MPR tables 6.2D.2-2 and 6.2D.2-3 apply for exemplary bands. Discuss how to capture Skyworks concern.

Issue 2-2-2: Specification impact regarding MPR

- Proposals

  - Option 1: Update the modifiedMPR behavior table in Annex L.1-1 (Qualcomm)

- Recommended WF

  - TBD.

Issue 2-2-3: SAR regulatory limits

- Proposals

  - Option 1: Discuss how to handle when P-MPR does not apply (MediaTek)

- Recommended WF

  - TBD.

## A-MPR

Sub-topic description: Discussion on various network signalling values for the exemplary bands.

Issue 2-3-1: A-MPR requirement for NS_05, NS_48, and NS_49 for n1

- Proposals

  - Option 1: Add a delta on top of existing PC2 A-MPR for n1 (MediaTek)

  - Option 2: Simulate instead of delta addition.

- Recommended WF

  - TBD.

Issue 2-3-2: Additional relaxation for NS_05, NS_48, and NS_49 for n1

- Proposals

  - Option 1: Extend NOTE 7 in Table 6.2.3.1-1 [TS 38.101-1] i.e., additional relaxation of 2 dB for 2 Tx devices for PC1.5 (Qualcomm)

- Recommended WF

  - TBD.

Issue 2-3-3: NS_03 for n25

- Proposals

  - Option 1: No need for 45 MHz channel bandwidth as it is DL only (Nokia)

- Recommended WF

  - TBD.

Issue 2-3-4: Requirement for UTRA NSs

Moderator comment: Following agreements were made regarding this in RAN4#116bis. Before discussing how to deal with these NSs, check with relevant operators if these NSs are needed.

Agreement: - Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49. - Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed. - If needed, how to define UTRA ACLR for PC1.5

- Proposals

  - Option 1: Specify same A-MPR requirements for NS_03U and NS_05U as NS_03 and NS_05 (CATT)

  - Extend the current A-MPR requirements for PC3 and PC2 for NS_100 to PC1.5 (CATT)

- Recommended WF

  - TBD.

Issue 2-3-5: UTRA ACLR for NS_03U, NS_05U, and NS_100

Moderator comment: Check first if these NSs are needed. If NS_100 is needed, then UTRA ACLR requirement needs to be discussed. Currently it is defined for PC2 and PC3.

Table 6.5.2.4.2-1: UTRA ACLR requirement

|  | Power class 2 | Power class 3 |
| --- | --- | --- |
| UTRAACLR1 | 33 dB | 33 dB |
| UTRAACLR2 | 36 dB | 36 dB |

- Proposals

  - Option 1: New value is needed based on co-existence simulations (MediaTek)

  - Option 2: Can reuse PC2 values (Huawei)

- Recommended WF

  - TBD.

## Reference sensitivity degradation

Issue 2-4-1: Duplexer isolation assumption

- Proposals

  - Option 1: (LGE)

|  | CBW≤20 MHz | CBW> 20 MHz |
| --- | --- | --- |
| n1 | 50 dB | 50 dB |
| n25 | 50 dB | 40 or 35 dB |

- Recommended WF

  - TBD.

Issue 2-4-2: Antenna isolation assumption

- Proposals

  - Option 1: 10 dB for handheld and 20 dB or above for FWA (Ericsson)

  - Option 2: 20 dB or higher for FWA (Nokia)

  - Option 3: 10dB for FDD bands with UL >1.7GHz i.e., n1, and n25, irrespective of the form factor

    - Option 3-1: Capture this aspect in the specification by modifying the title of Table 6.2D.2-2 (Apple)

- Recommended WF

  - TBD.

Issue 2-4-3: Methodology

- Proposals

  - Option 1: Utilize 1Tx PC2 RSD to derive PC1.5 RSD via the following steps (Apple)

    - Derive the interfering Tx noise in primary path from the existing 1Tx PC2 RSD specifications

    - Add 3dB to the calculated PC2 Tx noise to generate the noise for PC1.5

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 2: Add extra 3 dB to 2Tx PC2 RSD to get PC1.5 2Tx RSD (Huawei)

  - Option 3: Only consider duplexer isolation not antenna isolation (LGE)

  - Option 4: Calculate PC1.5 RSD via the following steps (Murata)

    - Derive the interfering Tx noise in primary path from the existing 1 Tx PC3 REFSENS specifications. Assume 6dB interference balance between primary and diversity.

    - Utilize the same tx noise as the primary and diversity path are symmetrical, to generate the noise for PC1.5

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 5: Calculate PC1.5 RSD via the following steps (Skyworks)

    - Derive the interfering Tx noise in primary path from the existing 1 Tx PC3 REFSENS specifications. Assume 6dB interference balance between primary and diversity.

    - Add 3dB to the calculated PC3 Tx noise and apply it to both primary and diversity branch i.e., with an imbalance of 0 dB,

    - Use the resulting PC1.5 Tx interfering noise from above to calculate the RSD for PC1.5

  - Option 6: Add extra X dB to PC3 RSD to get PC1.5 2Tx RSD according to supporting CBWs for example bands (MediaTek)

- Recommended WF

  - TBD.

Issue 2-4-4: LUT for RSD for future work

- Proposals

  - Option 1: Discuss if a general LUT based approach can be used (Murata)

- Recommended WF

  - TBD.

Issue 2-4-5: PC1.5 2 Tx RSD for n1

- Proposals

  - Option 1: 0dB for all channel bandwidth (CATT, Ericsson, MediaTek, ZTE)

- Recommended WF

  - Agree on Issue-2-4-3 first.

Issue 2-4-6: PC1.5 2 Tx RSD for n25

- Proposals

| Options | 5 MHz (dB) | 10 MHz (dB) | 15 MHz (dB) | 20 MHz (dB) | 25 MHz (dB) | 30 MHz (dB) | 35 MHz (dB) | 40 MHz (dB) | 45 MHz (dB) | 50 MHz (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Table 7.3.2-1d RSD for PC3 to PC2 | 1.5 | 1.5 | 1.6 | 1.6 | 1.7 | 6.0 | 6.2 | 6.7 | 7.1 |  |
| Option 1 | 3.6 (+2.1) | 3.6 (+2.1) | 3.7 (+2.1) | 3.7 (+2.1) | 3.9 (+2.2) | 8.7 (+2.7) | 8.9 (+2.7) | 9.4 (+2.7) | 9.8 (+2.7) |  |
| Option 2 | 4.5 (+3.0) | 4.5 (+3.0) | 4.6 (+3.0) | 4.6 (+3.0) | 4.7 (+3.0) | 9.0 (+3.0) | 9.2 (+3.0) | 9.7 (+3.0) | 10.1 (+3.0) |  |
| Option 3 | 3.5 (+2.0) | 3.5 (+2.0) | 3.6 (+2.0) | 3.6 (+2.0) | 3.7 (+2.0) | 8.5 (+2.5) | 8.7 (+2.5) | 9.2 (+2.5) | 9.6 (+2.5) |  |
| Option 4 | 1.7 (+0.2) | 1.7 (+0.2) | 1.8 (+0.2) | 2.2 (+0.6) | 2.5 (+0.8) | 7.2 (+1.2) | 7.3 (+1.1) | 8.4 (+1.7) | 8.7 (+1.6) |  |
| Option 5 | 2.4 (+0.9) | 2.5 (+1.0) | 2.5 (+0.9) | 2.6 (+1.0) | 2.8 (+1.1) | 8.8 (+2.8) | 9.1 (+2.9) | 9.8 (+3.1) | 10.2 (+3.1) |  |

  - Option 1: Apple

  - Option 2: Huawei

  - Option 3: MediaTek

  - Option 4: Murata

  - Option 5: ZTE

- Recommended WF

  - Agree on Issue-2-4-3 first.

Issue 2-4-7: RMC channel

- Proposals

  - Option 1: New RMC channel is required (MediaTek)

- Recommended WF

  - TBD.

- References

- RP-252942, New WID: UE RF enhancements for NR FR1, Phase 5, vivo, RAN meeting #109, Sep 2025.

- R4-2515086, Workplan for Rel20 UE RF enhancements Ph5, Nokia, Samsung, Huawei, T-Mobile USA, Ericsson, RAN4#116bis, Oct 2025.

- R4-2515087, Way Forward for [116bis][316] NR_UE_RF_Ph5, Nokia, Murata, RAN4#116bis, Oct 2025.

- Annex 1: Past Agreements

# RAN4#116bis

## 4Tx with up to 32 dBm MOP

### MPR/A-MPR assumptions

Agreement:

- Evaluate assuming following antenna isolation assumptions

  - For CPE, assuming both 10dB and 20dB,

  - For FWA, assuming 20dB.

Come back once MPR results are available, to evaluate if it is possible to select a single CPE antenna isolation assumption based on the MPR results.

### NS to be evaluated

Agreement:

- Evaluate A-MPR for NS_04 and NS_47 for n41.

## PC1.5 with 2Tx for FDD bands

### Maximum output power

Agreement:

- 29 dBm is the maximum output power.

### ACLR assumption

Agreement:

- 31 dB is the agreed ACLR value.

### NS to be evaluated

Agreement:

- Agree to evaluate PC1.5 A-MPR for NS_03, NS_05, NS_48, and NS_49.

- Encourage companies to check if NS_03U, NS_05U, and NS_100 are needed.

  - If needed, how to define UTRA ACLR for PC1.5

### REFSENS degradation

Agreement:

Encourage companies to provide the general methodology for deriving the REFSENS degradation (RSD) in next meeting.