3GPP TSG-RAN WG4 Meeting #117	R4-2522632

# Dallas, USA, 17th – 21st of November 2025

Title: 	WF on NR_ATG_enh_RRM

Agenda Item:	6.7.1

Source: 	CMCC

Document for:	Approval

<Topic 1> RRM perf requirement

Issue 2-1: whether and how to introduce Event triggered reporting on SCC with deactivated SCell test under non-DRX with signalling for neighbour cell skipping

Agreement:

  - Introduce the test case of Event triggered reporting on SCC with deactivated SCell test under non-DRX with signalling for neighbour cell skipping

  - For the measurement reporting type:

    - Periodic measurement reporting

Issue 2-2: Test configuration for Event triggered reporting on SCC with deactivated SCell test under non-DRX with signalling for neighbour cell skipping

Agreement:

  - For the test configuration, the test in Clause A.6.6.1.8 TS38.133 can be taken as the reference, except that:

    - No highSpeedMeasCA-Scell-r17 indication configured

    - the DRX configuration shall be OFF

    - the Initial UL BWP configuration and Active UL BWP configuration for SCell shall be N/A

    - the neighbour cell measurement skipping indication shall be configured

    - the propagation condition shall follow R18 ATG test cases

    - The T2 and test requirements shall be derived based on Table 9.2D.5.1-2 and Table 9.2D.5.2-2 and PUSCH resource configuration.

    - the measCycleSCell shall be configured as 160ms

    - measurement report configuration based on Issue 2-1

<Topic 2> RRM core requirement maintenance

Issue 1-1: Further refinement on Klayer1_measurement

Agreement:

  - Revise the Klayer1_measurement as following to reflect the impact from one common Rx beam.

    - Klayer1_measurement=1,

      - If inter-band carrier aggregation within FR1 is configured, and UE not capable of antennaArrayType-r18 on the other serving carrier, or UE capable of antennaArrayType-r18 on the other serving carrier and UE support two simultaneous separate Rx beams

        - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on one serving cell outside measurement gap are not fully overlapped by intra-frequency SMTC occasions of the same serving cell, or

        - if the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

      - Otherwise,

        - If the measured carrier is the SCC with servingcellMO configured, and the network indication neighSCellMeasSkipping is set to ‘enable’ to UE

          - If SMTC occasion for PCC and SCC are not overlapped in the time domain, or

          - If SMTC occasion for PCC and SCC are overlapped in the time domain, and all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by overlapped SMTC occasions of serving cells

        - Otherwise

          - if all of the reference signals configured for RLM, BFD, CBD or L1-RSRP for beam reporting on any serving frequency outside measurement gap are not fully overlapped by intra-frequency SMTC occasions

    - Klayer1_measurement=1.5, otherwise.

Issue 1-2: Further refinement on N1

Agreement:

- For ATG UE capable of antennaArrayType-r18: For the SCC measurement, N1=1 when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and inter-band carrier aggregation within FR1 is configured and UE not capable of antennaArrayType-r18 on the other serving carrier or UE support two simultaneous separate Rx beams, or when the network indication neighSCellMeasSkipping is set to ‘enable’ to UE and SMTC occasion for PCC and SCC are not overlapped in the time domain and intra-band carrier aggregation within FR1 is configured or inter-band carrier aggregation within FR1 is configured and UE support one common Rx beam. For other cases, N1 = 3 when network assistance information on ATG cells reference locations is provided, N1 = 4 when network assistance information on ATG cells reference locations is not provided.

CR work split

| Test number | Title | Company |
| --- | --- | --- |
| 1-1 | A.19.4.6 A.19.4.6.1 Interruption requirement at SRS antenna port switching with 1 SRS symbol in a slot | Huawei |
| 1-2 | A.19.4.6.2 Interruption requirement at SRS antenna port switching with more than 1 SRS symbol in a slot | Huawei |
| 2-1 | A.19.4.7 A.19.4.7.1 SCell Activation and deactivation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle (together with interruption requirement verification) | ZTE |
| 2-2 | A.19.4.7.2 SCell Activation and deactivation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle (together with interruption requirement verification) | ZTE |
| 2-3 | A.19.4.7.3 SCell Activation and deactivation of unknown SCell in FR1 in non-DRX (together with interruption requirement verification) | Ericsson |
| 2-4 | A.19.4.7.4 Direct SCell activation at SCell addition of known SCell in FR1 (together with interruption requirement verification) | Apple |
| 2-5 | A.19.4.7.5 Direct SCell activation at handover with known SCell in FR1 | Apple |
| 2-6 | A.19.4.7.6 Fast SCell Activation of known SCell in FR1 in non-DRX for 160 ms SCell measurement cycle | LGE |
| 2-7 | A.19.4.7.7 Fast SCell Activation of known SCell in FR1 in non-DRX for 640 ms SCell measurement cycle | LGE |
| 2-8 | A.19.4.7.8 SCell Activation of unknown SCell with valid L3 measurement results in FR1 in non-DRX for 160 ms SCell measurement cycle | Ericsson |
| 2-9 | A.19.4.7.9 TRS based SCell Activation of SSB-less SCell in FR1 inter-band CA in non-DRX | CATT |
| 3-1 | A.19.4.2.3 Beam Failure Detection and Link Recovery Test for FR1 SCell configured with CSI-RS-based BFD and SSB-based LR in non-DRX mode for ATG | CATT |
| 4-1 | A.19.5.1.5 Event triggered reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 640ms | CMCC |
| 4-2 | A.19.5.1.6 Periodic reporting on SCC with deactivated SCell test under non-DRX, with measurement cycle of no smaller than 160ms | CMCC |