3GPP TSG-RAN WG4 Meeting #117	R4-2522657

# Dallas, USA, 17th – 21th Nov., 2025

Title: 	WF on NR_ATG_enh_demod

Agenda Item:	6.7.1

Source: 	CMCC

Document for:	Approval

<Topic 1>

Issue 1-1: TT value for ATG

Agreement:

- Update the TT value as 0.3dB and corresponding requirement in spec 38.141-1 and 38.141-2 for ATG requirement in Rel-18 and Rel-19.

Annex.A

- Simulation assumption for DL CA

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U and 30D4S6U Note: The same simulation results can be assumed for two TDD patterns |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:5,10,15,20,25,30,35,40,45,50MHz TDD: 5,10,15,20,25,30,40,50,60,80,90,100MHz |
| MCS |  |  | 64QAM MCS22 |
| Antenna configuration |  |  | 2T2R and 2T4R |
| Propagation condition |  |  | FDD: AWGN+220Hz TDD: AWGN+500Hz |
| Rank |  |  | 1 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

- Simulation assumption for DL MIMO

| Parameter |  | Unit | Value |
| --- | --- | --- | --- |
| Duplex mode |  |  | FDD TDD: 7D1S2U and 30D4S6U Note: The same simulation results can be assumed for two TDD patterns |
| Subcarrier spacing |  |  | FDD: 15kHz TDD: 30kHz |
| CHBW |  |  | FDD:10MHz TDD:40MHz |
| MCS |  |  | 64QAM MCS 22 256QAM Table 2 MCS 24 |
| Antenna configuration |  |  | 2T2R and 2T4R with XP-low |
| Propagation condition |  |  | FDD: AWGN+220Hz TDD: AWGN+500Hz |
| Rank |  |  | 2 |
| PDSCH configuration | Mapping type |  | Type A |
|  | k0 |  | 0 |
|  | Starting symbol (S) |  | 2 |
|  | Length (L) |  | 12 |
|  | PDSCH aggregation factor |  | 1 |
|  | PRB bundling type |  | Static |
|  | PRB bundling size |  | 2 |
|  | Resource allocation type |  | Type 0 |
|  | RBG size |  | Config2 |
|  | VRB-to-PRB mapping type |  | Non-interleaved |
|  | VRB-to-PRB mapping interleaver bundle size |  | N/A |
| PDSCH DMRS configuration | DMRS Type |  | Type 1 |
|  | Number of additional DMRS |  | 1 |
|  | DMRS port |  | {1000},{1001} |
|  | Maximum number of OFDM symbols for DL front loaded DMRS |  | 1 |
| Receiver type |  |  | MMSE-IRC |

- Simulation assumption for UL MIMO

| Parameter |  | Value |
| --- | --- | --- |
| Transform precoding |  | Disabled |
| Default TDD UL-DL pattern (Note 1) |  | 15kHz SCS: N/A 30 kHz SCS: 7D1S2U, S=6D:4G:4U |
| CHBW |  | FDD: 5MHz TDD:10MHz |
| MCS |  | 64QAM Table 1 MCS 20 |
| Antenna configuration |  | 2T2R with X-pol low correlation |
| Propagation conditions |  | FDD 15kHz SCS: H= 1j1−j+200Hz TDD 30kHz SCS: H= 1j1−j+500Hz |
| HARQ | Maximum number of HARQ transmissions | 4 |
|  | RV sequence | 0, 2, 3, 1 |
| DM-RS | DM-RS configuration type | 1 |
|  | DM-RS duration | single-symbol DM-RS |
|  | Additional DM-RS position | pos1 |
|  | Number of DM-RS CDM group(s) without data | 2 |
|  | Ratio of PUSCH EPRE to DM-RS EPRE | -3 dB |
|  | DM-RS port | {0},{1} |
|  | DM-RS sequence generation | NID0=0, nSCID =0 |
| Time domain resource assignment | PUSCH mapping type | A |
|  | Start symbol | 0 |
|  | Allocation length | 14 |
| Frequency domain resource assignment | RB assignment | Full applicable test bandwidth |
|  | Frequency hopping | Disabled |
| Code block group based PUSCH transmission |  | Disabled |
| NOTE 1: The same requirements are applicable to TDD with different UL-DL pattern, e.g., 30D4S6U, S=14G for 30kHz SCS. |  |  |

Annex.B

| Spec | Clauses and Title | Notes | Company |
| --- | --- | --- | --- |
| 38.101-4 | 5.1.1.7.8 (new) Applicability and test rules for ATG | For CA feature, including Applicability and test rules Selection of CA configurations Applicability between different TDD pattern if introduced | CMCC |
|  | 5.1.1.12 Applicability of requirements for ATG | For DL-MIMO feature UE with Cross Polarized Antennas HARQ related | CMCC |
|  | 5.2.2.1.22, 5.2.2.2.23, 5.2.3.1.21, 5.2.3.2.22 Minimum requirements for ATG | For DL-MIMO feature, including the test requirements | Ericsson |
|  | 5.2A.2.7, 5.2A.3.6 (new) Minimum requirements for ATG CA | For CA feature, at least including: Test purpose Test parameters HARQ and K1 related configuration Test requirements | ZTE |
|  | A.3.2.1.1, A.3.2.2.2 Reference measurement channels | For CA feature and DL-MIMO feature | CMCC |
| 38.104 | 8.2.14 Requirements for PUSCH for ATG |  | Samsung |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Samsung |
|  | G.1 Static propagation condition | The static propagation condition model from TS38.101-4 can be taken as the reference Capture the static channel matrix in spec | ZTE |
|  | G.2.3.2.3 MIMO Correlation Matrices using cross polarized antennas |  | Nokia |
| 38.141-1 | 8.1.2.11 Applicability of PUSCH performance requirements for ATG scenario |  | CMCC |
|  | 8.2.14 Performance requirements for PUSCH for ATG |  | Ericsson |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Ericsson |
|  | G.1 Static propagation condition |  | ZTE |
|  | G.2.3.2.3 MIMO correlation matrices using cross polarized antennas |  | Nokia |
| 38.141-2 | 8.1.2.11 Applicability of ATG PUSCH performance requirements |  | ZTE |
|  | 8.2.14 Performance requirements for PUSCH for ATG |  | Ericsson |
|  | A.5 Fixed Reference Channels for performance requirements (64QAM, R=567/1024) |  | Ericsson |
|  | J.1 Static propagation condition |  | CMCC |
|  | J.2.3.2.3 MIMO correlation matrices using cross polarized antennas |  | Nokia |