3GPP TSG-RAN4 Meeting #117	R4-2522665

# Prague, Czechia, 18th Nov 2025 – 22nd Nov 2025

Title: 	WF on NR_UE_OTA_Enh

Agenda item:	7.3.1

Source:	Eutelsat Group

Document for:	Approval

# Topic #2: Enhanced test methodologies to cover NTN Ka and Ku bands (agenda 7.3.3)

### Sub-topic 1-1: TR Skeleton

The draft TR in R4-2520725 is approved.

# Topic #2: Enhanced test methodologies to cover NTN Ka and Ku bands (agenda 7.3.3)

### Sub-topic 2-1: DUT size

Agreement:

### Focus on VSAT with VSAT and antenna dimensions 65 cm x 65 cm x 15 cm with a weight around 12 kg. Larger VSAT including devices with dual panel exceeding 65 cm x 65 cm x 15 cm can be considered as required in future meetings

### Sub-topic 2-2: DUT cooling requirements

Agreement:

No cooling is assumed in RAN4 discussion in normal temperature.

### Sub-topic 2-4: White box vs black box

Agreement:

Agree on black-box approach as baseline, and white-box approach may be considered in the future.

### Sub-topic 2-5: Worst case antenna assumptions

Agreement:

Encourage the inputs from VSAT vendors in the next meeting based on proposals 7 and 8 from Keysight in R4-2522035

.

### Sub-topic 2-6: Applicability of polarization

RAN4 can discuss how to update the WID on LP or CP and try to reach consensus this meeting, the final decision of WID update will be RAN-P.

WID update wording for RAN #110

## 4.1 	Objective of SI or Core part WI or Testing part WI

Objectives for this core part work item are as follows

- Define enhanced test methodologies to cover NTN Ka and Ku bands

  - Study how to extend the applicability of RAN4 defined OTA test methods to support Ku and Ka testing

    - FR2 OTA test methods (DFF, IFF, NFTF) extension is the first priority

    - FR1 OTA test method extension is not precluded, if also applicable

  - Study whether and how to support circular polarization measurements

    - Linear polarization shall be supported and is the sole fallback if CP cannot be supported.

    - The goal is to support LP and CP in a single test system

### Sub-topic 2-7: Polarization support in test systems

The CRs in R4-2522421 and R4-2522422 in AI 10.7 [117][126] propose to add the necessary applicability statements for how the VSAT and SAN minimum requirements apply to specific manufacturer declared polarization capabilities

Encourage the inputs from VSAT vendors in the next meeting based on what devices exist using this table:

Table 1: Polarization Combinations for VSAT UE

| Satellite Orbit Scenario | Band | VSAT UL | VSAT DL |
| --- | --- | --- | --- |
| GSO | Ka | RHCP LHCP LP | LHCP RHCP LP |
|  | Ku | RHCP LHCP LP | LHCP RHCP LP |
| NGSO | Ka | RHCP LHCP LP | LHCP RHCP LP |
|  | Ku | RHCP LHCP LP | LHCP RHCP LP |

### Sub-topic 2-8: Beam peak search

### Agreement:

The direction of beam peak is assumed to be perpendicular to the VSAT antenna plane unless declared otherwise by manufacturers.

Global beam peak search is not needed. Local Beam peak search around the commanded beam direction shall be carried out for both mechanical and electronic antennas using the test system positioner, not the VSAT’s fine beam pointing capability.

Testability agreements should be reflected in TS 38.101-5.

### Sub-topic 2-9: Beam steering

### Agreement:

### there is no testing of VSAT beam steering to a moving SAN target. This should be reflected in TS 38.101-5.

### Sub-topic 2-10: Beamlock

Agreement:

RAN4 testability assumes that the VSAT beam direction can be controlled to a specific direction (azimuth and elevation) with reference to a declared VSAT antenna coordinate system and remains there. How to enable that depends on RAN5 discussions of test mode or other solution.

### Sub-topic 2-11: Beampointing stability

Moderator: This issue is covered separately by the CR from Keysight in R4-2522034

### Sub-topic 2-12: EIRP measuremetn method and grid baseline

### Agreement:

### The EIRP measurement method and grid baseline has to be reassessed due to the narrow beam widths of VSAT compared to FR2 handsets.

### Sub-topic 2-13: EIS measuremetn method and grid baseline

### Agreement:

### The EIS measurement method and grid baseline has to be reassessed due to the narrow beam widths of VSAT compared to FR2 handsets.

### Sub-topic 2-14: TRP measurement method

### Agreement:

### The step size grid will be redefined first. The TRP integration should also be updated.

### Sub-topic 2-15: Spectrum emission mask

Agreement:

All VSAT requirements are OTA.

### Sub-topic 2-16: Antenna gain requirements

Agreement:

### Investigating alternative core requirements to address the testability issues related to antenna gain  is not discussed in this WI.

### Sub-topic 2-19: Applicability of requirements

Agreement:

Consider the environmental condition for vibration in Clause E.2.3 of TS 38.101-5 a system level requirement and thus non testable from a conformance perspective.