3GPP TSG-RAN WG4 Meeting # 117	R4-2522805

# Dallas, USA, 17th November – 21st November

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR Test case for L1-SRS-RSRP measurement accuracy in FR1 and FR2 in SBFD operation |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-10-23 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | L1-SRS-RSRP measurement accuracy requirements in FR1 and FR2 in SBFD operation miss test cases in the RRM spec. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Test cases for L1-SRS-RSRP measurement accuracy requirements in FR1 and FR2 in SBFD operation are defined. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Test case for L1-SRS-RSRP measurement accuracy requirements in FR1 and FR2 in SBFD operation are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses: A.6.7.X.1, A.7.7.X.1. |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  | N/A. |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2520548. |  |  |  |  |  |  |  |  |

==============First change==============

A.6.7.X1.1	SA L1-SRS-RSRP measurement accuracy with FR1 serving cell with SBFD

A.6.7.X1.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SRS-RSRP measurement accuracy in SBFD operation is within the specified limits. This test will verify the requirements in clauses 10.1.X.1.1 with the testing configurations for NR cell in table A.6.7.X1.1.1-1.

Table A.6.7.X1.1.1-1: Applicable NR configurations for FR1 L1-SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 30 kHz SRS SCS, 100 MHz bandwidth, TDD duplex mode |

A.6.7.X1.1.2	Test Parameters

In this set of test cases there is one cell in the test, FR1 PCell (Cell 1) with SBFD operation. The test parameters for the Cell 1 are given in table A.6.7.X1.1.2-1 below. The test parameters for the (virtual) aggressor UE transmitting SRS are given in table A.6.7.X1.1.2-2.

Before the test UE is configured to perform L1-SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in UL subband in SBFD slots according to the SRS configuration in table A.6.7.X1.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 1 data symbol before SRS to be transmitted.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-SRS-RSRP.

Table A.6.7.X1.1.2-1: FR1 test parameters for L1-SRS-RSRP accuracy for PCell

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| SSB ARFCN |  | 1 |  | freq1 | freq1 |
| Duplex mode |  | 1 |  | TDD | TDD |
| TDD configuration |  | 1 |  | TDDConf.2.4 | TDDConf.2.4 |
| SBFD configuration |  | 1 |  | SBFD.1 FR1 | SBFD.1 FR1 |
| BWchannel_total |  | 1 | MHz | 100: NPRB,c = 273 | 100: NPRB,c = 273 |
| PDSCH Reference measurement channel |  | 1 |  | SR.2.1 TDD | SR.2.1 TDD |
| RMSI CORESET Reference Channel |  | 1 |  | CR.2.1 TDD | CR.2.1 TDD |
| Dedicated CORESET Reference Channel |  | 1 |  | CCR.2.1 TDD | CCR.2.1 TDD |
| SSB configuration |  | 1 |  | SSB.14 FR1 | SSB.14 FR1 |
| OCNG Patterns |  | 1 |  | OP.1 | OP.1 |
| TRS configuration |  | 1 |  | TRS.1.2 TDD | TRS.1.2 TDD |
| Initial BWP Configuration |  | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration |  | 1 |  | DLBWP.1.1 ULBWP.1.1 | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration |  | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system |  | 1 | s | 17.67 | 17.67 |
| EPRE ratio of PSS to SSS |  | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/15 kHz | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 3 | 1 | dBm/SRS SCS | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  | -109 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table A.6.7.X1.1.2-2: FR1 test parameters for L1-SRS-RSRP accuracy for aggressor UE

| Parameter |  | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- | --- |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/15 kHz | -91 | -114 |
|  | NR_TDD_FR1_C |  |  |  | -113 |
|  | NR_TDD_FR1_D |  |  |  | -112.5 |
|  | NR_TDD_FR1_E |  |  |  | -112 |
| Note2 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -88 | -111 |
|  | NR_TDD_FR1_C |  |  |  | -110 |
|  | NR_TDD_FR1_D |  |  |  | -109.5 |
|  | NR_TDD_FR1_E |  |  |  | -109 |
| on SRS |  | 1 | dB | 1 | 1 |
| SRS RSRP Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/SRS SCS | -87 | -110 |
|  | NR_TDD_FR1_C |  |  |  | -109 |
|  | NR_TDD_FR1_D |  |  |  | -108.5 |
|  | NR_TDD_FR1_E |  |  |  | -108 |
| Io Note3 | NR_TDD_FR1_A NOTE 5 | 1 | dBm/47.88 MHz | -52.43 | -75.43 |
|  | NR_TDD_FR1_C |  |  |  | -74.43 |
|  | NR_TDD_FR1_D |  |  |  | -73.93 |
|  | NR_TDD_FR1_E |  |  |  | -73.43 |
| on SRS |  | 1 | dB | 1 | 1 |
| Propagation condition |  | 1 |  | AWGN | AWGN |
| Antenna configuration |  | 1 |  | 1x2 | 1x2 |
| SRS configuration |  | 1 |  | SRSConf.1 | SRSConf.1 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of the test. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification. |  |  |  |  |  |

Table A.6.7.8.1.2-3: SRS configuration parameters for FR1 L1-SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl40, 16 |
|  | sequenceId | 0 |

A.6.7.X1.1.3	Test Requirements

The L1-SRS-RSRP measurement accuracy shall fulfil the requirements in clauses 10.1.X.1.1. The rate of correct L1-SRS-RSRP measurement accuracy observed during repeated tests shall be at least 90 %.

==============Next change==============

A.7.7.X.1	SA L1-SRS-RSRP measurement accuracy with FR2 serving cell with SBFD

A.7.7.X.1.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-SRS-RSRP measurement accuracy in SBFD operation is within the specified limits. This test will verify the requirements in clauses 10.1.X.1.1 with the testing configurations for NR cells in table A.7.7.X.1.1-1.

Table A.7.7.X.1.1-1: Applicable NR configurations for FR2 L1-SRS-RSRP accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, [100] MHz bandwidth, TDD duplex mode |

A.7.7.X.1.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1) with SBFD operation. The test parameters for the Cell 1 are given in table A.7.7.X.1.2-1 and A.7.7.X.1.2-2 below. The test parameters for the (virtual) aggressor UE transmitting SRS are given in table A.7.7.X.1.2-2.

Before the test UE is configured to perform L1-SRS-RSRP measurement. During the test, the test system transmits SRS resources for measurement in the UL subband in SBFD slots according to the SRS configuration in table A.7.7.X.1.2-3. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on SRS symbol to be transmitted and on 2 data symbols before SRS to be transmitted.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-SRS-RSRP.

Table A.7.7.X.1.2-1: FR2 test parameters for L1-SRS-RSRP accuracy

| Parameter | Config | Unit | Test 1 | Test 2 |
| --- | --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 | freq1 |
| Duplex mode | 1 |  | TDD | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 | TDDConf.3.1 |
| SBFD configuration | 1 |  | SBFD.1 FR2 | SBFD.1 FR2 |
| BWchannel_total | 1 | MHz | 100: NPRB,c = 66 | 100: NPRB,c = 66 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.3 FR2 | SSB.3 FR2 |
| OCNG Patterns | 1 |  | OP.1 | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 | SMTC.1 |
| Time offset between DL from serving cell and SRS from test system | 1 | s | 7.76 | 7.67 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |
| Propagation condition | 1 |  | AWGN | AWGN |
| Antenna configuration | 1 |  | 1x2 | 1x2 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |  |  |

Table A.7.7.X.1.2-2: L1-SRS-RSRP accuracy OTA related test parameters for PCell and aggressor UE in FR2

| Parameter | Unit | T1 | T2 |
| --- | --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine | Fine |
| Note1 | dBm/15 kHzNote3 | -100 | N/A |
| Note1 | dBm/SCSNote3 | -91 | N/A |
|  | dB | 2 | N/A |
| Es | dBm/SCSNote3 |  | (Table B.2.7-2 Rx Beam Peak) |
| SRS_RPNote2 | dBm/SCS | -89 | (Table B.2.7-2 Rx Beam Peak) |
| BB Note4 | dB | >1 | 1 |
| IoNote2 | dBm/46.08 MHz Note3 | -61.03 | (Table B.2.7-2 Rx Beam Peak +[50.79] dB) |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2 dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |  |  |

Table A.7.7.X.1.2-3: SRS configuration parameters for FR2 L1-SRS-RSRP accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| SRS-ResourceSet | srs-ResourceSetId | 0 |
|  | srs-ResourceIdList | 0 |
|  | resourceType | Periodic |
|  | Usage | Codebook |
| SRS-Resource | SRS-ResourceId | 0 |
|  | nrofSRS-Ports | Port1 |
|  | transmissionComb | n2 |
|  | combOffset-n2 | 0 |
|  | cyclicShift-n2 | 0 |
|  | resourceMapping startPosition | 0 |
|  | resourceMapping nrofSymbols | n1 |
|  | resourceMapping repetitionFactor | n1 |
|  | freqDomainPosition | 0 |
|  | freqDomainShift | 0 |
|  | freqHopping c-SRS | 12 |
|  | freqHopping b-SRS | 0 |
|  | freqHopping b-hop | 0 |
|  | groupOrSequenceHopping | Neither |
|  | resourceType | Periodic |
|  | periodicityAndOffset-p | sl160, 26 |
|  | sequenceId | 0 |

A.7.7.X.1.3	Test Requirements

The L1-SRS-RSRP measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.X.1.1 and the rate of correct L1-SRS-RSRP measurement accuracy observed during repeated tests shall be at least 90 %. The following requirements are to be verified:

During T1:

The UE is deemed to meet the requirement if the reported L1-SRS-RSRP is in the range shown in table A.7.7.X.1.3-1.

During T2:

The UE is deemed to meet the requirement if the reported L1-SRS-RSRP is in the range shown in table A.7.7.X.1.3-1.

Table A.7.7.X.1.3-1: L1-SRS-RSRP absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
| SRS | SRS_RP -δ +Gmin ≤ Reported SRS-RSRP(dBm) ≤SRS_RP +δ +Gmax |
| NOTE 1: SRS_RP is the equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone configured in the test NOTE 2: δ is the RSRP absolute accuracy requirement from Table 10.1.22.1.1-2, selected according to the Io used in the test NOTE 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

==============End of change==============