3GPP TSG-RAN4 Meeting #117	R4-2522806

# Dallas, USA, 17th – 21st November, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | DraftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for CSI-RS based BFR and LR Test for FR1 and FR2 in SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | No BFD and LR test cases for a UE operating on a SBFD scenario |  |  |  |  |  |  |  |  |
| Summary of change: |  | New test cases of beam failure detection and link recovery for FR1/FR2 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for a UE operating on a SBFD scenario are added. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | No test cases for beam failure detection and link recoverty for a UE operating on a SBFD scenario |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.5.5.x, A.7.5.5.x |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS38.521-4 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

----------------------START OF CHANGE #1----------------------------

A.6.5.5.x	Beam Failure Detection and Link Recovery Test for FR1 PCell configured with CSI-RS-based BFD and LR in non-DRX mode for a UE operating with SBFD

A.6.5.5.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting supportSBFD properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell operating on SBFD, and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the DL subband of UEs active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR1 serving cell requirements in clause 8.5.

Supported test configurations are specified in Table A.6.5.5.x.1-1. General test parameters as specified in Table A.6.5.5.3.1-2 with config 3 apply except those specified in Table A.6.5.5.x.1-2. Cell specific test parameters as specified in Table A.6.5.5.3.1-3 apply except those specified in Table A.6.5.5.x.1-3.

The test procedure specified in clause A.6.5.5.3.1 applies to this test. In addition, during T3, there is overlapping between occasions of the CSI-RS resource for BFD (q0) and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.6.5.5.x.1-1: Supported test configurations for FR1 PCell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 30 kHz SSB SCS, 100 MHz bandwidth |

Table A.6.5.5.x.1-2: General test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| TDD Configuration |  | TDDConf.2.4 | A.3.1.4 |
| SBFD configuration |  | SBFD.2 FR1 | A.3.X |
| RMSI CORESET Reference Channel |  | CR.2.1 TDD | A.3.1.2 |
| Dedicated CORESET Reference Channel |  | CCR.2.1 TDD | A.3.1.3 |
| SSB Configuration |  | SSB.14 FR1 | A.3.10.1 |
| CSI-RS configuration for q0 and q1 |  | CSI-RS.2.2 TDD | A.3.14 |
| CSI-RS configuration for CSI reporting |  | CSI-RS.2.1 TDD | A.3.14 |
| TRS configuration |  | TRS.1.2 TDD | A.3.17.1 |
| CSI-RS-Index assigned as RLM RS |  | CSI-RS.2.2 TDD | A.3.14 |
| T1 | s | [0.2] | During this time the the UE shall be fully synchronized to Cell 1 |
| T2 | s | TBD |  |
| T3 | s | TBD |  |
| T4 | s | 0 |  |
| T5 | s | TBD |  |
| D1 | s | TBD |  |
| NOTE 1: UE-specific PDCCH is not transmitted after T1 starts. |  |  |  |

Table A.6.5.5.x.1-3: Cell specific test parameters for FR1 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | T1 | T2 | T3 | T4 | T5 |
| SNR_CSI-RS of set q0 | dB | 5 | -3 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | dB | -10 | -10 | 10 | 10 | 10 |
| CSI-RS_RP of set q1 | dBm/SCS kHz | -105 | -105 | -85 | -85 | -85 |
|  | dBm/15 kHz | -98 |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for Ues other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the Res carrying CSI-RS. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.6.5.5.3.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. |  |  |  |  |  |  |

A.6.5.5.x.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 30+20 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90 %.

----------------------END OF CHANGE #1----------------------------

----------------------START OF CHANGE #2----------------------------

A.7.5.5.x	Beam Failure Detection and Link Recovery Test for FR2 Pcell configured with CSI-RS-based BFD and LR in non-DRX mode for a UE operating with SBFD

A.7.5.5.x.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting supportSBFD properly detects CSI-RS-based beam failure in the set q0 configured for a serving cell operating on SBFD, and that the UE performs correct CSI-RS-based link recovery based on beam candicate set q1. The purpose is to test the downlink monitoring for beam failure detection within the Ues active DL BWP, during the evaluation period, and link recovery, when no DRX is used. This test will partly verify the CSI-RS based beam failure detection and link recovery for an FR2 serving cell requirements in clause 8.5.

Supported test configurations are specified in Table A.7.5.5.x.1-1. General test parameters as specified in Table A.7.5.5.3.1-2 apply except those specified in Table A.7.5.5.x.1-2. Cell specific test parameters as specified in Table A.7.5.5.3.1-3 apply except those specified in table A.7.5.5.x.1-3.

The test procedure specified in clause A.7.5.5.3.1 applies to this test. In addition, during T3, there is overlapping between occasions of the CSI-RS resource for BFD (q0) and dynamic UL transmission on SBFD symbols, as specified in A.3.

Table A.7.5.5.x.1-1: Supported test configurations for FR2 Pcell

| Configuration | Description |
| --- | --- |
| 1 | TDD duplex mode, 120 kHz SSB SCS, [100] MHz bandwidth |

Table A.7.5.5.x.1-2: General test parameters for FR2 Pcell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
|  |  | Test 1 |  |
| TDD Configuration |  | TDDConf.3.1 |  |
| SBFD configuration |  | SBFD.2 FR2 | A.3.X |
| BWchannel |  | [100]: NPRB,c = [66] |  |
| Data PRBs allocated |  | [66] |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 120 |  |
| PDSCH Reference Channel |  | SR.3.2 TDD | A.3.1.1 |
| RMSI CORESET Reference Channel |  | CR.3.1 TDD | A.3.1.2 |
| Dedicated CORESET Reference Channel |  | CCR.3.1 TDD | A.3.1.3 |
| SSB Configuration |  | SSB.28 FR2 | A.3.10.2 |
| CSI-RS for tracking |  | TRS.2.1 TDD | A.3.17.2 |
| CSI-RS configuration for BFD/CBD/RLM |  | CSI-RS.3.2 TDD | A.3.14.2 |
| CSI-RS configuration for CSI reporting |  | CSI-RS.3.1 TDD | A.3.14.2 |
| CSI reporting periodicity | slot | 40 |  |
| CSI reporting offset | slot | 4 |  |
| T310 | ms | 1000 |  |
| N310 |  | 2 |  |
| T1 | s | [1] | The UE shall be fully synchronized to cell 1 during T1 |
| T2 | s | TBD |  |
| T3 | s | TBD |  |
| T4 | s | 0 |  |
| T5 | s | TBD |  |
| D1 | s | TBD |  |

Table A.7.5.5.x.1-3: Cell specific test parameters for FR2 PCell for CSI-RS-based beam failure detection and link recovery testing in non-DRX mode

| Parameter |  | Unit | Test 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| AoA setup |  |  | Setup 1 defined in A.3.15 |  |  |  |  |
| Assumptpion for UE beams Note 10 |  |  | Rough |  |  |  |  |
| SNR_CSI-RS of set q0 | Config 1 | dB | 5 Note 11 | -3 Note 11 | -12 | -12 | -12 |
| SNR_CSI-RS of set q1 | Config 1 | dB | 0.2 | 0.2 | 20.2 | 20.2 | 20.2 |
| CSI-RS_RP of set q1 | Config 1 | dBm/SCS | -104.5 | -104.5 | -84.5 | -84.5 | -84.5 |
|  | Config 1 | dBm/120 KHz | -104.7 |  |  |  |  |
| Propagation condition |  |  | TDL-A 30 ns 75 Hz |  |  |  |  |
| NOTE 1: OCNG shall be used such that the resources in Cell 1 are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 3: NZP CSI-RS resource set configuration for CSI reporting are assigned to the UE prior to the start of time period T1. NOTE 4: Void NOTE 5: The timers and layer 3 filtering related parameters are configured prior to the start of time period T1. NOTE 6: The signal contains PDCCH for UEs other than the device under test as part of OCNG. NOTE 7: SNR levels correspond to the signal to noise ratio over the REs carrying CSI-RS. NOTE 8: The SNR in time periods T1, T2, T3, T4 and T5 is denoted as SNR1, SNR2 and SNR3 respectively in figure A.7.5.5.3.1-1. NOTE 9: The SNR values are specified for testing a UE which supports 2RX on at least one band. For testing of a UE which supports 4RX on all bands, the SNR during T3 is modified as specified in clause A.3.6. NOTE 10: Information about types of UE beam is given in clause B.2.1.3 and does not limit UE implementation or test system implementation. NOTE 11: This value allows up to 1 dB degradation from applied SNR to UE baseband |  |  |  |  |  |  |  |

A.7.5.5.x.2	Test Requirements

The UE behaviour during time durations T1, T2, T3, T4 and T5 shall be as follows:

During the time duration T1 and T2, the UE shall transmit uplink signal at least in all subframes configured for CSI transmission on Cell 1.

During the period from time point A to time point B the UE shall transmit uplink signal in Cell 1 in all uplink slots configured for CSI transmission according to the configured periodic CSI reporting for Cell 1.

During T3 the shall detect beam failure and initiat link recovery. During T4 and T5 the UE measures and evaluate beam candidate from beam candidate set q1.

No later than time point F occurring no later than D1 = 260+10 ms after the start of T5, the UE shall transmit preamble on a beam associated with the candidate beam set q1. The UE shall not transmit preamble on a beam associated with the candidate beam set q1 earlier than time point B.

Test is concluded once the test equipment has received the initial preamble transmission from the UE. The rate of correct events observed during repeated tests shall be at least 90 %.

----------------------END OF CHANGE #2----------------------------