3GPP TSG-RAN4 Meeting #117	R4-2522809

# Dallas, USA, 17 – 21 Novermber, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | xxxx | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | draftCR on TC7-8 (L1-CLI-RSSI accuracy) for SBFD |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Huawei, HiSilicon |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-05 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Based on WF R4-2515166, test cases TC7-8 for L1-CLI-RSSI measurement accuracy in FR1/FR2 should be defined for SBFD RRM. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Introduce test cases TC7-8 for L1-CLI-RSSI measurement accuracy in FR1/FR2 for SBFD RRM. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Agreed test cases are missing. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.6.7.x.2 (new), A.7.7.x.2 (new) |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | x |  | Test specifications |  |  |  | TS 38.533 |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

<Start of Change 1>

#### A.6.7.x.2	L1-CLI-RSSI measurement accuracy in FR1 with SBFD

##### A.6.7.X.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.X.2.1 with the testing configurations for NR cells in table A.6.7.X.2.1-1.

Table A.6.7.X.2.1-1: Applicable NR configurations for FR1 L1-CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | NR 30 kHz SCS, 100 MHz bandwidth, TDD duplex mode |

##### A.6.7.X.2.2	Test parameters

In this set of test cases there is one cell in the test, the FR1 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.6.7.X.2.2-1 below.

Before the test UE is configured to perform L1-CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for L1-CLI-RSSI resource. The L1-CLI-RSSI measurement resource configuration is in table A.6.7.X.2.2-2.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-CLI-RSSI.

Table A.6.7.X.2.2-1: FR1 test parameters for L1-CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD configuration | 1 |  | TDDConf.2.4 |
| BWchannel | 1 | MHz | 100: NPRB,c = 273 |
| SBFD configuration | 1 |  | SBFD.2 FR1 |
| PDSCH Reference measurement channel | 1 |  | SR.2.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.2.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.2.1 TDD |
| SSB configuration | 1 |  | SSB.14 FR1 |
| OCNG Patterns Note6 | 1 |  | OP.1 |
| TRS configuration | 1 |  | TRS.1.2 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.1 ULBWP.1.1 |
| SMTC configuration | 1 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1 | s | 0 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| on CLI-RSSI measurement resource Note2 | 1 | dBm/30 kHz | -103 |
| on CLI-RSSI measurement resource | 1 | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note3 | 1 | dBm/30 kHz | -Infinity |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/17.28 MHz | -75.40 |
| Io on CLI-RSSI measurement resource Note3 | 1 | dBm/1.08 MHz | -87.44 |
| on CLI-RSSI measurement resource | 1 | dB | -Infinity |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| T1 | 1 | ms | 200 |
| T2 | 1 | ms | 100 |
| NOTE 1: OCNG shall be used such that a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: RSRP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. NOTE 5: The test configuration excludes support for band n51 and it is not required to run this test on band n51 in this release of the specification NOTE 6: OCNG is not transmitted in the CLI-RSSI measurement resources. |  |  |  |

Table A.6.7.X.2.2-2: L1-CLI-RSSI measurement resource configuration for FR1 L1-CLI-RSSI accuracy

|  | Field | Config | SRSConf.1 |
| --- | --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 1 | 0 |
|  | startPRB | 1 | 110 |
|  | nrofPRBs | 1 | 48 |
|  | startPosition | 1 | 3 |
|  | nrofSymbols | 1 | 11 |
|  | rssi-PeriodicityAndOffset | 1 | sl40, 9 |
| NOTE 1: startPRB is set such that CLI-RSSI measurement resource is fully contained within UL subband. |  |  |  |

##### A.6.7.X.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the requirements in clauses 10.1.X.2.1.

<End of Change 1>

<Start of Change 2>

#### A.7.7.x.2	L1-CLI-RSSI measurement accuracy in FR1 with SBFD

##### A.7.7.X.2.1	Test Purpose and Environment

The purpose of this test is to verify that the L1-CLI-RSSI measurement accuracy is within the specified limits. This test will verify the requirements in clauses 10.1.X.2.1 with the testing configurations for NR cells in table A.7.7.X.2.1-1.

Table A.7.7.X.2.1-1: Applicable NR configurations for FR2 CLI-RSSI accuracy test

| Config | Description |
| --- | --- |
| 1 | 120 kHz SRS SCS, 200 MHz bandwidth, TDD duplex mode |

##### A.7.7.X.2.2	Test parameters

In this set of test cases there is one cell in the test, FR2 PCell (Cell 1). The test parameters for the Cell 1 are given in table A.7.7.X.2.2-1 and A.7.7.X.2.2-2 below.

Before the test UE is configured to perform L1-CLI-RSSI measurement. There is no measurement gap configured in the test. During the test, the test system does not transmit PDCCH/PDSCH/OCNG on symbols for L1-CLI-RSSI resource. The L1-CLI-RSSI measurement resource configuration is in table A.7.7.X.2.2-3.

The test consists of two successive time periods, with time duration of T1 and T2, respectively. At the beginning of T2, the test equipment sends an DCI to trigger UE to report L1-CLI-RSSI.

Table A.7.7.X.2.2-1: FR2 test parameters for CLI-RSSI accuracy

| Parameter | Config | Unit | Value |
| --- | --- | --- | --- |
| SSB ARFCN | 1 |  | freq1 |
| Duplex mode | 1 |  | TDD |
| TDD configuration | 1 |  | TDDConf.3.1 |
| BWchannel | 1 | MHz | 200: NPRB,c = 132 |
| SBFD configuration | 1 |  | SBFD.2 FR2 |
| PDSCH Reference measurement channel | 1 |  | SR.3.1 TDD |
| RMSI CORESET Reference Channel | 1 |  | CR.3.1 TDD |
| Dedicated CORESET Reference Channel | 1 |  | CCR.3.1 TDD |
| SSB configuration | 1 |  | SSB.28 FR2 |
| OCNG Patterns Note2 | 1 |  | OP.1 |
| TRS configuration | 1 |  | TRS.2.1 TDD |
| Initial BWP Configuration | 1 |  | DLBWP.0.1 ULBWP.0.1 |
| Dedicated BWP configuration | 1 |  | DLBWP.1.3 ULBWP.1.3 |
| SMTC configuration | 1 |  | SMTC.1 |
| Time offset between DL from serving cell and OCNG from test system | 1 | s | 0 |
| EPRE ratio of PSS to SSS | 1 | dB | 0 |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |
| Propagation condition | 1 |  | AWGN |
| Antenna configuration | 1 |  | 1x2 |
| T1 | 1 | ms | 200 |
| T2 | 1 | ms | 100 |

Table A.7.7.X.2.2-2: CLI-RSSI accuracy OTA related test parameters

| Parameter | Unit | Value |
| --- | --- | --- |
| Angle of arrival configuration |  | Setup 1 defined A.3.15.1 |
| Beam assumption Note 5 |  | Fine |
| on CLI-RSSI measurement resource Note1 | dBm/15 kHzNote3 | -100 |
| on CLI-RSSI measurement resource Note1 | dBm/SCSNote3 | -91 |
| on CLI-RSSI measurement resource | dB | -Infinity |
| RSRP on CLI-RSSI measurement resource Note2 | dBm/SCS | -Infinity |
| BB on CLI-RSSI measurement resource Note4 | dB | -Infinity |
| Io on CLI-RSSI measurement resource Note2 | dBm/40.32 MHz Note3 | -65.44 |
| Io on CLI-RSSI measurement resource Note2 | dBm/1.08 MHz | -81.46 |
| NOTE 1: Where used, interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: SRS_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 4: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 36.101-2 [19], and an allowance of 2 dB for UE multi-band relaxation factor ∑MBP from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 5: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation. |  |  |

Table A.7.7.X.2.2-3: CLI-RSSI measurement resource configuration for FR2 CLI-RSSI accuracy

|  | Field | SRSConf.1 |
| --- | --- | --- |
| CLI-RSSI measurement resource | rssi-ResourceId | 0 |
|  | startPRB | 50 |
|  | nrofPRBs | 30 |
|  | startPosition | 3 |
|  | nrofSymbols | 11 |
|  | rssi-PeriodicityAndOffset | sl160, 9 |
| NOTE 1: startPRB is set such that CLI-RSSI measurement resource is fully contained within UL subband. |  |  |

##### A.7.7.X.2.3	Test Requirements

The CLI-RSSI measurement accuracy shall fulfil the absolute accuracy requirements in clauses 10.1.X.2.1. The UE is deemed to meet the requirement if the reported CLI-RSSI is in the range shown in table A.7.7.X.2.3-1.

Table A.7.7.X.2.3-1: L1-CLI-RSSI absolute accuracy test requirement

|  | Test requirement Notes1,2,3 |
| --- | --- |
|  | Io -δ +Gmin ≤ Reported CLI-RSSI(dBm) ≤Io +δ +Gmax |
| NOTE 1: Io is the equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone configured in the test for 1.08 MHz NOTE 2: δ is the L1-CLI-RSSI absolute accuracy requirement from Table 10.1.X.2.1-2, selected according to the Io used in the test NOTE 3: Gmin and Gmax are the minimum and maximum UE gain values from Table B.2.1.5.1-1, selected according to the UE power class |  |

<End of Change 2>