3GPP TSG-RAN4 Meeting # 117	R4-2522842

# Dallas, USA, 17th November – 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft big CR on performance requirements for NR_RRM_Ph5 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson, CATT |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_RRM_Ph5-Perf |  |  |  |  |  | Date: |  |  | 2025-11-25 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Update the performancet requirements concerning CSSFand FBS based on endorsed CRs in RAN4 #117 meeting. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Draft CRs endorsed in RAN4#117: T-doc; Company; Title. R4-2520810; Nokia, Nokia Shanghai Bell; draftCR on test case for intra-frequency measurement with CSSF solution 1. R4-2521326; OPPO; Draft CR on TC of inter-frequency measurement without MG with CSSF solution 3. R4-2520140; CATT; DraftCR Test cases requirements with FBS for RRC re-establishment and RRC connection release with redirection to NR. R4-2522824; Samsung; Draft CR on measurement delay reduction TC for SSB based Inter-frequency measurement with MG. R4-2520142; CATT; DraftCR Test case for EMR based fast normal SCell activation in FR1. R4-2522822; ZTE Corporation, Sanechips; Test case for EMR based direct SCell activation in FR1. R4-2521344; OPPO; DraftCR on test case for EMR based normal SCell activation in FR2. R4-2521800; Nokia; DraftCR Test case for direct SCell activation with EMR in FR2. R4-2522823; Ericsson; Draft CR on test case for PUCCH scell activation for UE supporting Rel-18 EMR. And BigCR endorsed in post RAN4 #116bis meeting R4-2514874; Ericsson, CATT; Draft big CR on performance requirements for NR_RRM_Ph5. |
| Consequences if not approved: |  | No test cases for the core requirements introduced by this WI. |  |  |  |  |  |  |  |  |
| Clauses affected: |  |  |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

#### A.5.6.1.X	EN-DC event triggered reporting test without gap under non-DRX for UE configured with cssf-Config

##### A.5.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2 based on the enhanced CSSFoutside,gap by measuring one serving carrier per FR2 band. Supported test configurations are shown in table A.5.6.1.X.1-1.

Table A.5.6.1.X.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | LTE FDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 4 | LTE TDD, 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are four cells in the test, E-UTRAN PCell (Cell 1), FR2 PSCell (Cell 2), FR2 SCell (Cell 3) and a FR2 neighbour cell (Cell 4) on the same frequency as the PSCell. All the FR2 cells are on the same FR2 band. The test parameters and applicability for Cell 1 are defined in A.3.7.2. The test parameters for the Cell 2, Cell 3 and Cell 4 are given in table A.5.6.1.X.1-2, A.5.6.1.X.1-3 and A.5.6.1.X.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PSCell and the frequency of the SCells respectively, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T1 the UE receives an RRC message by which the PSCell and SCells (Cell 2 and Cell 3) becomes configured on NR. UE is also indicated to perform enhanced measurement by measuring one serving CC per FR2 band. During time duration T1, the UE shall not have any timing information of Cell 4.

Table A.5.6.1.X.1-2: General test parameters for intra-frequency event triggered reporting for EN-DC with TDD PSCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1~4 | E-UTRAN PCell (Cell 1) PSCell (Cell 2) SCell (Cell 3) |  |
| Neighbour cell |  | 1~4 | Cell 4 | Cell to be identified. |
| RF Channel Number |  | 1~4 | 1: Cell 1 2: Cell 2 and Cell 4 3: Cell 3 | One TDD carrier frequency is used for the NR PSCell and neighbor cell. The other TDD carrier frequency is used for SCell on the same band with PSCell, One TDD or FDD carrier frequency is used for E-UTRAN cell. |
| SMTC configuration |  | 1~4 | SMTC.1 |  |
| A3-Offset | dB | 1~4 | -11 |  |
| CP length |  | 1~4 | Normal |  |
| Hysteresis | dB | 1~4 | 0 |  |
| Time To Trigger | s | 1~4 | 0 |  |
| Filter coefficient |  | 1~4 | 0 | L3 filtering is not used |
| DRX |  | 1~4 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1~4 | 3 s | Synchronous EN-DC |
| Time offset between Cell 2 and Cell 3 |  | 1~4 | 3 s | Synchronous cells |
| Time offset between Cell 2 and Cell 4 |  | 1~4 | 3 s | Synchronous cells |
| T1 | s | 1~4 | 5 |  |
| T2 | s | 1~4 | 5 |  |

Table A.5.6.1.X.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for EN-DC with TDD PSCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 2/Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1~4 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1~4 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  | 1,2 | 24 |  | 24 |  |
|  |  | 3,4 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1~4 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1~4 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1~4 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1~4 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1,2 | SR.3.2 TDD |  | N/A |  |
|  |  | 3,4 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1,2 | CR.3.1 TDD |  | N/A |  |
|  |  | 3,4 | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1,2 | CCR.3.1 TDD |  | N/A |  |
|  |  | 3,4 | CCR.3.7 TDD |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1~4 | 120 |  | 120 |  |
| OCNG Patterns |  | 1~4 | OP.5 |  | N/A |  |
| TRS configuration |  | 1~4 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  | 1~4 | TCI.State.2 |  | N/A |  |
| cellIndividualOffset | dB | 1~4 | N/A |  | 16 |  |
| SSB configuration |  | 1, 2 | SSB.3 FR2 |  | SSB.7 FR2 |  |
|  |  | 3, 4 | SSB.4 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1~4 | No external noise (Note 1) |  | No external noise (Note 1) |  |
| NOTE 1: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |

Table A.5.6.1.X.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for EN-DC with TDD PSCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 2/Cell 3 |  | Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  | 1~4 | Setup 3 defined in A.3.15.3 |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |
| Assumption for UE beamsNote 4 |  | 1~4 | Rough |  | Rough |  |
|  | dBm/SCS | 1, 2 | -89 | -89 | -Infinity | -89 |
|  |  | 3, 4 | -86 | -86 | -Infinity | -86 |
| BB Note 5 | dB | 1~4 | -0.12 | -0.12 | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1, 2 | -89 | -89 | -Infinity | -89 |
|  |  | 3, 4 | -86 | -86 | -Infinity | -86 |
|  | dBm/95.04 MHz | 1,2 | -64.41 | -64.41 | -Infinity | -64.41 |
|  |  | 3,4 | -61.41 | -61.41 | -Infinity | -61.41 |
| Time multiplexing of the downlink transmissions from each AoA |  | 1~4 | Defined in figure A.5.6.1.1.1-1 |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Void NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in B.2.1.3, and does not limit UE implementation or test system implementation. NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |

Figure A.5.6.1.X.1-1: Time multiplexed downlink transmissions (Config 1,2 example)

##### A.5.6.1.X.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4 s for a UE supporting power class 1,

-	1.44 s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.5.6.2.X	EN-DC: event triggered reporting tests under non-DRX

##### A.5.6.2.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and FR1 only EN-DC 3-searcher capability makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.5.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

##### A.5.6.2.X.2	Test parameters

In this test, LTE cell 1 as PCell in FR1 on LTE RF channel 1, NR cell 2 as PSCell in FR1 on NR RF channel 2, NR cell 3 as SCell in FR1 on NR RF channel 3. NR cell 4 as neighbour cell in FR1 on NR RF channel 4 which is in the same band as NR Cell 2.  and The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 2 and Cell 4 should be different in frequency location within the cell bandwidth.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.5.6.2.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 2 | LTE cell: LTE FDD FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.5.6.2.X.2-2: General test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
|  |  |  | Test 2 |  |
| NR RF Channel Number |  | Config 2 | 2,3,4 |  |
| E-UTRA RF Channel Number |  | Config 2 | 1 | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 2 | LTE cell 1 (PCell), NR cell 2(PSCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 2 | NR cell 4 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 2 | SSB.1 FR1 | As specified in clause A.3.10.1 for FR1 and A.3.10.2 for FR2 |
| A3-Offset | dB | Config 2 | -6 |  |
| Hysteresis | dB | Config 2 | 0 |  |
| A3-Threshold | dBm | Config 2 | -11 |  |
| CP length |  | Config 2 | Normal |  |
| TimeToTrigger | s | Config 2 | 0 |  |
| Filter coefficient |  | Config 2 | 0 | L3 filtering is not used |
| DRX |  | Config 2 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 2 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 2 | 3s | Synchronous cells |
| T1 | s | Config 2 | 5 |  |
| T2 | s | Config 2 | 1.5 |  |

Table A.5.6.2.X.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  | Config 2 | Not Applicable |  | Not Applicable |  | Not Applicable |  |
| BWchannel |  | MHz | Config 2 |  |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  | Config 2 |  |  | 52 |  | 52 |  |
| BWP BW |  | MHz | Config 2 |  |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  | Config 2 |  |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 2 |  |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 2 |  |  | CR.1.1 TDD |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 2 |  |  | CCR.1.1 TDD |  | - |  |
| SMTC configuration defined |  |  | Config 2 |  |  | SMTC.2 |  | SMTC.5 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 2 |  |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 2 |  |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 2 |  |  | -94 |  | -Infinity | -91 |
| SSB_RP Note 3 |  | dBm/SCS |  |  |  | -94 |  | -Infinity | -91 |
|  |  | dB | Config 2 |  |  | 4 |  | -Infinity | 7 |
| Ês /Noc |  | dB | Config 2 |  |  | 4 |  | -Infinity | 7 |
| Io |  | dBm/9.36 MHz | Config 2 |  |  | -64.60 |  | -Infinity | -64.60 |
| Propagation Condition |  |  | Config 2 |  |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

##### A.5.6.2.X.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.7.6.1.Y	SA event triggered reporting test without gap under non-DRX for UE configured with cssf-Config

##### A.7.6.1.Y.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the TDD intra-frequency cell search requirements in clause 9.2.5.1 and 9.2.5.2 based on the enhanced CSSFoutside,gap by measuring one serving carrier per FR2 band. Supported test configurations are shown in table A.7.6.1.Y.1-1.

Table A.7.6.1.Y.1-1: supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | 240 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE: The UE is only required to be tested in one of the supported test configurations. |  |

There are four cells in the test, one FR1 PCell (Cell 1), two FR2 SCells (Cell 2 and Cell 3) and a FR2 neighbor cell (Cell 4). All the FR2 cells i.e. Cell2, Cell3 and Cell4 are on the same band. The FR2 neighbor cell is on the same frequency as one FR2 SCell (Cell 2). The test parameters for Cell 1 are defined in A.6.6.1. The test parameters for the Cell 2, Cell 3 and Cell 4 are given in table A.7.6.1.Y.1-2, A.7.6.1.Y.1-3 and A.7.6.1.Y.1-4 below.

In the measurement control information, a measurement object is configured for the frequency of the PCell and two SCells respectively, and it is indicated to the UE that event-triggered reporting with Event A3 is used. In addition, UE is indicated to perform the measurement on the SCC corresponding to Cell 2 via cssf-MeasMO-List.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T1 the UE receives an RRC message by which the SCells (Cell 2 and Cell 3) becomes configured on NR. UE is indicated to perform enhanced measurement by measuring the SCC as indicated by cssf-MeasMO-List per band. During time duration T1, the UE shall not have any timing information of Cell 4.

Table A.7.6.1.Y.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR1 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active cell |  | 1, 2 | PCell (Cell 1) SCell 1 (Cell 2) SCell 2 (Cell 3) |  |
| Neighbor cell |  | 1, 2 | Cell 4 | Cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 2: Cell 2 and Cell 4 3: Cell 3 | One TDD carrier frequency is used for the NR PCell. One TDD carrier frequency is used for the NR SCell1 (Cell2) and the neighbor cell (Cell 4). One TDD carrier frequency is used for the NR SCell2. Cell 1 is on one band. Cell 2, Cell 3 and Cell 4 are on the other band. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between Cell 1 and Cell 2 |  | 1, 2 | 3 s | Synchronous cells |
| Time offset between Cell 2 and Cell 3 |  | 1, 2 | 3 s | Synchronous cells |
| Time offset between Cell 2 and Cell 4 |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.1.Y.1-3: NR Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR1 without gap without DRX

| Parameter | Unit | Config | Cell2/Cell3 |  | Cell4 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
|  |  | 2 | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
|  |  | 2 | CCR.3.7 TDD |  | N/A |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  |
| cellIndividualOffset | dB | 1~2 | N/A |  | 16 |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1, 2 | No external noise (Note 1) |  | No external noise (Note 1) |  |
| NOTE 1: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |

Table A.7.6.1.Y.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR1 without gap without DRX

| Parameter | Unit | Config |  | Cell 2/3 |  |  | Cell 4 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 |  | T2 | T1 |  | T2 |
| AoA setup |  | 1, 2 |  | Setup 3 defined in A.3.15.3 |  |  |  |  |  |
|  |  |  |  | AoA1 |  |  | AoA2 |  |  |
| Beam assumptionNote 4 |  | 1,2 |  | Rough |  |  | Rough |  |  |
| Es | dBm/SCS | 1 |  | -89 |  | -89 |  | -Infinity | -89 |
|  |  | 2 |  | -86 |  | -86 |  | -Infinity | -86 |
| BB Note 5 | dB | 1, 2 |  | -0.12 |  | -0.12 |  | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1 |  | -89 |  | -89 | -Infinity |  | -89 |
|  |  | 2 |  | -86 |  | -86 | -Infinity |  | -86 |
|  | dBm/95.04 MHz | 1 |  | -64.41 |  | -64.41 | -Infinity |  | -64.41 |
|  |  | 2 |  | -61.41 |  | -61.41 | -Infinity |  | -61.41 |
| Time multiplexing of the downlink transmissions from each AoA |  |  | 1, 2 |  | Defined in figure A.7.6.1.1.1-1 |  |  |  |  |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Void NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |  |

Figure A.7.6.1.Y.1-1: Time multiplexed downlink transmissions (Config 1 example)

##### A.7.6.1.Y.2	Test Requirements

In the test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

-	2.4 s for a UE supporting power class 1,

-	1.44 s for a UE supporting power class 2, 3 and 4

The UE is not required to read the neighbour cell SSB index in this test.

The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled.

The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

### A.7.6.x	Inter-RAT Measurements

#### A.7.6.x.1	SA event triggered reporting test without gap under non-DRX for UE configured with [MeasuringoneCCperFR2band] in FR2 inter-band CA

##### A.7.6.x.x.1	Test purpose and Environment

The purpose of this test is to verify that if the UE indicates ‘nogap-noncsg’ via NeedForGapNCSG-InfoEUTRA for the inter-RAT measurement, the UE is configured by the network with [MeasuringoneCCperFR2band], the UE makes correct reporting of an event. This test will partly verify the SA inter-RAT cell search requirements in clause 9.4.8.3 and 9.4.8.4 based on the enhanced CSSFoutside,gap in clause 9.1.5.1 by measuring one serving carrier per FR2 band. Supported test configurations are shown in table A.7.6.x.x.1-1.

Table A.7.6.1.x.x-1: SA event triggered reporting tests without SSB index reading for FR2

| Configuration | Description |
| --- | --- |
| 1 | LTE FDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE TDD, NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations. |  |

In this test, there are three cells: NR cell 1 as PCell in FR2 on RF channel 1, NR cell 2 as SCell in FR2 on RF channel 2, which is within the same band with RF channel 1, and E-UTRA cell 3 as neighbour cell in FR1 on RF channel 3. The SSB of cell 3 is completely within UE’s active BWP BW. The PRBs containing SSB from cell 1 and cell 3 should be different in frequency location within the cell bandwidth. The test parameters for the cell 1 and cell 2 are given in table A.7.6.x.x.1-2, A.7.6.x.x.1-3 and A.7.6.x.x.1-4 below. The test parameters for cell 3 are given in tables A.7.6.x.x.1-5, A.7.6.x.x.1-6 and A.7.6.x.x.1-7.

The cell specific test parameters for E-UTRA Cell 1 as PCell are defined in clause A.3.7.2.2.

In the measurement control information, measurement objects are configured for the frequency of the PCell and SCells respectively, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. At the beginning of T1 the UE receives an RRC message by which the SCells (Cell 2 and Cell 3) becomes configured on NR. UE is also indicated to perform enhanced measurement by measuring one serving CC per band. During time duration T1, the UE shall have no timing information of Cell 2 and Cell 3.

Table A.7.6.x.x.1-2: General test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Value | Comment |
| --- | --- | --- | --- | --- |
| Active primary cell |  | 1, 2 | PCell (Cell 1) |  |
| Active secondary cell |  | 1, 2 | SCell (Cell 2) |  |
| Neighbour cell |  | 1, 2 | E-UTRA Cell 3 | Neighbour cell to be identified. |
| RF Channel Number |  | 1, 2 | 1: Cell 1 2: Cell 2 3: Cell 3 | NR Cell 1 is on RF channel number 1. NR Cell 2 is on channel number 2, which is within the same band with RF channel 1 in FR2. E-UTRA Cell 3 is on RF channel 3. Which is in FR 1. |
| SMTC configuration |  | 1, 2 | SMTC.1 |  |
| A3-Offset | dB | 1, 2 | -11 |  |
| CP length |  | 1, 2 | Normal |  |
| Hysteresis | dB | 1, 2 | 0 |  |
| Time To Trigger | s | 1, 2 | 0 |  |
| Filter coefficient |  | 1, 2 | 0 | L3 filtering is not used |
| DRX |  | 1, 2 | OFF |  |
| Time offset between serving and neighbour |  | 1, 2 | 3 s | Synchronous cells |
| T1 | s | 1, 2 | 5 |  |
| T2 | s | 1, 2 | 5 |  |

Table A.7.6.x.x.1-3: Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1, Cell 2 |  | Cell3 |  |
| --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 | T2 |
| TDD configuration |  | 1, 2 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel | MHz | 1, 2 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  | 1 | 24 |  | 24 |  |
|  |  | 2 | 48 |  | 48 |  |
| Intial BWP configuration |  | 1, 2 | DLBWP.0.1 ULBWP.0.1 |  | DLBWP.0.1 ULBWP.0.1 |  |
| Active DL BWP configuration |  | 1, 2 | DLBWP.1.1 |  | DLBWP.1.1 |  |
| Active UL BWP configuration |  | 1, 2 | ULBWP.1.1 |  | ULBWP.1.1 |  |
| RLM-RS |  | 1, 2 | SSB |  | SSB |  |
| PDSCH RMC configuration |  | 1 | SR.3.2 TDD |  | N/A |  |
|  |  | 2 | SR.3.3 TDD |  |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  | N/A |  |
|  |  | 2 | CR.3.2 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  | N/A |  |
|  |  | 2 | CCR.3.7 TDD |  | N/A |  |
| TRS configuration |  | 1, 2 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI states |  | 1, 2 | TCI.State.2 |  | N/A |  |
| PDSCH/PDCCH subcarrier spacing | kHz | 1, 2 | 120 |  | 120 |  |
| OCNG Patterns |  | 1, 2 | OP.5 |  | N/A |  |
| cellIndividualOffset | dB | 1~2 | N/A |  | 16 |  |
| SSB |  | 1 | SSB.1 FR2 |  | SSB.7 FR2 |  |
|  |  | 2 | SSB.2 FR2 |  | SSB.8 FR2 |  |
| Propagation Condition |  | 1, 2 | No external noise (Note 1) |  | No external noise (Note 1) |  |
| NOTE 1: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |

Table A.7.6.x.x.1-4: NR OTA Cell specific test parameters for intra-frequency event triggered reporting for SA with TDD PCell in FR2 without gap without DRX

| Parameter | Unit | Config | Cell 1 |  | Cell 2/Cell 3 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T1 |  | T2 |
| AoA setup |  | 1, 2 | Setup 3 defined in A.3.15.3 |  |  |  |  |
|  |  |  | AoA1 |  | AoA2 |  |  |
| Beam assumptionNote 4 |  | 1,2 | Rough |  | Rough |  |  |
| Es | dBm/SCS | 1 | -89 | -89 |  | -Infinity | -89 |
|  |  | 2 | -86 | -86 |  | -Infinity | -86 |
| BB Note 5 | dB | 1, 2 | -0.12 | -0.12 |  | -Infinity | -0.12 |
| SSB_RP | dBm/SCS | 1 | -89 | -89 | -Infinity |  | -89 |
|  |  | 2 | -86 | -86 | -Infinity |  | -86 |
|  | dBm/95.04 MHz | 1 | -64.41 | -64.41 | -Infinity |  | -64.41 |
|  |  | 2 | -61.41 | -61.41 | -Infinity |  | -61.41 |
| NOTE 1: The resources for uplink transmission are assigned to the UE prior to the start of time period T2. NOTE 2: Void NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBP from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |

##### A.7.6.x.x.2	Test Requirements

The UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

5120 for UE supporting power class 1, or

3200 for UE supporting other power class.

The UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.7.6.2.X	SA event triggered reporting tests for FR2 without SSB time index detection when DRX is not used (FR1+FR2 CA and LTE+ FR2 EN-DC) for UE supporting [CSSF enhancement for one CC measurement per-band]

##### A.7.6.2.X.1	Test Purpose and Environment

The purpose of this test is to verify that if UE supports interFrequencyMeas-NoGap-r16 and the flag interFrequencyConfig-NoGap-r16 is configured by the network, the UE makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4 for UE supporting [CSSF enhancement for one CC measurement per-band].

In this test, there are two tests with different configurations, the UE is only required to pass one of the two tests. The two tests are:

-	Test 1: FR1+FR2 CA configuration:  NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 and NR cell 3 as SCells in FR2 on NR RF channel 2 and NR RF channel 3, where NR RF channel 2 and NR RF channel 3 are in one same band, furthermore, NR cell 4 as neighbour cell in FR2 on NR RF channel 4.

-	Test 2: FR2 EN-DC configuration: LTE cell 1 as PCell in FR1 on LTE RF channel 1, NR cell 1 as PSCell in FR2 on NR RF channel 1 and NR cell 2 as SCells in FR2 on NR RF channel 2, where NR RF channel 1and NR RF channel 2 are in one same band, furthermore, NR cell 3 as neighbour cell in FR2 on NR RF channel 3.

In the measurement control information, SCells with only SSB based L3 measurement are configured, and a measurement object is configured for the frequency of the neigbor cell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 2.

Supported test configurations of tests  are shown in table A.7.6.2.X.1-1.

Table A.7.6.2.X.1-1 Event triggered reporting tests without SSB index reading

| Configuration | Description |
| --- | --- |
| 1 | FR1 cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE cell: LTE FDD FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

The test parameters of tests are shown in table A.7.6.2.X.1-2, A.7.6.2.X.1-3 and A.7.6.2.X.1-4 below.

Table A.7.6.2.X.1-2: General test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration |  | Value |  | Comment |
| --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | Config 2 | 4 | 3 | One NR FR1 and three NR FR2 carrier frequency is used. |
| E-UTRA RF Channel Number |  | N/A | Config 2 | N/A | 1 | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 1 | Config 2 | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) | LTE cell 1 (PCell), NR cell 1(PSCell), NR cell 2(SCell) | LTE Cell 1 is on E-UTRA RF channel number 1. NR Cell 1 is on NR RF channel number 1. NR Cell 2 is on NR RF channel number 2. NR Cell 3 is on NR RF channel number 3. |
| Neighbour cell |  | Config 1 | Config 2 | NR cell 4 | NR cell 3 | NR Cell 3 is on NR RF channel number 3. NR cell 4 is on NR RF channel number 4. |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | Config 2 | SSB.1 FR2 | SSB.1 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | Config 2 | 16 |  |  |
| Hysteresis | dB | Config 1 | Config 2 | 0 |  |  |
| A3-Threshold | dBm | Config 1 | Config 2 | -11 |  |  |
| CP length |  | Config 1 | Config 2 | Normal |  |  |
| TimeToTrigger | s | Config 1 | Config 2 | 0 |  |  |
| Filter coefficient |  | Config 1 | Config 2 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | Config 2 | OFF |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | Config 2 | 3 s |  | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | Config 2 | 3s |  | Synchronous cells |
| T1 | s | Config 1 | Config 2 | 5 |  |  |
| T2 | s | Config 1 | Config 2 | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC |  |

Table A.7.6.2.X.1-3: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 1

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  | Config 1 | N/A |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
|  |  |  |  |  | N/A |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  |  | Config 1 | N/A |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 1 | Not Applicable |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 1 | 52 |  | 66 |  | 66 |  |
| BWP BW |  |  | MHz | Config 1 | 52 |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1 | DLBWP.0.1 |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 1 | SR.1.1 FDD |  | - |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 1 | CR.1.1 FDD |  | - |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 1 | CCR.1.1 FDD |  | - |  | - |  |
| SMTC configuration defined |  |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 1 | -98 |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 1 | 16 |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 1 | -64.60 |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 1 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.7.6.2.X.1-4: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 2

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  | Config 2 |  |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
|  |  |  |  |  |  |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  |  | Config 2 |  |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  |  | Config 2 |  |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 2 |  |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 2 |  |  | Not Applicable |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | Config 2 |  |  | NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 2 |  |  | 66 |  | 66 |  |
| BWP BW |  |  | MHz | Config 2 |  |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  |  | Config 2 |  |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 2 |  |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| SMTC configuration defined |  |  |  | Config 2 |  |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 2 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 2 |  |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 2 |  |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 2 |  |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 2 |  |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 2 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

##### A.7.6.2.X.2	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

6400 for UE supporting power class 1, or

3840 for UE supporting other power class.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

3200 for UE supporting power class 1, or

1920 for UE supporting other power class.

In test 1 and test 2 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.7.6.2.Y	SA event triggered reporting tests for FR2 under non-DRX

##### A.7.6.2.Y.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and FR1+FR2 CA (PCell is FR1 only) 3-searcher capability makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.5.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

##### A.7.6.2.Y.2	Test parameters

In this test, NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR2 on NR RF channel 2. NR cell 3 as neighbour cell in FR2 on NR RF channel 3 which is in the same band as NR Cell 2.  and The SSB of Cell 2 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 2 and Cell 3 should be different in frequency location within the cell bandwidth.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.7.6.2.Y.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 3 | FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.7.6.2.Y.2-2: General test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 3 | 1,2,3 |  |
| E-UTRA RF Channel Number |  | NA | NA | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 3 | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 3 | NR cell 3 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 3 | SSB.1 FR1 SSB.1 FR2 | As specified in clause A.3.10.1 for FR1 and A.3.10.2 for FR2 |
| A3-Offset | dB | Config 3 | -6 |  |
| Hysteresis | dB | Config 3 | 0 |  |
| A3-Threshold | dBm | Config 3 | -11 |  |
| CP length |  | Config 3 | Normal |  |
| TimeToTrigger | s | Config 3 | 0 |  |
| Filter coefficient |  | Config 3 | 0 | L3 filtering is not used |
| DRX |  | Config 3 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 3 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 3 | 3s | Synchronous cells |
| T1 | s | Config 3 | 5 |  |
| T2 | s | Config 3 | 3 for power class 1, or 1.5 for other power class. |  |

Table A.7.6.2.Y.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  | Config 3 | N/A |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
| Beam AssumptionNote 7 |  |  |  | Config 3 | N/A |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  |  | Config 3 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 3 | FDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 3 | Not Applicable |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | Config 3 | 10: NPRB,c = 52 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP BW |  |  | MHz | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  |  | Config 3 | DLBWP.0.1 |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 3 | SR.1.1 FDD |  | SR.3.1 TDD |  | N/A |  |
| RMSI CORESET Reference |  |  |  | Config 3 | SR.3.1 TDD |  | CR.3.1 TDD |  | N/A |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 3 | CCR.1.1 FDD |  | CCR.3.1 TDD |  | N/A |  |
| SMTC configuration defined |  |  |  | Config 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 3 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 3 | -94 |  | -98 |  | -Infinity | -85 |
| SSB_RP Note 3 |  |  | dBm/SCS | Config 3 | -94 |  | -89 |  | -Infinity | -85 |
| BB Note 8 |  |  | dB | Config 3 | 4 |  | 3.45 |  | -Infinity | 7.45 |
| Io |  |  | dBm/95.04 MHz | Config 3 | N/A |  | -58.56 |  | -Infinity | -55.38 |
|  |  |  | dBm/9.36 MHz | Config 3 | -64.60 |  | N/A |  | N/A | N/A |
| Ês /Noc |  |  | dBl | Config 3 | 4 |  | 3.45 |  | -Infinity | 7.45 |
| Propagation Condition |  |  |  | Config 3 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

##### A.7.6.2.Y.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

2.4s for UE supporting power class 1, or

1.44s for UE supporting other power class.

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.6.6.1.X	SA event triggered reporting tests without gap under non-DRX (FR1 CA, FR1 EN-DC and FR1+FR2 CA) for UE supporting [FR1 only CA and FR1 only NR-DC 3-searcher capability] and/or [FR1+FR2 NR-DC (PCell is FR1 only) 3-searcher capability]

##### A.6.6.1.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE makes correct reporting of an event. This test will partly verify the intra-frequency cell search requirements in clauses 9.2.5.1 and 9.2.5.2 for UE supporting [FR1 only CA and FR1 only NR-DC 3-searcher capability] and/or [FR1+FR2 NR-DC (PCell is FR1 only) 3-searcher capability].

##### A.6.6.1.X.2	Test parameters

In this test, there are three tests with different configurations, The UE is required to pass only one of the three tests that matches its supported capabilities.. The three tests are:

-	Test 1: FR1 CA configuration:  NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 on NR RF channel 2,  NR cell 3 as SCell in FR1 on NR RF channel 3, where NR RF channel 1, NR RF channel 2 and NR RF channel 3 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR1 on NR RF channel 2 same as NR cell 2.

-	Test 2: FR1 EN-DC configuration: LTE cell 1 as PCell in FR1 on LTE RF channel 1, NR cell 1 as PSCell in FR1 on NR RF channel 2, NR cell 2 as SCell in FR1 on NR RF channel 3 and NR cell 3 as SCell in FR1 on NR RF channel 4, where NR RF channel 2, NR RF channel 3  and NR RF channel 4 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR1 on NR RF channel 2 same as NR cell 2.

-	Test 3: FR1+FR2 CA configuration:  NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR2 on NR RF channel 2,  NR cell 3 as SCell in FR2 on NR RF channel 3, where NR RF channel 1, NR RF channel 2 and NR RF channel 3 are in differnt bands, furthermore, NR cell 4 as neighbour cell in FR2 on NR RF channel 2 same as NR cell 2.

In the measurement control information, SCells with only SSB based L3 measurement are configured, and a measurement object is configured for the frequency of the neigbor cell, and it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration T1, the UE shall not have any timing information of Cell 4.

Supported test configurations of tess are shown in table A.6.6.1.X.1.2-1.

Table A.6.6.1.X.1.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FR1 cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 2 | LTE cell: LTE FDD FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| 3 | FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

The test parameters of tests are shown in table A.6.6.1.X.2-2, A.6.6.1.X.2-3, A.6.6.1.X.2-4  and A.6.6.1.X.2-5 below.

Table A.6.6.1.X.2-2: General test parameters for intra-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter | Unit | Test configuration |  |  | Value |  |  | Comment |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Test 1 | Test 2 | Test 3 | Test 1 | Test 2 | Test 3 |  |
| NR RF Channel Number |  | Config 1 | Config 2 | Config 3 | 3 | 3 | 3 |  |
| E-UTRA RF Channel Number |  | NA | Config 2 | NA | NA | 1 | NA | One E-UTRAN FDD carrier frequency is used. |
| Active cell |  | Config 1 | Config 2 | Config 3 | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) | LTE cell 1 (PCell), NR cell 1(PSCell), NR cell 2(SCell) , NR cell 3(SCell) | NR cell 1 (PCell), NR cell 2(SCell), NR cell 3(SCell) |  |
| Neighbour cell |  | Config 1 | Config 2 | Config 3 | NR cell 4 | NR cell 4 | NR cell 4 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | Config 2 | Config 3 | SSB.1 FR1 | SSB.1 FR1 | SSB.1 FR1 SSB.1 FR2 | As specified in clause A.3.10.2 |
| offsetMO | dB | Config 1 | Config 2 | Config 3 | 16 |  |  |  |
| Hysteresis | dB | Config 1 | Config 2 | Config 3 | 0 |  |  |  |
| A3-Threshold | dBm | Config 1 | Config 2 | Config 3 | -11 |  |  |  |
| CP length |  | Config 1 | Config 2 | Config 3 | Normal |  |  |  |
| TimeToTrigger | s | Config 1 | Config 2 | Config 3 | 0 |  |  |  |
| Filter coefficient |  | Config 1 | Config 2 | Config 3 | 0 |  |  | L3 filtering is not used |
| DRX |  | Config 1 | Config 2 | Config 3 | OFF |  |  | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | Config 2 | Config 3 | 3 s |  |  | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | Config 2 | Config 3 | 3s |  |  | Synchronous cells |
| T1 | s | Config 1 | Config 2 | Config 3 | 5 |  |  |  |
| T2 | s | Config 1 | Config 2 | Config 3 | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC | 5.2 for PC1; 3.5 for other PC |  |

Table A.6.6.2.X.2-3: Cell specific test parameters for intra-frequency event triggered reporting for FR2 without SSB time index detection, for test case 1

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  |  | Config 1 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 1 | FDD |  | FDD |  | FDD |  |
| TDD configuration |  |  |  | Config 1 | N/A |  | N/A |  | N/A |  |
| BWchannel |  |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  |  | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP BW |  |  | MHz | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  |  | Config 1 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | N/A |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | N/A |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 1 | SR.1.1 FDD |  | - |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 1 | CR.1.1 FDD |  | - |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 1 | CCR.1.1 FDD |  | - |  | - |  |
| SMTC configuration defined |  |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 1 | 15 |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 1 | -98 |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 1 | 16 |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz | Config 1 | -64.60 |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 1 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.6.6.2.X.2-4: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 2

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| TDD configuration |  |  |  | Config 2 | Not Applicable |  | Not Applicable |  | Not Applicable |  |
| BWchannel |  |  | MHz | Config 2 |  |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  |  | Config 2 |  |  | 52 |  | 52 |  |
| BWP BW |  |  | MHz | Config 2 |  |  | 52 |  | 52 |  |
| BWP configuration | Initial DL BWP |  |  | Config 2 |  |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 2 |  |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 2 |  |  | SR.1.1 TDD |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 2 |  |  | CR.1.1 TDD |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 2 |  |  | CCR.1.1 TDD |  | - |  |
| SMTC configuration defined |  |  |  | Config 2 |  |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 2 |  |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 2 |  |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 2 |  |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 2 |  |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 2 |  |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 2 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

Table A.6.6.2.X.1-5: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection, for test case 3

| Parameter |  |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2,3 |  | Neighbour Cell 4 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| AoA setup |  |  |  | Config 3 | N/A |  | Setup 3 as specified in clause A.3.15 |  | Setup 3 as specified in clause A.3.15 |  |
|  |  |  |  |  | N/A |  | AoA1 |  | AoA2 |  |
| Beam AssumptionNote 7 |  |  |  | Config 3 | N/A |  | Rough |  | Rough |  |
| NR RF Channel Number |  |  |  | Config 3 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  |  | Config 3 | FDD |  | TDD |  | TDD |  |
| TDD configuration |  |  |  | Config 3 | Not Applicable |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  |  | MHz | Config 3 | 10: NPRB,c = 52 |  | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  |  | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP BW |  |  | MHz | Config 3 | 52 |  | 66 |  | 66 |  |
| BWP configuration | Initial DL BWP |  |  | Config 3 | DLBWP.0.1 |  | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  |  | ULBWP.0.1 |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  |  | Config 3 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  |  | Config 3 | SR.1.1 FDD |  |  |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  |  | Config 3 | CR.1.1 FDD |  |  |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  |  | Config 3 | CCR.1.1 FDD |  |  |  | - |  |
| SMTC configuration defined |  |  |  | Config 3 | SMTC.1 |  | SMTC.1 |  | SMTC.1 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | Config 3 | 15 |  | 120 |  | 120 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  | Config 3 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |
| Ês |  |  | dBm/SCS | Config 3 | -98 |  | -87 |  | -Infinity | -87 |
| SSB_RP Note 3 |  |  | dBm/SCS |  |  |  |  |  |  |  |
|  |  |  | Note 5 |  |  |  |  |  |  |  |
| BB Note 8 |  |  | dB | Config 3 | 16 |  | 14.69 |  | -Infinity | 14.69 |
| Io |  |  | dBm/9.36 MHz dBm/95.04 MHz Note 5 | Config 3 | -64.60 |  | -58.01 |  | -Infinity | -58.01 |
| Propagation Condition |  |  |  | Config 3 |  |  | No external noise (Note 9) |  | No external noise (Note 9) |  |
|  |  | NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 8: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. NOTE 9: The downlink connection between the System Simulator and the UE is without Additive White Gaussian Noise, and has no fading or multipath effects as specified in TS 38.521-2 B.0 [40]. |  |  |  |  |  |  |  |  |

##### A.6.6.1.X.3	Test Requirements

In test 1, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 2, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 3, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X ms from the beginning of time period T2, where X is

1600 for UE supporting power class 1, or

960 for UE supporting other power class.

In test 1, test 2 and test 3 UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.6.6.2.X	SA event triggered reporting tests under non-DRX

##### A.6.6.2.X.1	Test purpose and Environment

The purpose of this test is to verify that the UE which supports interFrequencyMeas-Nogap-r16 and FR1 only CA and FR1 only NR-DC 3-searcher capability or FR1+FR2 CA (PCell is FR1 only) 3-searcher capability makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search without measurement gap requirements in clause 9.3.9.

The UE is only required to pass one of the three tests in A.5.6.2.x for FR1 EN-DC, A.6.6.2.x for FR1 CA, A.7.6.2.Y for FR1 and FR2 CA.

##### A.6.6.2.X.2	Test parameters

In this test, NR cell 1 as PCell in FR1 on NR RF channel 1, NR cell 2 as SCell in FR1 on NR RF channel 2. NR cell 3 as neighbour cell in FR1 on NR RF channel 3 which is in the same band as NR Cell 1. The SSB of Cell 3 is completely within UE’s active BWP BW. The RBs containing SSB from Cell 1 and Cell 3 should be different in frequency location within the cell bandwidth. Supported test configurations are shown in table A.6.6.2.X.2-1.

In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. During time duration

T1, the UE shall not have any timing information of NR Cell 2.

UE needs to be provided at least once every 500 ms with new Timing Advance Command MAC control element to restart the Time alignment timer to keep UE uplink time alignment. Furthermore, UE is allocated with PUSCH resource

at every DRX cycle.

Table A.6.6.2.X.2-1: Supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FR1 cell: NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode FR2 cell: 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations |  |

Table A.6.6.2.X.2-2: General test parameters for inter-frequency event triggered reporting for FR1 without SSB time index detection

| Parameter | Unit | Test 1 | Value | Comment |
| --- | --- | --- | --- | --- |
| NR RF Channel Number |  | Config 1 | 1,2,3 |  |
| E-UTRA RF Channel Number |  | NA | NA |  |
| Active cell |  | Config 1 | NR cell 1 (PCell), NR cell 2(SCell), |  |
| Neighbour cell |  | Config 1 | NR cell 3 |  |
| SMTC-SSB parameters on NR RF Channels |  | Config 1 | SSB.1 FR1 | As specified in clause A.3.10.1 for FR1 |
| A3-Offset | dB | Config 1 | -6 |  |
| Hysteresis | dB | Config 1 | 0 |  |
| A3-Threshold | dBm | Config 1 | -11 |  |
| CP length |  | Config 1 | Normal |  |
| TimeToTrigger | s | Config 1 | 0 |  |
| Filter coefficient |  | Config 1 | 0 | L3 filtering is not used |
| DRX |  | Config 1 | OFF | DRX is not used |
| Time offset between serving and neighbour cells |  | Config 1 | 3 s | Synchronous NR CA Synchronous EN-DC |
|  |  | Config 1 | 3s | Synchronous cells |
| T1 | s | Config 1 | 5 |  |
| T2 | s | Config 1 | 1.5 |  |

Table A.6.6.2.X.2-3: Cell specific test parameters for inter-frequency event triggered reporting for FR2 without SSB time index detection

| Parameter |  | Unit | Test configuration | Serving cell 1 |  | Serving cell 2 |  | Neighbour Cell 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 | T1 | T2 |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2,3 |  | 4 |  |
| Duplex mode |  |  | Config 1 | FDD |  | FDD |  | FDD |  |
| TDD configuration |  |  | Config 1 | N/A |  | N/A |  | N/A |  |
| BWchannel |  | MHz | Config 1 | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  | 10: NPRB,c = 52 |  |
| Data PRBs allocated |  |  | Config 1 | 52 |  | 52 |  | 52 |  |
| BWP BW |  | MHz | Config 1 | 52 |  | 52 |  | N/A |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | Config 1 | OP.1 |  | OP.1 |  | OP.1 |  |
| PDSCH Reference |  |  | Config 1 | SR.1.1 FDD |  | - |  | - |  |
| measurement channel |  |  |  |  |  |  |  |  |  |
| RMSI CORESET Reference |  |  | Config 1 | CR.1.1 FDD |  | - |  | - |  |
| Channel |  |  |  |  |  |  |  |  |  |
| Dedicated CORESET RMC configuration |  |  | Config 1 | CCR.1.1 FDD |  | - |  | - |  |
| SMTC configuration defined |  |  | Config 1 | SMTC.2 |  | SMTC.2 |  | SMTC.5 |  |
| in A.3.11.1 and A.3.11.2 |  |  |  |  |  |  |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 15 |  | 15 |  | 15 |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |
| Ês |  | dBm/SCS | Config 1 | -94 |  | -94 |  | -Infinity | -91 |
| SSB_RP Note 3 |  | dBm/SCS | Config 1 | -94 |  | -94 |  | -Infinity | -91 |
| Ês /Noc |  | dBl | Config 1 | 4 |  | 4 |  | -Infinity | 7 |
|  |  | dB | Config 1 | 4 |  | 4 |  | -Infinity | 7 |
| Io |  | dBm/9.36 MHz | Config 1 | -64.60 |  | -64.60 |  | -Infinity | -58.01 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Void NOTE 3: SS B_RP, Es/Iot and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |  |

##### A.6.6.2.X.3	Test Requirements

In this test, the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than 1080 ms from the beginning of time period T2.

UE is not required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.7.3.1.X	Intra-frequency handover from FR2 to FR2; unknown target cell; for UE supporting fast beam sweeping

##### A.7.3.1.X.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 intra frequency handover for the UE supporting fast beam sweeping specified in clause 6.1.1.4.

##### A.7.3.1.X.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.X.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.X.2-2, and A.7.3.1.X.2-3.

The test scenario comprises of two cells on same carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting from T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.X.2-1: Intra-frequency handover from NR FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.X.2-2: General test parameters Intra-frequency handover from NR FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.X.2-3: Cell specific test parameters for NR FR2-FR2 Intra frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1 as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 1 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| Data PRBs allocated |  |  |  | 66 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  |  | SSB. 3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -104.7 |  |  |  |
| Note2 |  |  | dBm/SCS | -95.7 |  |  |  |
|  |  |  | dB | 6 | -1.8 | -Infinity | 0 |
|  |  |  | dB | 6 | 6 | -Infinity | 7 |
| IoNote3 |  |  | dBm/ BW | -59.7 | -56.7 | -59.7 | -56.7 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.3.1.X.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than （N*20+72）  ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90 %.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = （N*20+62） ms in the test where N is the UE reported value via capability [reduced BSF]. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of （N*20+72） ms.

==============Next change==============

#### A.7.3.1.Y	Inter-frequency handover from FR2 to FR2; unknown target cell; for UE supporting fast beam sweeping

##### A.7.3.1.Y.1	Test Purpose and Environment

This test is to verify the requirement for the NR FR2-NR FR2 inter frequency handover for the UE supporting fast beam sweeping specified in clause 6.1.1.4.

##### A.7.3.1.Y.2	Test Parameters

Supported test configurations are shown in table A.7.3.1.Y.2-1. Both handover delay and interruption length are tested by using the parameters in table A.7.3.1.Y.2-2, and A.7.3.1.Y.2-3.

The test scenario comprises of carriers and one cell on each carrier. No gap patterns are configured in the test case. The test consists of two successive time periods, with time durations of T1, T2 respectively. At the start of time duration T1, the UE does not have any timing information of cell 2. Starting T2, cell 2 becomes detectable and the UE receives a RRC handover command from the network. The start of T2 is the instant when the last TTI containing the RRC message implying handover is sent to the UE.

Table A.7.3.1.Y.2-1: Inter-frequency handover from FR2 to FR2 test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.1.Y.2-2: General test parameters Inter-frequency handover from FR2 to FR2

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 10 |  |

Table A.7.3.1.Y.2-3: Cell specific test parameters for NR FR2-FR2 Inter frequency handover test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| Data PRBs allocated |  |  |  | 66 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP.1 |  |  |  |
| SMTC Configuration |  |  |  | SMTC pattern 1 |  |  |  |
| SSB Configuration |  |  |  | SSB. 3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuraiton |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -104.7 |  | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  | -95.7 |  |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
| IoNote3 | Config 1,2 |  | dBm/ BW | -60.5 | -60.5 | -66.7 | -60.5 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.3.1.Y.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than（60*N+72）ms from the beginning of time period T2.

The rate of correct handovers observed during repeated tests shall be at least 90 %.

NOTE:	The handover delay can be expressed as: RRC procedure delay + Tinterrupt, where:

RRC procedure delay = 10 ms and is specified in clause 12 in TS 38.331 [2].

Tinterrupt = （60*N+62） ms in the test where N is the UE reported value via capability [reduced BSF]. Tinterrupt is defined in clause 6.1.1.4.2.

This gives a total of （60*N+72） ms.

==============Next change==============

##### A.7.3.2.1.7	Intra-frequency RRC Re-establishment in FR2 with UE capable of reduced beam sweeping factor

###### A.7.3.2.1.7.1	Test Purpose and Environment

The purpose is to verify that the NR intra-frequency RRC re-establishment delay in FR2 without known target cell is within the specified limits, where the UE is capable of reduced beam sweeping factor. These tests will verify the requirements in clause 6.2.1. While inter-frequency RRC re-establishment delay for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is not available to the UE in clause A.7.3.2.1.8, intra-frequency RRC re-establishment delay test case for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is still available to UE in this clause.

The test parameters are given in table A.7.3.2.1.7.1-1, table A.7.3.2.1.7.1-2 and table A.7.3.2.1.7.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure.

Table A.7.3.2.1.7.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.7.1-2: General test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 |  |
|  | Neighbour cells |  | 1 | Cell 2 |  |
| Final condition | Active cell |  | 1 | Cell 2 |  |
| RF Channel Number |  |  | 1 | 1 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1 | 3 |  |

Table A.7.3.2.1.7.1-3: Cell specific test parameters for NR intra-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NPRB,c = 66 |  |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| AoA setup |  | 1 | Setup 1 defined in A.3.15.1 |  |  | Setup 1 defined in A.3.15.1 |  |  |
|  | dB | 1 | -0.12 | -infinity | -infinity | -3.46 | 2 | 2 |
| Note2 | dBm/15 kHz | 1 | -104.7 |  |  |  |  |  |
| Note2 | dBm/SCS | 1 | -95.7 |  |  |  |  |  |
|  | dB | 1 | 4 | -infinity | -infinity | 2 | 2 | 2 |
| SS-RSRP Note3 | dBm/SCS | 1 | -91.7 | -infinity | -infinity | -93.7 | -93.7 | -93.7 |
| Io | dBm/95.04 MHz | 1 | -59.64 | -62.59 | -62.59 | -59.94 | -62.59 | -62.59 |
| Propagation Condition |  | 1 | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SS-RSRP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |  |

###### A.7.3.2.1.7.2	Test Requirements

The RRC re-establishment delay is defined as the time from the moment UE declares RLF, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR intra frequency cell shall be less than 3 s.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90 %.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 1.

Tidentify_intra_NR = N1*440 + Tproc ms, where N1 = reduced N and Tproc = 2 ms.

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target intra-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of:

- 1747 ms for N1=2, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case, or

- 2147 ms for N1=4, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case, or

- 2547 ms for N1=6, which allows 7.84 s (4.84 s + 3 s) from the beginning of T2 in the test case.

==============Next change==============

##### A.7.3.2.1.8	Inter-frequency RRC Re-establishment in FR2 without serving cell timing with UE capable of reduced beam sweeping factor

###### A.7.3.2.1.8.1	Test Purpose and Environment

The purpose is to verify that the NR inter-frequency RRC re-establishment delay in FR2 without serving cell timing is within the specified limits, when the UE is capable of reduced beam sweeping factor. These tests will verify the requirements in clause 6.2.1. While intra-frequency RRC re-establishment delay for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is still available to the UE in clause A.7.3.2.1.7, inter-frequency RRC re-establishment delay test case for UE capable of reduced beam sweeping factor is tested under the condition that the serving cell timing is not available to UE in this clause.

The test parameters are given in table A.7.3.2.1.8.1-1, table A.7.3.2.1.8.1-2 and table A.7.3.2.1.8.1-3 below. The test consists of 3 successive time periods, with time duration of T1, T2 and T3 respectively. At the start of time period T2, cell 1, which is the active cell, becomes inactive. The time period T3 starts after the occurrence of the radio link failure. During T1, the UE shall be configured with the carrier frequency of cell 2 (with RF Channel Number #2) to ensure that the UE has the context of the carrier frequency of cell 2 by the end of T1. During T1 and T2, cell 2 is not detectable to the UE.

Table A.7.3.2.1.8.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.1.8.1-2: General test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter |  | Unit | Test configuration | Value | Comment |
| --- | --- | --- | --- | --- | --- |
| Initial condition | Active cell |  | 1 | Cell 1 |  |
|  | Neighbour cells |  | 1 | Cell 2 |  |
| Final condition | Active cell |  | 1 | Cell 2 |  |
| RF Channel Number |  |  | 1 | 1, 2 |  |
| Time offset between cells |  |  | 1 | 3 s | Synchronous cells |
| N310 |  | - | 1 | 1 | Maximum consecutive out-of-sync indications from lower layers |
| N311 |  | - | 1 | 1 | Minimum consecutive in-sync indications from lower layers |
| T310 |  | ms | 1 | 0 | Radio link failure timer; T310 is disabled |
| T311 |  | ms | 1 | 5000 | RRC re-establishment timer |
| Access Barring Information |  | - | 1 | Not Sent | No additional delays in random access procedure. |
| SSB configuration |  |  | 1 | SSB.1 FR2 |  |
| SMTC configuration |  |  | 1 | SMTC pattern 1 |  |
| DRX cycle length |  | s | 1 | OFF |  |
| PRACH configuration |  |  | 1 | FR2 PRACH configuration 1 | Table A.3.8.3.1-1 |
| T1 |  | s | 1 | 5 |  |
| T2 |  | s | 1 | 4.84 | Time for the UE to detect RLF (Summation of TEvaluate_out_SSB defined in clause 8.1 in TS 38.133, T310 and the period for UE turns off transmitter defined in clause 8.1.5 in TS 38.133 ) |
| T3 |  | s | 1 | 5 |  |

Table A.7.3.2.1.8.1-3: Cell specific test parameters for NR inter-frequency RRC Re-establishment test case in FR2

| Parameter | Unit | Test configuration | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Assumption for UE beamsNote 4 |  |  | Rough |  |  | Rough |  |  |
| AoA setup |  | 1 | Setup 3 as specified in clause A.3.15 |  |  |  |  |  |
|  |  |  | AoA1 |  |  | AoA2 |  |  |
| TDD configuration |  | 1 | TDDConf.3.1 |  |  | TDDConf.3.1 |  |  |
| BWchannel | MHz | 1 | 100: NPRB,c = 66 |  |  | 100: NPRB,c = 66 |  |  |
| Data PRBs allocated |  | 1 | 24 |  |  | 24 |  |  |
| PDSCH RMC configuration |  | 1 | SR.3.1 TDD |  |  | N/A |  |  |
| RMSI CORESET RMC configuration |  | 1 | CR.3.1 TDD |  |  | CR.3.1 TDD |  |  |
| Dedicated CORESET RMC configuration |  | 1 | CCR.3.1 TDD |  |  | CCR.3.1 TDD |  |  |
| TRS configuration |  | 1 | TRS.2.1 TDD |  |  | N/A |  |  |
| PDSCH/PDCCH TCI state |  | 1 | TCI.State.2 |  |  | N/A |  |  |
| OCNG Pattern |  | 1 | OP.1 defined in A.3.2.1 |  |  | OP.1 defined in A.3.2.1 |  |  |
| Initial DL BWP configuration |  | 1 | DLBWP.0.1 |  |  | DLBWP.0.1 |  |  |
| Initial UL BWP configuration |  | 1 | ULBWP.0.1 |  |  | ULBWP.0.1 |  |  |
| RLM-RS |  | 1 | SSB |  |  | SSB |  |  |
| Note2 | dBm/15 kHz | 1 | -92.1 |  |  | -92.1 |  |  |
| Note2 | dBm/SCS | 1 | -83.1 |  |  | -83.1 |  |  |
|  | dB | 1 | 0 | -infinity | -infinity | -infinity | -infinity | 0 |
| BB Note 5 | dB | 1 | -1.01 | -infinity | -infinity | -infinity | -infinity | -1.01 |
| SSB_RP Note3 | dBm/SCS | 1 | -83.1 | -infinity | -infinity | -infinity | -infinity | -83.1 |
| Io | dBm/95.04 MHz | 1 | -55.5 | -58.51 | -58.51 | -58.51 | -58.51 | -55.5 |
| Propagation Condition |  | 1 | AWGN |  |  | AWGN |  |  |
| NOTE 1: OCNG shall be used such that a constant total transmitted power is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation NOTE 5: Calculation of Es/IotBB includes the effect of UE internal noise up to the value assumed for the associated Refsens requirement in clause 7.3.2 of TS 38.101-2 [19], and an allowance of 1 dB for UE multi-band relaxation factor ΔMBS from TS 38.101-2 [19] Table 6.2.1.3-4. |  |  |  |  |  |  |  |  |

###### A.7.3.2.1.8.2	Test Requirements

The RRC re-establishment delay is defined as the time from the start of time period T3, to the moment when the UE starts to send PRACH preambles to cell 2 for sending the RRCReestablishmentRequest message to cell 2.

The RRC re-establishment delay to an unknown NR inter frequency cell shall be less than the delay specified below for Tre-establish_delay.

The rate of correct RRC re-establishments observed during repeated tests shall be at least 90 %.

NOTE:	The RRC re-establishment delay in the test is derived from the following expression:

Tre-establish_delay= TUL_grant + TUE_re-establish_delay.

Where:

TUL_grant = It is the time required to acquire and process uplink grant from the target cell. The PRACH reception at the system simulator is used as a trigger for the completion of the test; hence TUL_grant is not used.

TUE_re-establish_delay=50 ms+Tidentify_intra_NR+i=1Nfreq-1Tidentify_inter_NR,i+TSI-NR+TPRACH

Nfreq = 2.

Tidentify_intra_NR = MAX (1000, (N1 x 10 x TSMTC + Tproc)) ms, where N1 = reduced N and Tproc = 2 ms.

Tidentify_inter_NR = MAX (1000, (N1 x 13 x TSMTC, i + Tproc)) ms, where N1 = reduced N and Tproc = 2 ms.

TSI = 1280 ms; it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target inter-frequency NR cell.

TPRACH = 15 ms; it is the additional delay caused by the random access procedure.

This gives a total of:

- 3345 ms for N1=2, which allows 5 s in the test case, or

- 3385 ms for N1=4, which allows 5 s in the test case, or

- 4107 ms for N1=6, which allows 5 s in the test case.

==============Next change==============

##### A.7.3.2.3.2	Redirection from NR in FR2 to NR in FR2 with UE capable of reduced beam sweeping factor

###### A.7.3.2.3.2.1	Test Purpose and Environment

This test is to verify RRC connection release with redirection from NR to NR requirements when the UE is capable of reduced beam sweeping factor, which are specified in clause 6.2.3.2.1.

###### A.7.3.2.3.2.2	Test Parameters

Supported test configurations are shown in table A.7.3.2.3.2.2-1. The time delay is tested by using the parameters in table A.7.3.2.3.2.2-2, and A.7.3.2.3.2.2-3.

The test consists of two successive time periods, with time duration of T1, and T2 respectively. The RRCRelease message shall be sent to the UE during period T1 and the start of T2 is the instant when the last TTI containing the RRC message is sent to the UE. Prior to time duration T2, the UE shall not have any timing information of Cell 2. Cell 2 is powered up at the beginning of the T2.

Table A.7.3.2.3.2.2-1: Redirection from NR to NR test configurations

| Config | Description |
| --- | --- |
| 1 | Source cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode Target cell: NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.3.2.3.2.2-2: General test parameters for Redirection from NR to NR test case

| Parameter |  | Unit | Value | Comment |
| --- | --- | --- | --- | --- |
| Initial conditions | Active cell |  | Cell 1 |  |
|  | Neighbouring cell |  | Cell 2 |  |
| Final condition | Active cell |  | Cell 2 |  |
| Filter coefficient |  |  | 0 | L3 filtering is not used |
| Access Barring Information |  | - | Not Sent | No additional delays in random access procedure. |
| Time offset between cells |  |  | 3 s | Synchronous cells |
| T1 |  | s | 5 |  |
| T2 |  | s | 3 |  |

Table A.7.3.2.3.2.2-3: Cell specific test parameters for Redirection from NR to NR test case

| Parameter |  |  | Unit | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| Assumption for UE beamsNote 6 |  |  |  | Rough |  | Rough |  |
| AoA setup |  |  |  | Setup 1as defined in A.3.15 |  |  |  |
| NR RF Channel Number |  |  |  | 1 |  | 2 |  |
| Duplex mode |  |  |  | TDD |  |  |  |
| TDD configuration |  |  |  | TDDConf.3.1 |  |  |  |
| BWchannel |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| BWP BW |  |  | MHz | 100: NPRB,c = 66 |  |  |  |
| Data PRBs allocated |  |  |  | 66 |  |  |  |
| DRX Cycle |  |  | ms | Not Applicable |  |  |  |
| PDSCH Reference measurement channel |  |  |  | SR3.1 TDD |  |  |  |
| RMSI CORESET Reference Channel |  |  |  | CR3.1 TDD |  |  |  |
| Control Channel RMC |  |  |  | CCR.3.1 TDD |  |  |  |
| OCNG Patterns |  |  |  | OP. 1 |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |
| SSB Configuration |  |  |  | SSB.3 FR2 |  |  |  |
| PDSCH/PDCCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PUCCH/PUSCH subcarrier spacing |  |  | kHz | 120 kHz |  |  |  |
| PRACH configuration |  |  |  | FR2 PRACH configuration 1 |  |  |  |
| TRS configuration |  |  |  | TRS.2.1 TDD |  |  |  |
| PDSCH/PDCCH TCI state |  |  |  | TCI.State.2 |  |  |  |
| BWP configuration |  | Initial DL BWP |  | DLBWP.0.1 |  |  |  |
|  |  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |
|  |  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |
|  |  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  | 0 |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  |  | dBm/15 kHz | -104.7 |  | -104.7 |  |
| Note2 |  |  | dBm/SCS | -95.7 |  | -95.7 |  |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
|  |  |  | dB | 5 | 5 | -Infinity | 5 |
| IoNote3 |  |  | dBm/95.04 MHz | -60.5 | -60.5 | -66.7 | -60.5 |
| Propagation condition |  |  | - | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 6: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

###### A.7.3.2.3.2.3	Test Requirements

The UE shall start to transmit the PRACH to Cell 2 less than the below specified delay Tconnection_release_redirect_NR from the beginning of time period T2.

The rate of correct RRC connection release redirection to NR observed during repeated tests shall be at least 90 %.

NOTE:	The redirection delay can be expressed as:

Tconnection_release_redirect_NR = TRRC_procedure_delay + Tidentify-NR + TSI-NR + TRACH,

where:

TRRC_procedure_delay = 110 ms in the test.

Tidentify-NR = MAX (880, N x 11 x Trs + Tproc) ms, where N = reduced beam sweeping factor, Trs = 20 ms, Tproc = 2 ms.

TSI-NR = 1280 ms, it is the time required for receiving all the relevant system information as defined in TS 38.331 for the target NR cell.

TRACH = 10 ms in the test.

This gives a total of:

-	2280 ms for N=2, allow 3 s in the test case for T2, or

-    2282 ms for N=4, allow 3 s in the test case for T2, or

-    2722 ms for N=6, allow 3 s in the test case for T2.

==============Next change==============

#### A.7.6.2.X1	SA serving cell quality triggered reporting tests for FR2 with SSB time index detection when DRX is used (PCell in FR2)

##### A.7.6.2.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the UE supporting [reduced RX beam sweeping factor capability] makes correct reporting of an event. This test will partly verify the SA inter-frequency NR cell search requirements in clause 9.3.4

In this test, there are two cells: NR cell 1 as PCell in FR2 on NR RF channel 1 and NR cell 2 as neighbour cell in FR2 on NR RF channel 2.  The test parameters and configurations are given in tables A.7.6.2.X1.1-1, A.7.6.2.X1.1-2, and A.7.6.2.X1.1-3.

Measurement gap pattern configuration defined in table A.7.6.2.X1.1-2 is provided for a UE that does not support per-FR gap, and no gap pattern (Gap Pattern Id and Measurement gap offset) is configured for a UE capable of per-FR gap.

[In the measurement control information, it is indicated to the UE that event-triggered reporting with Event A3 is used, serving cell’s RSRP and/or-RSRQ-triggered activating/deactivating multi-Rx for L3 measurement is used.] The test consists of two successive time periods, with time duration of T1, and T2 respectively.

During time duration T1, the UE shall not have any timing information of NR cell 2. Supported test configurations are shown in table A.7.6.X1.4.1-1.

UE needs to be provided with new Timing Advance Command MAC control element at least once during each time alignment timer period to maintain uplink time alignment. Furthermore UE is allocated with PUSCH resource at every DRX cycle.

Table A.7.6.2.X1.1-1: SA serving cell quality triggered reporting tests with SSB index reading for FR2-FR2

| Config | Description |
| --- | --- |
| 1 | 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.6.2.X1.1-2: General test parameters for SA serving cell quality triggered reporting for FR2 with SSB time index detection

| Parameter | Unit | Test configuration | Value |  | Comment |
| --- | --- | --- | --- | --- | --- |
|  |  |  | Test 1 | Test 2 |  |
| NR RF Channel Number |  | Config 1 | 1, 2 |  | Two FR2 NR carrier frequencies is used. |
| Active cell |  | Config 1 | NR cell 1 (PCell) |  | NR Cell 1 is on NR RF channel number 1. |
| Neighbour cell |  | Config 1 | NR cell 2 |  | NR cell 2 is on NR RF channel number 2. |
| Gap Pattern Id |  | Config 1 | 13 |  | As specified in clause 9.1.2-1. |
| Measurement gap offset |  | Config 1 | 39 |  |  |
| SMTC-SSB parameters |  | Config 1 | SSB.3 FR2 |  | As specified in clause A.3.10.2 |
| A3-Offset | dB | Config 1 | -6 |  |  |
| Hysteresis | dB | Config 1 | 0 |  |  |
| CP length |  | Config 1 | Normal |  |  |
| TimeToTrigger | s | Config 1 | 0 |  |  |
| Filter coefficient |  | Config 1 | 0 |  | L3 filtering is not used |
| DRX |  | Config 1 | DRX.1 | DRX.7 | As specified in clause A.3.3 |
| Time offset between serving and neighbour cells |  | Config 1 | 3s |  | Synchronous cells. |
| FBS-ThresholdP |  | 1, 2 | [-95] |  |  |
| FBS-ThresholdQ |  | 1, 2 | [-5] |  |  |
| T1 | s | Config 1 | 5 |  |  |
| T2 | s | Config 1 | 11 for PC1; 6.5 for other PC | 108 for PC1; 67 for other PC |  |

Table A.7.6.2.X1.1-3: Cell specific test parameters for SA serving cell quality triggered reporting with SSB time index detection

| Parameter |  | Unit | Test configuration | Cell 1 |  | Cell 2 |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T1 | T2 |
| AoA setup |  |  | Config 1 | [Setup 5 as specified in clause A.3.15] |  |  |  |
| Beam AssumptionNote 7 |  |  | Config 1 | Rough |  | Rough |  |
| NR RF Channel Number |  |  | Config 1 | 1 |  | 2 |  |
| Duplex mode |  |  | Config 1 | TDD |  | TDD |  |
| TDD configuration |  |  | Config 1 | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | Config 1 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| Data PRBs allocated |  |  | Config 1 | 66 |  | 66 |  |
| BWP BW |  | MHz | Config 1 | 100: NPRB,c = 66 |  | 100: NPRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | Config 1 | DLBWP.0.1 |  | N/A |  |
|  | Initial UL BWP |  |  | ULBWP.0.1 |  | N/A |  |
|  | Dedicated DL BWP |  |  | DLBWP.1.1 |  | N/A |  |
|  | Dedicated UL BWP |  |  | ULBWP.1.1 |  | N/A |  |
| OCNG Patterns defined in A.3.2.1.1 |  |  | Config 1 | OP.1 |  | OP.1 |  |
| PDSCH Reference measurement channel |  |  | Config 1 | SR.3.1 TDD |  | - |  |
| CORESET Reference Channel |  |  | Config 1 | CR.3.1 TDD |  | - |  |
| SMTC configuration defined in A.3.11.1 and A.3.11.2 |  |  | Config 1 | SMTC.1 |  | SMTC.1 |  |
| PDSCH/PDCCH subcarrier spacing |  | kHz | Config 1 | 120 |  | 120 |  |
| TRS configuration |  |  | Config 1 | TRS.2.1 TDD |  | N/A |  |
| PDSCH/PDCCH TCI state |  |  | Config 1 | TCI.State.2 |  | N/A |  |
| EPRE ratio of PSS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  | Config 1 | 0 |  | 0 |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |
| Note2 |  | dBm/15 kHz Note 5 |  | [-104.7] |  | [-104.7] |  |
| Note2 |  | dBm/SCS Note 4 | Config 1 | [-95.7] |  | [-95.7] |  |
| SS-RSRP Note 3 |  | dBm/SCS Note 5 | Config 1 | -110 | -110 | -Infinity | -86.7 |
|  |  | dB | Config 1 | [-14.3] | [-14.3] | -Infinity | 9 |
|  |  | dB | Config 1 | [-14.3] | [-14.3] | -Infinity | 9 |
| IoNote3 |  | dBm/95.04 MHz Note 5 | Config 1 | [-59.7] | [-59.7] | -66.7 | -57.2 |
| Propagation Condition |  |  | Config 1 | AWGN |  | AWGN |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 3: SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: Void NOTE 5: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 6: As observed with 0 dBi gain antenna at the centre of the quiet zone NOTE 7: Information about types of UE beam is given in clause B.2.1.3, and does not limit UE implementation or test system implementation |  |  |  |  |  |  |  |

##### A.7.6.2.X2.2	Test Requirements

In test 1 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X1 ms from the beginning of time period T2, where X1 is

1800 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 2.

3120 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 4.

4680 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 6.

In test 2 the UE shall send one Event A3 triggered measurement report, with a measurement reporting delay less than X2 ms from the beginning of time period T2, where X2 is

16640 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 2.

33280 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 4.

49920 for UE supporting [reduced RX beam sweeping factor capability], the values of which are 6.

In test 1 and 2 UE is required to report SSB time index. The UE shall not send event triggered measurement reports, as long as the reporting criteria are not fulfilled. The rate of correct events observed during repeated tests shall be at least 90 %.

NOTE:	The actual overall delays measured in the test may be up to 2xTTIDCCH higher than the measurement reporting delays above because of TTI insertion uncertainty of the measurement report in DCCH.

==============Next change==============

#### A.6.5.3.X1	EMR based SCell activation of unknown SCell in FR1

##### A.6.5.3.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based SCell activation delay is within the requirements stated in clause 8.3.2A, when the SCell in FR1 is unknown by the UE at the time of activation.

In the test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as SCell in FR1 on NR RF channel 2. The supported test configurations for PCell and SCell are provided in table A.6.5.3.X1.1-1 and table A.6.5.3.X1.1-1A respectively. The general test parameters are given in table A.6.5.3.X1.1-2 and cell-specific parameters for PCell and SCell are given in table A.6.5.3.X1.1-3 and table A.6.5.3.X1.1-4 respectively.

Table A.6.5.3.X1.1-1: supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.X1.1-1A: supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

The test consists of 6 successive time periods, with time duration of T1, T2, T3, T4, T5 and T6 respectively.

During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2. UE is configured with inter-frequency measurement reporting for Cell 2 in:

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or idleInactiveNR-MeasReport-r16 only, or

measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

Beam level reporting for early measurements is configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shal not be performed. Signal level of Cell 2 is set to the value given in table A.6.5.3.X1.1-4. The duration of T2 is set to a fixed value according to the table A.6.5.3.X1.1-2.

At the beginning of T3, the signal level of the Cell 2 is set to another value according to the table A.6.5.3.X1.1-4. The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

During T2 and T3, UE is in RRC_IDLE mode.

The time when TE sends the paging message is defined as the starting point of T4. During T4, the UE shall send a valid measurement report with SSB index of Cell 2 to the PCell.

At the beginning of T5 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. UE is only monitoring the PCC and shall be continuously scheduled in the PCell when UE is connected to PCell. Then the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T6. The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length, as defined in clause 8.3.2A. The UE shall start reporting CSI in PCell after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3 msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.6.5.3.X1.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two FR1 NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| SCell |  | Cell 2 | Secondary cell on NR RF channel number 2 configured by RRC during the test. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of reported idle/inactive and reselection measurements. |
| T4 | s | 1 | During this time UE shall send a valid measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 4 | During this time SCell (Cell 2) is configured via RRC signalling. |
| T6 | s | 1 | During this time UE completes the activation of SCell (Cell 2). |
| Filter coefficient |  | 0 | L3 filtering is not used |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length, where k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k1 should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.X1.1-3: Cell specific test parameters for NR PCell

| Parameter |  | Unit | Cell 1 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  |  |  |
|  | Config 3 |  | 10 |  |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |  |  |  |
|  | Config 3 |  | 5 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 19.08 MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 [26] clause 5.2.2.5. |  |  |  |  |  |  |  |  |

Table A.6.5.3.X1.1-4: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |  |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |  |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | N/A |  |  |  |  |  |
| reportQuantity |  |  | N/A |  |  |  |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |  |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -infinity | -87 | -97 |  |  |  |
|  | ConfigSCell 3 |  | -infinity | -84 | -94 |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -infinity | -87 | -97 |  |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36 MHz | -76.05 | -58.96 | -68.26 |  |  |  |
|  | ConfigSCell 3 | dBm/ 19.08 MHz | -69.96 | -52.87 | -62.17 |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |  |  |  |

##### A.6.5.3.X1.2	Test Requirements

The UE shall complete the SCell activation no later than at slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

The UE shall report non-zero CQI for SCell from slot n + THARQ+Tactivation_time+TCSI_ReportingNR slot length and onwards throughout time period T6.

Tactivation_time  is defined in clause 8.3.2A in TS 38.133 for FR1 SCell as Tactivation_time is TFirstSSB_MAX + Trs + 5ms.

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

==============Next change==============

#### A.6.5.3.X2	EMR based Direct SCell activation at SCell addition of unknown SCell in FR1

##### A.6.5.3.X2.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based direct SCell activation delay as defined in clause 8.3.2A, when the SCell in FR1 is unknown by the UE at the time of activation. The supported test configurations for NR PCell are shown in table A.6.5.3.X2.1-1. The supported test configurations for NR SCell are shown in table A.6.5.3.X2.1-1A. Test configurations for NR PCell and test configurations for NR SCell are chosen independently.

The test scenario comprises one PCell (Cell 1) and one SCell (Cell 2) as outlined in table A.6.5.3.X2.1-2. Cell-specific parameters for PCell and SCell are provided in table A.6.5.3.X2.1-3 and table A.6.5.3.X2.1-4, respectively.

There are two carriers, each with one cell. Cell 1 (PCell) is on RF channel 1 (PCC), and Cell 2 (SCell) is on RF channel 2 (SCC). Cell 1 and Cell 2 both operate according to one of the configurations in table A.6.5.3.X2.1-1 and table A.6.5.3.X2.1-1A respectively.

Before the test starts the UE is connected to Cell 1 on RF channel 1. The UE is only monitoring RF channel 1 and is not aware of Cell 2 on RF channel 2.

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4, and T5 respectively.

During T1, the UE is connected to Cell 1 only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting for Cell 2 in

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or

idleInactiveNR-MeasReport-r16 only, or

measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements-r18.

Beam level reporting for early measurements is configured. The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

During T2 and T3 the UE is in idle mode.

At the beginning of T2, Cell 2 becomes detectable however cell reselection shall not be performed. Signal level of Cell 2 is set to the value given in table A.6.5.3.X2.1-4. The time when T331 timer expires defines the ending point of T2.

At the beginning of T3, the signal level of the neighbour cell is set to turned off.  The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

The time when TE sends the paging message is defined as the starting point of T4. During T4, in this test the UE shall send measurement report with SSB index of Cell 2 to the PCell.

At the beginning of T5 the UE  receives the RRC connection reconfiguration message at the UE antenna connector. The corresponding slot at which the message is received at the UE antenna connector is denoted n. The UE shall complete activation of the SCell no later than in  slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengthNdirectNR slot length, as specified in clause 8.3.2A. From slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot lengthNdirectNR slot length and onwards the UE shall report valid CSI both for PCell and SCell.

The test equipment verifies the activation time by counting the slots between the RRC connection reconfiguration message is sent and until CSI report with non-zero CQI for both PCell and SCell is received.

Table A.6.5.3.X2.1-1: Supported test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, 10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, 10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30kHz SSB SCS, 40 MHz bandwidth, TDD duplex mode |

NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration,

Table A.6.5.3.X2.1-1A: Supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration. |  |

Table A.6.5.3.X2.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| NR RF Channel Number |  | 1, 2 | Two NR radio channels are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Inter-frequency neighbor cell (SCell to-be) |  | Cell 2 | Inter-frequency neighbor cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Measurement gap pattern |  | gp0 | Measurement gap is used during parts of time period T5 for detection of Cell 2. |
| CSI reporting periodicity | ms | 2 | CSI reporting periodicity for periodic reporting of CQI for PCell and, when added, SCell. |
| SCell measurement cycle (measCycleSCell) | ms | 160 | Measurement cycle for SCell does not come into effect in direct activation at SCell addition. |
| Timing offset between Cell 1 and Cell 2 | s | MRTD | The value of maximum timing offset depends upon the carrier aggregation scenario. |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of reported idle/inactive and reselection measurements. |
| T4 | s | 1 | During this time UE shall send a measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 5 | During this time SCell (Cell 2) is configured via RRC signalling and the UE completes the activation of SCell (Cell 2) |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| A3-offset | dB | -15 |  |
| THARQ | ms | k1×NR slot length | k1 is a number of slots indicated by the PDSCH-to-HARQ_feedback timing indicator field in a corresponding DCI format or provided by dl-DataToUL-ACK if the PDSCH-to-HARQ feedback timing field is not present in the DCI format, the value is defined in 38.213 [3] |
| TCSI_Reporting | ms | 2 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |
| k | ms |  | As specified in clause 4.3 of TS 38.213 [3] |

Table A.6.5.3.X2.1-3: NR Cell specific test parameters for NR PCell

| Parameter |  |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | T1 | T2 | T3 | T4 | T5 |
| Duplex mode |  | Config 1 |  | FDD |  |  |  |  |
|  |  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration |  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  |  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel |  | Config 1,2 | MHz | 10: NPRB,c = 52 |  |  |  |  |
|  |  | Config 3 |  | 40: NPRB,c = 106 |  |  |  |  |
| BWP configuration |  | Initial DL |  | DLBWP.0.1 |  |  |  |  |
|  |  | Initial UL |  | ULBWP.0.1 |  |  |  |  |
|  |  | Dedicated DL |  | DLBWP.1.1 |  |  |  |  |
|  |  | Dedicated UL |  | ULBWP.1.1 |  |  |  |  |
| TCI state |  |  |  | TCI.State.0 |  |  |  |  |
| CSI-RS configuration for CSI reporting |  | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |
| TRS Configuration |  | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel |  | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters |  | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| RMSI CORESET parameters |  | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  |  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  |  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Pattern |  |  |  | OP.1 |  |  |  |  |
| SSB Configuration |  | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  |  | Config 3 |  | SSB.2 FR1 |  |  |  |  |
| SMTC configuration |  |  |  | SMTC.1 |  |  |  |  |
| reportConfigType |  |  |  | periodic |  |  |  |  |
| reportQuantity |  |  |  | cri-RI-PMI-CQI |  |  |  |  |
| CSI reporting periodicity |  | Config 1,2 | slot | 5 |  |  |  |  |
|  |  | Config 3 |  | 10 |  |  |  |  |
| CSI reporting offset |  | Config 1,2 | slot | 3 |  |  |  |  |
|  |  | Config 3 |  | 5 |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |
| Noc Note2 | Config 1,2 |  | dBm/SCS | -104 |  |  |  |  |
|  | Config 3 |  |  | -101 |  |  |  |  |
| Ês/Iot |  |  | dB | 17 |  |  |  |  |
| Ês/Noc |  |  | dB | 17 |  |  |  |  |
| SS-RSRP Note3 | Config 1,2 |  | dBm/SCS | -87 |  |  |  |  |
|  | Config 3 |  |  | -84 |  |  |  |  |
| Io Note3 | Config 1,2 |  | dBm/9.36 MHz | -59.0 |  |  |  |  |
|  | Config 3 |  | dBm/38.16 MHz | -52.9 |  |  |  |  |
| Propagation condition |  |  |  | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  |  | 1x2 Low |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |

Table A.6.5.3.X2.1-4: NR Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 |  | T4 | T5 |  |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |  |  |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |  |  |  |  |
| TDD configuration | ConfigSCell 2 |  | TDDConf.1.1 |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |  |  |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | 10: NPRB,c = 52 |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | 40: NPRB,c = 106 |  |  |  |  |  |  |
| BWP configuration | Initial DL |  | DLBWP.0.1 |  |  |  |  |  |  |
|  | Initial UL |  | N/A |  |  |  |  |  |  |
|  | Dedicated DL |  | DLBWP.1.1 |  |  |  |  |  |  |
|  | Dedicated UL |  | N/A |  |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |  |
| CSI-RS configuration for CSI reporting | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |  |  |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | SR.1.1 FDD |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | SR.1.1 TDD |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | SR.2.1 TDD |  |  |  |  |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | CCR.1.1 FDD |  |  |  |  |  |  |
|  | ConfigSCell 2 |  | CCR.1.1 TDD |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | CCR.2.1 TDD |  |  |  |  |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |  |  |
|  | ConfigSCell 2 |  |  |  |  |  |  |  |  |
|  | ConfigSCell 3 |  |  |  |  |  |  |  |  |
| OCNG Pattern |  |  | OP.1 |  |  |  |  |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR1 |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR1 |  |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |  |
| reportConfigType |  |  | N/A |  |  |  |  |  |  |
| reportQuantity |  |  | N/A |  |  |  |  |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note1 |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note1 |  |  |  |  |  |  |  |  |  |
| Noc Note2 | ConfigSCell 1,2 | dBm/SCS | -98 |  |  |  |  |  | -104 |
|  | ConfigSCell 3 |  | -95 |  |  |  |  |  | -101 |
| Ês/Iot |  | dB | -infinity | 7 |  | -2 | -2 |  | 17 |
| Ês/Noc |  | dB | - infinity | 7 |  | -2 | -2 |  | 17 |
| SS-RSRP Note3 | ConfigSCell 1,2 | dBm/SCS | -infinity | -91 |  | -100 | -100 |  | -87 |
|  | ConfigSCell 3 |  | -infinity | -88 |  | -97 | -97 |  | -84 |
| Io Note3 | ConfigSCell 1,2 | dBm/9.36 MHz | -70.5 | -62.26 |  | -67.92 | -67.92 |  | -59.0 |
|  | ConfigSCell 3 | dBm/38.16 MHz | -63.96 | -56.17 |  | -61.83 | -61.83 |  | -52.9 |
| Propagation condition |  |  | AWGN |  |  |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 1x2 Low |  |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for Noc to be fulfilled. NOTE 3: SS-RSRP, SCH_RP, and Io levels have been derived from other parameters for information purpose. They are not settable parameters themselves. |  |  |  |  |  |  |  |  |  |

##### A.6.5.3.X2.2	Test Requirements

The UE shall complete the SCell activation no later than at slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

The UE shall report non-zero CQI for SCell from slot n + THARQ+Tactivation_time+TCSI_ReportingNR slot length and onwards throughout time period T5.

Tactivation_time  is defined in clause 8.3.2A in TS 38.133 for FR1 SCell as Tactivation_time is TFirstSSB_MAX + Trs + 5ms.

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

==============Next change==============

#### A.7.5.3.X	EMR based SCell activation of unknown SCell in FR2

##### A.7.5.3.X.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based SCell activation delay is within the requirements stated in clause 8.3.2A, when the SCell in FR2 is unknown by the UE at the time of activation.

In the test, there are two cells: NR Cell 1 as PCell in FR2 on NR RF channel 1 and NR Cell 2 as SCell in FR2 on NR RF channel 2. The supported test configurations are shown in table A.7.5.3.X.1-1 below. The general test parameters are described in Tables A.7.5.3.X.1-2, and cell specific test parameters are described in Tables A.7.5.3.X.1-3. OTA related test parameters are shown in table A.7.5.3.X.1-4 below.

The test consists of 6 successive time periods, with time duration of T1, T2, T3, T4, T5 and T6 respectively.

During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2. UE is configured with early measurement reporting for Cell 2 in:

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or idleInactiveNR-MeasReport-r16 only, or

measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

Beam level reporting for early measurements is configured. The time point when UE receives RRC_Release message from the TE defines as the starting point of T2.

At the beginning of T2, Cell 2 becomes detectable however no cell reselection is being performed. Signal level of Cell 2 is set to fixed value according to Table A.7.5.3.X.1-4. The duration of T2 is set to fixed value according to the Table A.7.5.3.X.1-2.

At the beginning of T3, the signal level of the Cell 2 is set to another value according to the table A.7.5.3.X.1-4. The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

During T2 and T3, UE is in RRC_IDLE mode.

The time when TE sends the paging message is defined as the starting point of T4. During T4, the UE shall send a valid measurement report with SSB index of Cell 2 to the PCell.

At the beginning of T5 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2. UE is only monitoring the PCC and shall be continuously scheduled in the PCell when UE is connected to PCell. Then the test equipment sends a MAC message for activation of the SCell.

The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T6.

The test equipment verifies the activation time by counting the slots from the time when the SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.7.5.3.X.1-1: Supported test configurations for EMR based SCell activation in FR2

| Configuration | Description |
| --- | --- |
| 1 | NR 120 kHz SSB SCS, 100 MHz bandwidth, TDD duplex mode |

Table A.7.5.3.X.1-2: General test parameters for EMR based SCell activation in FR2

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channels are used for this test. RF channel number 1 is in band 1 and RF channel number 2 is in band 2, where bands 1 and 2 are inter-band CA operating bands in FR2 as specified in Table 5.2A.2-1 in TS 38.101-2 [19]. |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| CQI/PMI periodicity and offset configuration index |  | 0 | CQI reporting for SCell every second subframe |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | ≤8 | A random value from 0s to 8s |
| T331 | s | 30 |  |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of reported idle/inactive and reselection measurements. |
| T4 | s | 1 | During this time UE shall send a valid measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 4 | During this time SCell (Cell 2) is configured via RRC signalling. |
| T6 | s | 1 | During this time UE completes the activation of SCell (Cell 2). |
| Filter coefficient |  | 0 | L3 filtering is not used |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2. |
| THARQ | ms | k1×NR slot length | k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] depends on UE’s capability |
| TCSI_Reporting | ms | 10 | the delay uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.X.1-3: Cell specific test parameters in Idle and Connected mode for EMR based SCell activation in FR2

| Parameter |  | Unit | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 | T1 | T2 | T3 | T4 | T5 | T6 |
| NR RF Channel Number |  |  | 1 |  |  |  |  |  | 1 |  |  |  |  |  |
| Duplex mode |  |  | TDD |  |  |  |  |  | TDD |  |  |  |  |  |
| TDD configuration |  |  | TDDConf.3.1 |  |  |  |  |  | TDDConf.3.1 |  |  |  |  |  |
| BWchannel |  | MHz | 100: NPRB,c = 66 |  |  |  |  |  | 100: NPRB,c = 66 |  |  |  |  |  |
| BWP BW |  | MHz | 100: NPRB,c = 66 |  |  |  |  |  | 100: NPRB,c = 66 |  |  |  |  |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  | DLBWP.0.1 |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  | DLBWP.1.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  | ULBWP.1.1 |  |  |  |  |  |
| TRS configuration |  |  | TRS.2.1 TDD |  |  |  |  |  | TRS.2.1 TDD |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  | TCI.State.0 |  |  |  |  |  |
| OCNG Patterns defined in A.3.2.1.1 (OP.1) |  |  | OP.1 |  |  |  |  |  | OP.1 |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| RMSI CORESET Parameters |  |  | CR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| Dedicated CORESET Parameters |  |  | CCR.3.1 TDD |  |  |  |  |  | NA |  |  |  |  |  |
| CSI-RS configuration |  |  | NA |  |  |  |  |  | CSI-RS.3.1 TDD Note 2 |  |  |  |  |  |
| CSI reporting periodicity Note 3 |  |  | NA |  |  |  |  |  | 5 |  |  |  |  |  |
| SSB parameters |  |  | SSB.1 FR2 |  |  |  |  |  | SSB.1 FR2 |  |  |  |  |  |
| SMTC configuration defined in A.3.11 |  |  | SMTC.1 |  |  |  |  |  | SMTC.1 |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  |  | 0 |  |  |  |  |  | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Propagation Condition |  |  | AWGN |  |  |  |  |  | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: CSI-RS for CSI measurement is (re)configured in the next DL slot after slot n+ TFirstSSB_MAX + 15*TSMTC_MAX during T2. NOTE 3: L1-RSRP measurement and reporting are configured to the the UE prior to the start of time period T1. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

Table A.7.5.3.X.1-4: OTA related test parameters for EMR based SCell activation in FR2

| Parameter |  | Unit |  |  | Cell 1 |  |  |  |  |  | Cell 2 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 | T6 | T1 | T2 | T3 | T4 | T5 | T6 |
| Angle of arrival configuration |  |  | Setup 3 defined in clause A.3.15.1 |  |  |  |  |  | Setup 3 defined in clause A.3.15.1 |  |  |  |  |  |
|  |  |  |  |  | AoA1 |  |  |  |  |  | AoA2 |  |  |  |  |  |
| Assumption for UE beams Note 7 |  |  | Rough |  |  |  |  |  | Rough |  |  |  |  |  |
| Note 1 | Config 1,2,3 | dBm/15 kHz | -104.7 |  |  |  |  |  | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 | -104.7 |
| Note 1 | Config 1,2,3 | dBm/SCS | -95.7 |  |  |  |  |  | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 | -95.7 |
|  | Config 1,2,3 | dB | 7 |  |  |  |  |  | -infinity | 7 | -0.5 | -0.5 | -0.5 | - 0.5 |
|  | Config 1,2,3 | dB | 7 |  |  |  |  |  | -infinity | 7 | -0.5 | -0.5 | -0.5 | -0.5 |
| SSB_RPNote 2, Note 4 | Config 1,2,3 | dBm/SCS | -88.7 |  |  |  |  |  | -infinity | -88.7 | -96.5 | -96.5 | -96.5 | -96.5 |
| IoNote 2, Note 4 | Config 1,2,3 | dBm/95.04 MHz | -58.92 |  |  |  |  |  | -66.68 | -58.92 | -69.3 | -69.3 | -69.3 | -69.3 |
| NOTE 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. NOTE 2: Es/Iot, SSB_RP and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 3: Void NOTE 4: Equivalent power received by an antenna with 0 dBi gain at the centre of the quiet zone NOTE 5: Void NOTE 6: Void NOTE 7: Information about types of UE beam is given in clause B.2.1.3 and does not imit UE implementation or test system implementation. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

##### A.7.5.3.X.2	Test Requirements

During T6, the UE shall start reporting CSI after at least one CSI-RS transmission occasion for channel measurement plus nCSI_ref  slots, as defined in 5.2.2.5 in [26], and reporting after slot n+THARQ+3 msNR slot length and shall report CQI index 0 (out-of-range) until the SCell activation has been completed.

During T6, the UE shall be able to send CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot n+THARQ+Tactivtion_time+TCSI_ReportingNR slot length as defined in clause 8.3.2A, where

-		THARQ is defined in Table A.7.5.3.X.1-2

-		Tactivation_time = TFirstSSB_MAX + 15*TSMTC_MAX + max(Tuncertainty_MAC + 5ms + TFineTiming, Tuncertainty_RRC + TRRC_delay-THARQ), which allows 350 ms.

- For this test case, TFirstSSB_MAX=TSMTC_MAX=Trs=20 ms.

-		TCSI_Reporting = 10 ms

-		NR slot length is 0.125 ms for this test case.

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

==============Next change==============

#### A.7.5.3.X1	EMR based SCell activation of unknown SCell in FR2 in RRC Inactive

##### A.7.5.3.X1.1	Test Purpose and Environment

The purpose of this test is to verify that the EMR (early measurement reporting) based direct SCell activation delay is within the requirements stated in clause 8.3.2A, when the SCell in FR2 is unknown by the UE but the EMR measurements are available  at the time of activation.

In the test, there are two cells: NR Cell 1 as PCell in FR1 on NR RF channel 1 and NR Cell 2 as SCell in FR2 on NR RF channel 2. The supported test configurations for PCell and SCell are provided in table A.7.5.3.X1.1-1 and table A.7.5.3.X1.1-1A respectively. The general test parameters are given in table A.7.5.3.X1.1-2 and cell-specific parameters for PCell and SCell are given in table A.7.5.3.X1.1-3 and table A.7.5.3.X1.1-4 respectively.

Table A.7.5.3.X1.1-1: supported test configurations for NR PCell

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.7.5.3.X1.1-1A: supported test configurations for NR SCell

| ConfigSCell | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

The test consists of 5 successive time periods, with time duration of T1, T2, T3, T4, and T5  respectively.

During T1, the UE is connected to Cell 1 (PCell) only and shall not have any timing information of Cell 2. UE is configured with inter-frequency measurement reporting for Cell 2 in:

- measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18 or idleInactiveNR-MeasReport-r16 only, or

- measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

- The UE is configured with beam level reporting for early measurements.

The time point when UE receives RRC_Release message from the TE defines the starting point of T2.

At the beginning of T2, Cell 2 becomes detectable but the cell reselection threshold is not occurring. Signal level of Cell 2 is set to the value given in table A.7.5.3.X1.1-4. The duration of T2 is set to a fixed value according to the table A.7.5.3.X1.1-2.

At the beginning of T3, the signal level of the Cell 2 is set to a value according to the table A.7.5.3.X1.1-4.

The duration of the T3 equals to measIdleValidityDuration-r18 or measReselectionValidityDuration-r18 depending on the UE capabilities of the UE under test.

During T2 and T3, UE is in RRC_IDLE mode.

The time when TE sends the paging message is defined as the starting point of T4.

During T4, the UE shall send a valid measurement report with SSB index of Cell 2 to the PCell in RRCResumeComplete message

At the beginning of T5 the UE receives an RRC message by which the SCell (Cell 2) becomes configured on radio channel 2.

T5: At the beginning of T5, the UE receives RRC message for direct addition and activation of the SCell (Cell 2). The point in time at which the RRC message is received at the UE antenna connector is denoted as slot #n.

The UE shall be able to report valid CSI in PCell for the activated SCell at latest in slot n+Ndirect/(NR slot length) where Ndirect = TRRC_Process + T1 + Tactivation_time + TCSI_Reporting - 3 ms (TCI state is not indicated within Tactivation_time) or

Ndirect = TRRC_Process + T1 + THARQ + Tactivation_time + TCSI_Reporting (TCI state is not indicated within Tactivation_time).

Table A.7.5.3.X1.1-2: General test parameters

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1, 2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1 FR1 |
| SCell |  | Cell 2 | Secondary cell on NR RF channel number 2 FR2 configured by RRC during the test. |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | ms | 0 |  |
| Time alignment error between cell2 and cell1 | ms | £ Time alignment error as specified in TS 38.104 [13] clause 7.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| T1 | s | 10 | During this time UE connects to PCell and early measurement report on the carrier frequency of Cell 2 is configured. |
| T2 | s | 30 | During this time UE is in RRC_IDLE state and Cell 2 becomes detectable. |
| T3 | s | 5 | During this time UE determines the validity of idle/inactive and reselection measurements by performing validity check, or if the measurements meet the accuracy requirements. |
| T4 | s | 1 | During this time UE shall send a valid measurement report with SSB index of Cell 2 to the PCell. |
| T5 | s | 5 | During this time SCell (Cell 2) is configured via RRC signalling and the UE completes the activation of SCell (Cell 2). |
| Filter coefficient |  | 0 | L3 filtering is not used |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| PRACH configuration index |  | 102 | The detailed configuration is specified in TS 38.211 clause 6.3.3.2. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length, where k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k1 should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | The delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.7.5.3.X1.1-3: Cell specific test parameters for NR PCell

| Parameter |  | Unit | Cell 1 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |
|  | Config 3 |  | SSB.2 FR2 |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |
| CSI reporting periodicity | Config 1,2 | slot | 5 |  |  |  |  |
|  | Config 3 |  | 10 |  |  |  |  |
| CSI reporting offset | Config 1,2 | slot | 3 |  |  |  |  |
|  | Config 3 |  | 5 |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |  |  |  |  |
|  | Config 3 | dBm/ 19.08 MHz | -52.87 |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  | - | 2x2 Low |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[2] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in TS 38.214 [26] clause 5.2.2.5. |  |  |  |  |  |  |  |

Table A.7.5.3.X1.1-4: Cell specific test parameters for NR SCell

| Parameter |  | Unit | Cell 2 |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T4 | T5 |
| Duplex mode | ConfigSCell 1 |  | FDD |  |  |  |  |
|  | ConfigSCell 2,3 |  | TDD |  |  |  |  |
| TDD configuration | ConfigSCell 1 |  | Not applicable |  |  |  |  |
|  | ConfigSCell 2 |  | TDDConf.1.1 |  |  |  |  |
|  | ConfigSCell 3 |  | TDDConf.2.1 |  |  |  |  |
| BWchannel | ConfigSCell 1,2 | MHz | Note 7 |  |  |  |  |
|  | ConfigSCell 3 |  | Note 7 |  |  |  |  |
| BWoccupied | ConfigSCell 1,2 | RB | 52 Note 5 |  |  |  |  |
|  | ConfigSCell 3 |  | 106 Note 6 |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.1 |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |
| TRS Configuration | ConfigSCell 1 |  | TRS.1.1 FDD |  |  |  |  |
|  | ConfigSCell 2 |  | TRS.1.1 TDD |  |  |  |  |
|  | ConfigSCell 3 |  | TRS.1.2 TDD |  |  |  |  |
| PDSCH Reference measurement channel | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| Dedicated CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| RMSI CORESET parameters | ConfigSCell 1 |  | N/A |  |  |  |  |
|  | ConfigSCell 2 |  | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| OCNG Patterns | ConfigSCell 1,2 |  | OP.1 Note 5 |  |  |  |  |
|  | ConfigSCell 3, |  | OP.1 Note 6 |  |  |  |  |
| SSB Configuration | ConfigSCell 1,2 |  | SSB.1 FR2 |  |  |  |  |
|  | ConfigSCell 3 |  | SSB.2 FR2 |  |  |  |  |
| CSI-RS configuration for CSI reporting Note 8 | ConfigSCell 1 |  | CSI-RS.1.1 FDD |  |  |  |  |
|  | ConfigSCell 2 |  | CSI-RS.1.1 TDD |  |  |  |  |
|  | ConfigSCell 3 |  | CSI-RS.2.1 TDD |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |
| reportConfigType |  |  | N/A |  |  |  |  |
| reportQuantity |  |  | N/A |  |  |  |  |
| CSI reporting periodicity | ConfigSCell 1,2 | slot | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| CSI reporting offset | ConfigSCell 1,2 | slot | N/A |  |  |  |  |
|  | ConfigSCell 3 |  | N/A |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS Note 1 |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |
| Note2 | ConfigSCell 1,2 | dBm/SCS | -104 |  |  |  |  |
|  | ConfigSCell 3 |  | -101 |  |  |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |
|  |  | dB | -infinity | 17 | 7 |  |  |
| SS-RSRPNote3 | ConfigSCell 1,2 | dBm/SCS | -infinity | -87 | -97 |  |  |
|  | ConfigSCell 3 |  | -infinity | -84 | -94 |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -infinity | -87 | -97 |  |  |
| Io Note3 | ConfigSCell 1,2 | dBm/ 9.36 MHz | -76.05 | -58.96 | -68.26 |  |  |
|  | ConfigSCell 3 | dBm/ 19.08 MHz | -69.96 | -52.87 | -62.17 |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |
| Correlation Matrix and Antenna Configuration |  |  | 2x2 Low |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |  |  |

##### A.7.5.3.X1.2	Test Requirements

The UE shall complete the SCell activation no later than at slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length.

The UE shall report non-zero CQI for SCell from slot n + THARQ+Tactivation_time+TCSI_ReportingNR slot length and onwards throughout time period T5.

Tactivation_time  is defined in clause 8.3.2A in TS 38.133 for FR2 SCell as Tactivation_time is TFirstSSB_MAX + 15*TSMTC_MAX  + 5ms

The observed SCell activation delay fulfiiling the SCell activation delay requirements specified in clause 8.3.2A in TS 38.133 is counted as correct. The rate of correct observed SCell activation delay during repeated tests shall be at least 90 %.

==============Next change==============

#### A.6.5.3.x	PUCCH SCell Activation and deactivation for UE supporting EMR in FR1

##### A.6.5.3.x.1	Test Purpose and Environment

The purpose of this test is to verify that EMR (early measurement reporting) based SCell activation and deactivation are done within the required time period defined in clause 8.3.2A, when PUCCH for a being activated SCell is configured on the NR FR1 SCell. The PCell and SCell are in different FR1 band. The SCell is unknown by the UE and the UE does not have valid TA for a sTAG which the SCell belongs to at the time of activation. Supported test configurations are shown in table A.6.5.3.X.1-1.

The general test parameters and NR cell specific test parameters are given in Table A.6.5.3.X.1-2 and A.6.5.3.X.1-3 below.

In the test there are two cells: Cell 1 and Cell 2. Cell 1 is PCell, Cell 2 is the PUCCH SCell being activated and deactivated. The test consists of three successive time periods with duration of T1, T2 and T3, respectively. The UE shall be continuously scheduled in Cell 1 (PCell) throughout the test.

Before the test starts, the UE is connected to the PCell (Cell 1) on NR radio channel 1 (PCC), but is not aware of SCell (Cell 2) on NR radio channel 2 (SCC). The PCell is in the pTAGs and the SCell is in a sTAG. The UE is only monitoring the PCC and configured with inter-frequency measurement reporting for Cell 2 in:

measIdleCarrierListNR-r16 for UE supporting measValidationReportEMR-r18

or idleInactiveNR-MeasReport-r16 only,

or measReselectionCarrierListNR-r18 for UE supporting measValidationReportReselectionMeasurements.

During T1 the UE is only configured to the PCell (Cell 1) only. The time when UE receives the early measurement report configuration denote as the starting time of T2. UE will perform the inter-frequency measurement during T2 with the configuration with SCC. The time when UE receive paging from the TE denoted as the starting time of T3. During T3 UE needs to send a valid early measurement report. The time when TE receives the EMR report denote as the end of the T3.

Immediately after the receive of the EMR report, the TE will send an MAC CE message to activate the PUCCH Scell (SCC).The point in time at which the MAC message is received at the UE antenna connector, in slot # denoted n, defines the start of time period T4.

During T4, the test equipment monitors the L1-RSRP measurement reporting for the SCell when test equipment receives a valid L1-RSRP report is denoted as slot m+TL1-RSRP. In the next DL slot after slot m+TL1-RSRP, the test equipment sends a MAC message for the activation of the TCI state of the RMC CORESET of the SCell. In the same slot, the test equipment also sends an RRC message to configure the CSI-RS resources for SCell.

During T4, the test equipment should send a PDCCH order to the UE to initiate RA procedure on the PUCCH SCell at slot (m+THARQ+Tactivation_timeNR slot length) as defined in clause 8.3 after UE report on PCell.

Any PCell interruption due to activation of PUCCH SCell shall occur in the slot n+1+THARQNR slot length to n+1+THARQ+3 ms+TXNR slot length+Ninterruption, as defined in clause 8.3, where Ninterruption is the interruption length given in clause 8.2.

Time period T5 starts when a MAC message for deactivation of PUCCH SCell, sent from the test equipment to the UE in a slot # denoted m, is received at the UE antenna connector.

The deactivation message shall be sent after is sent until a CSI report with other than CQI index 0 is received at the TE.

[The UE shall carry out deactivation of the PUCCH SCell in a slot m+THARQ+3msNR slot length, as defined in clause 8.3, and The starting point of any PCell interruption due to the deactivation shall occur in the slot m+1+THARQNR slot length to m+1+THARQ+3 msNR slot length, as defined in clause 8.3.]

The test equipment verifies that potential interruption is carried out in the correct time span by monitoring ACK/NACK sent in PCell during activation and deactivation of PUCCH SCell, respectively.

The test equipment verifies the activation time by counting the slots from the time when the PUCCH SCell activation command is sent until a CSI report with other than CQI index 0 is received.

Table A.6.5.3.x.1-1: known FR1 SCell activation test configurations

| Config | Description |
| --- | --- |
| 1 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, FDD duplex mode |
| 2 | NR 15 kHz SSB SCS, ≥10 MHz bandwidth, TDD duplex mode |
| 3 | NR 30 kHz SSB SCS, ≥40 MHz bandwidth, TDD duplex mode |
| NOTE 1: The UE is only required to be tested in one of the supported test configurations NOTE 2: The UE is only required to be tested in one with smallest aggregated channel bandwidth from supported band combinations which is composed of CCs ≥ the bandwidth (BWchannel) defined in each test configuration, |  |

Table A.6.5.3.x.1-2: General test parameters for known FR1 SCell activation case

| Parameter | Unit | Value | Comment |
| --- | --- | --- | --- |
| RF Channel Number |  | 1,2 | Two NR radio channel (1, 2) are used for this test |
| Active PCell |  | Cell 1 | Primary cell on NR RF channel number 1. |
| Configured deactivated SCell |  | Cell 2 | Configured deactivated secondary cell on NR RF channel number 2 |
| CP length |  | Normal |  |
| DRX |  | OFF | Continuous monitoring of primary cell |
| Cell-individual offset for cells on NR channel number | dB | 0 | Individual offset for cells on primary component carrier. |
| SCell measurement cycle (measCycleSCell) | ms | 160 |  |
| Cell 2 timing offset to cell1 | s | 0 |  |
| Time alignment error between cell2 and cell1 | s | Time alignment error as specified in TS 38.104 [13] clause 6.5.3.1. | The value of time alignment error depends upon the type of carrier aggregation. |
| TimeAlignmentTimer | ms | 500 | Test 1 and Test 2 |
| T1 | s | 10 | During this time the PCell is connected to the UE |
| T2 | s | 11.25 | During this time the UE shall perform early measurement. |
| T3 | s | 10 | During this time the UE shall send a valid measurement report |
| T4 | s | 2 | During this time the UE shall activate a PUCCH Scell |
| T5 | s | 1 | During this time the UE shall deactivate the PUCCH Scell |
| measIdleValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportEMR-r18 is tested in this test. |
| measReselectionValidityDuration-r18 | s | 5 | Applicable and configured when UE supporting measValidationReportReselectionMeasurements is tested in this test. |
| measIdleDuration-r16 | s | 60 | Value for timer T331 for UE performing idle/inactive measurements while in RRC_IDLE or RRC_INACTIVE. |
| THARQ | ms | Config 1: 2 Config 2: 3 Config 3: 2.5 | k1×NR slot length k1 is a number of slots and is indicated by the PDSCH-to-HARQ-timing-indicator field in the DCI format, if present, or provided by dl-DataToUL-ACK, the value of k should be the minimum value defined in TS 38.213 [3] that will meet the timing constraints of this test case. |
| TCSI_Reporting | ms | 15 | the delay (in ms) including uncertainty in acquiring the first available downlink CSI reference resource, UE processing time for CSI reporting (clause 5.2.2.5 in TS 38.214 [26]) and uncertainty in acquiring the first available CSI reporting resources as specified in TS 38.331 [2] |

Table A.6.5.3.x.1-3: Cell specific test parameters for known FR1 SCell activation case

| Parameter |  | Unit | Cell 1 |  |  | Cell 2 |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | T1 | T2 | T3 | T1 | T2 | T3 |
| Duplex mode | Config 1 |  | FDD |  |  |  |  |  |
|  | Config 2,3 |  | TDD |  |  |  |  |  |
| TDD configuration | Config 1 |  | Not applicable |  |  |  |  |  |
|  | Config 2 |  | TDDConf.1.1 |  |  |  |  |  |
|  | Config 3 |  | TDDConf.2.1 |  |  |  |  |  |
| BWchannel | Config 1,2 | MHz | Note 7 |  |  |  |  |  |
|  | Config 3 |  | Note 7 |  |  |  |  |  |
| BWoccupied | Config 1,2 | RB | 52 Note 5 |  |  |  |  |  |
|  | Config 3 |  | 106 Note 6 |  |  |  |  |  |
| Initial BWP configuration |  |  | DLBWP.0.2 |  |  |  |  |  |
| TCI state |  |  | TCI.State.0 |  |  |  |  |  |
| TRS Configuration | Config 1 |  | TRS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | TRS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | TRS.1.2 TDD |  |  |  |  |  |
| PDSCH Reference measurement channel | Config 1 |  | SR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | SR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | SR.2.1 TDD |  |  | - |  |  |
| Dedicated CORESET parameters | Config 1 |  | CCR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CCR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CCR.2.1 TDD |  |  | - |  |  |
| RMSI CORESET parameters | Config 1 |  | CR.1.1 FDD |  |  | - |  |  |
|  | Config 2 |  | CR.1.1 TDD |  |  | - |  |  |
|  | Config 3 |  | CR.2.1 TDD |  |  | - |  |  |
| OCNG Patterns | Config 1,2 |  | OP.1 Note 5 |  |  |  |  |  |
|  | Config 3, |  | OP.1 Note 6 |  |  |  |  |  |
| SSB Configuration | Config 1,2 |  | SSB.1 FR1 |  |  |  |  |  |
|  | Config 3 |  | SSB.2 FR1 |  |  |  |  |  |
| CSI-RS configuration for CSI reporting (Note 8) | Config 1 |  | CSI-RS.1.1 FDD |  |  |  |  |  |
|  | Config 2 |  | CSI-RS.1.1 TDD |  |  |  |  |  |
|  | Config 3 |  | CSI-RS.2.1 TDD |  |  |  |  |  |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| reportConfigType |  |  | periodic |  |  |  |  |  |
| reportQuantity |  |  | cri-RI-PMI-CQI |  |  |  |  |  |
| CSI reporting periodicity for PCell | Config 1,2 | slot | 5 |  |  | - |  |  |
|  | Config 3 |  | 10 |  |  | - |  |  |
| CSI reporting offset for PCell | Config 1,2 | slot | 3 |  |  | - |  |  |
|  | Config 3 |  | 5 |  |  | - |  |  |
| CSI reporting periodicity for SCell | Config 1,2 | slot | - |  |  | 5 |  |  |
|  | Config 3 |  | - |  |  | 10 |  |  |
| CSI reporting offset for SCell | Config 1,2 | slot | - |  |  | 2 |  |  |
|  | Config 3 |  | - |  |  | 4 |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 |  |  |  |  |  |
| EPRE ratio of PBCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSS(Note 1) |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS (Note 1) |  |  |  |  |  |  |  |  |
| Note2 | Config 1,2 | dBm/SCS | -104 |  |  |  |  |  |
|  | Config 3 |  | -101 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
|  |  | dB | 17 |  |  |  |  |  |
| SS-RSRPNote3 | Config 1,2 | dBm/SCS | -87 |  |  |  |  |  |
|  | Config 3 |  | -84 |  |  |  |  |  |
| SCH_RP Note 3 |  | dBm/15 kHz | -87 |  |  |  |  |  |
| Io Note3 | Config 1,2 | dBm/ 9.36 MHz | -58.96 |  |  |  |  |  |
|  | Config 3 | dBm/ 38.16 MHz | -52.87 |  |  |  |  |  |
| Propagation condition |  | - | AWGN |  |  |  |  |  |
| NOTE 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. NOTE 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled within BWoccupied. NOTE 3: SS-RSRP, Io and SCH_RP levels have been derived from other parameters for information purposes. They are not settable parameters themselves. NOTE 4: The uplink resources for CSI reporting are assigned to the UE prior to the start of time period T2. NOTE 5: All UL/DL transmission shall be confined within BWoccupied (i.e. 10 MHz, 52 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 6: All UL/DL transmission shall be confined within BWoccupied (i.e. 40 MHz, 106 PRBs) from FC,low, and Io is independent of the BWchannel configured. NOTE 7: NPRB,c. is derived from Table 5.3.2-1 in TS38.101-1[18] with configured BWchannel. NOTE 8: On top of the reference configurations, CSI-RS offset should be set to meet the CSI reference resource timing definition in clause 5.2.2.5 in TS 38.214 [26]. |  |  |  |  |  |  |  |  |

##### A.6.5.3.x.2	Test Requirements

The UE behaviour during time durations t2 and t3 shall be as follows:

During the time period t2 the UE is in Idle mode and the signal level of Cell 2 is changed. The UE shall not perform reselection. The UE shall perform Idle Mode CA measurement according to section 4.4.

At the start of t3 the UE is paged for connection setup. During the connection setup the UE is requested to transmit early measurement report for Cell 2. The UE shall send early measurement report to the PCell.

After receiving the requested early measurement report, the test equipment verifies the accuracy of measurement reported for Cell 2 meets the requirements in section 10.1.4B for SS-RSRP and in section 10.1.8B for SS-RSRQ and test ends.

The rate of correct events observed during repeated tests shall be at least 90%.

All of the above test requirements shall be fulfilled in order for the observed PUCCH SCell activation delay and PUCCH SCell deactivation delay to be counted as correct. The rate of correct observed PUCCH SCell activation delay and PUCCH SCell deactivation delay during repeated tests shall be at least 90 %.

During T4, the UE shall start sending valid L1-RSRP report for the SCell in the configured slots for CSI reporting no later than at slot n+THARQ+Tactivation_time+TCSI_ReportingNR slot length as defined in clause 8.3.2A.

During T4, the UE shall start sending CSI reports for the SCell with non-zero CQI index in the configured slots for CSI reporting no later than slot n + THARQ+Tactivation_time+TCSI_ReportingNR slot length, where

-		THARQ is defined in Table A.7.5.3.X.1-2

-		Tactivation_time  is defined in clause 8.3.2A

During T5, the UE shall stop sending CSI reports for SCell no later than slot n+THARQ+3 msNR slot length, as defined in clause 8.3.2A.

[During T5, the starting point of interruption of PCell during SCell deactivation shall not happen outside the slot n+1+THARQNR slot length to n+1+THARQ+3 msNR slot length as defined in clause 8.3.2A.]

==============End of change==============