3GPP TSG-RAN WG4 Meeting #117	R4-2522940

Dallas, USA, Nov. 17-21, 2025

| CR-Form-v12.3 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.101-5 | CR | draftCR | rev | - | Current version: | 19.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for 38.101-5, On UE demodulation requirements for 5G broadcast over GSO |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Incorporated |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_band_5G_bcast_GSO-Perf |  |  |  |  |  | Date: |  |  | 2025-11-04 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Intrduction of paramteres and reqirements for UE demodulaton for LTE based 5G broadcast over GSO. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Add the parameters and correspoding UE demodulation requirement for LTE based broadcast over GSO |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | UE demodulation requirements for LTE based 5G broadcast over GSO is missing in the specification. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 8.4, A.1.3, D.1.1, D.1.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

## <Start of Change 1>

## 8.4	Demodulation performance requirements for BOG

### 8.4.1	Minimum requirements for PMCH

#### 8.4.1.1      Minimum requirements with 15kHz subcarrier spacing

The receive characteristic of MBMS is determined by the BLER.

For the parameters specified in Table 8.4.1.1-1, the average downlink SNR shall be below the specified value for the BLER shown in Table 8.4.1.1-2.

Table 8.4.1.1-1: Test Parameters

| Parameter |  |  | Unit |  | Value |  |
| --- | --- | --- | --- | --- | --- | --- |
| Number of HARQ processes |  |  | Processes |  | None |  |
| Subcarrier spacing |  |  | kHz |  | 15 kHz |  |
| Allocated subframes per Radio Frame |  |  |  |  | 10 subframes |  |
| Number of OFDM symbols for PDCCH |  |  |  |  | 2 |  |
| Cyclic Prefix |  |  |  |  | Extended |  |
| Downlink power allocation |  |  | dB |  | 0 |  |
|  |  |  | dB |  | 0 (Note 1) |  |
|  |  |  | dB |  | 0 |  |
| at antenna port |  |  | dBm/15kHz |  | -98 |  |
| Note 1: . |  |  |  |  |  |  |

Table 8.4.1.1-2: Minimum performance

| Test number | Cell | Bandwidth (MHz) | Reference Channel | OCNG Pattern | Propagation condition | Correlation Matrix and antenna | Reference value |  | MBMS UE Category |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | BLER (%) | SNR(dB) |  |
| 1 | MBMS Dedicated Cell | 10 | R.PMCH.1 FDD | NA | NTN-TDLC5-5 | 1x2 low | 1 | [3.7] | ≥1 |
| 2 | MBMS Dedicated Cell | 10 | R.PMCH.2 FDD | NA | NTN-TDLC5-5 | 1x2 low | 1 | [10.6] | ≥1 |

#### <End of Change 1>

## <Start of Change 2>

### A.1.3	Reference measurement channels for PMCH performance requirements

#### A.1.3.1	SDO

A.1.3.1-1 Fixed Reference Channel for PMCH Receiver Requirements (15 kHz SCS)

| Parameter | PMCH (15 kHz SCS) |  |  |
| --- | --- | --- | --- |
|  | Unit | Value |  |
| Reference channel |  | R.PMCH.1 | R.PMCH.2 |
| PMCH bandwidth | MHz | 10 | 10 |
| Allocated resource blocks |  | 50 | 50 |
| Allocated subframes per Radio Frame(Note1) |  | 10 | 10 |
| Modulation |  | QPSK | 16QAM |
| Target Coding Rate |  | 1/3 | 1/2 |
| Information Bit Payload (Note 2) |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 3624 | 9912 |
| Number of Code Blocks per Sub-Frame (Note 3) |  | 1 | 2 |
| Binary Channel Bits Per Subframe |  |  |  |
| For Sub-Frames 0,1,2,3,4,5,6,7,8,9 | Bits | 10200 | 20400 |
| MBMS UE Category |  | ≥ 1 | ≥ 1 |
| Note 1: For FDD mode, all 10 subframes are available for MBMS, in line with TS 36.331 [6]. Note 2: 2 OFDM symbols are reserved for PDCCH; and no CRS allocated as per TS 36.211 [3]. Note 3: If more than one Code Block is present, an additional CRC sequence of L = 24 Bits is attached to each Code Block (otherwise L = 0 Bit). |  |  |  |

#### <End of Change 2>

## <Start of Change 3>

## D.1.1	Delay profiles

The delay profiles are derived from the TR 38.811 [13] NTN-TDL models for the desired delay spread and tap resolution. After scaling the normalized delay spread values for each tap by the desired RMS delay spread, the tap delays are quantized to a delay resolution of 5ns by rounding to the nearest multiple of the delay resolution.

Table D.1.1-1: Delay profiles for IoT NTN and BOG channel models

| Type | Model | Delay spread (r.m.s.) | Delay resolution |
| --- | --- | --- | --- |
| NLOS | NTN-TDLA100 | 100 ns | 5 ns |
| LOS | NTN-TDLC5 | 5 ns | 5 ns |

Table D.1.1-2: NTN-TDLA100 (DS = 100 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | 0 | Rayleigh |
| 2 | 110 | -4.7 | Rayleigh |
| 3 | 285 | -6.5 | Rayleigh |

Table D.1.1-3 NTN-TDLC5 (DS = 5 ns)

| Tap # | Delay [ns] | Power [dB] | Fading distribution |
| --- | --- | --- | --- |
| 1 | 0 | -0.6 | LOS path |
|  | 0 | -8.9 | Rayleigh |
| 2 | 60 | -21.5 | Rayleigh |
| Note 1: Tap #1 follows a Rician distribution. |  |  |  |

## D.1.2	Combinations of channel model parameters

The propagation conditions used for the performance measurements in multi-path fading environment are indicated as a combination of a channel model name and a maximum Doppler frequency, i.e., NTN-TDLA<DS>-<Doppler>, or NTN-TDLC<DS>-<Doppler> where '<DS>' indicates the desired delay spread and '<Doppler>' indicates the maximum Doppler frequency (Hz).

Table D.1.2-1 show the propagation conditions that are used for the performance measurements in multi-path fading environment for NLOS and LOS propagation conditions.

Table D.1.2-1: Channel model parameters for NTN

| Combination name | Model | Maximum Doppler frequency |
| --- | --- | --- |
| NTN-TDLA100-10 | NTN-TDLA100 | 10 Hz |
| NTN-TDLA100-200 | NTN-TDLA100 | 200 Hz |
| NTN-TDLC5-30 | NTN-TDLC5 | 30 Hz |
| NTN-TDLC5-200 | NTN-TDLC5 | 200 Hz |
| NTN-TDLC5-5 | NTN-TDLC5 | 5 Hz |

## <End of Change 3>