3GPP TSG-RAN4 Meeting #117	R4-2522945

# Dallas, USA,  17th – 21st November 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.141-2 | CR | draftCR | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | DraftCR to TS 38.141-2 on test requirements for SBFD-capable BS |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Nokia |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_duplex_evo-Perf |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | B |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | Introduction of test requirements for SBFD-capable BS to TS 38.141-2. |  |  |  |  |  |  |  |  |
| Summary of change: |  | 9.2.5 OTA transmitted signal quality for SBFD 9.2.6 OTA unwanted emissions for SBFD 9.2.7 OTA transmitter intermodulation for SBFD 9.2.8 OTA transient period between SBFD and non-SBFD operation 9.2.9 OTA In-channel adjacent subband leakage for SBFD |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | SBFD-capable BS will not have test requirements for Rel-19. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | New clauses introduced to specification: 9.2.5, 9.2.6, 9.2.7, 9.2.8, 9.2.9 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  |  |  |  |  |  |  |  |  |  |

==============First change==============

### 9.2.5	OTA transmitted signal quality for SBFD

9.2.5.1		General

The general description for OTA transmitted signal quality as clause 6.6.1 shall be applied.

9.2.5.2		OTA frequency error for SBFD

The minimum requirement for BS type 1-O and 2-O is specified in TS 38.104 [2], clause 12.5.6.2, and is defined as a directional requirement at the RIB and shall be met within the OTA coverage range. The test purpose defined in clause 6.6.3.3 shall apply. The method of test defined in clause 6.6.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.3.5 shall apply.

9.2.5.3		OTA modulation quality for SBFD

The minimum requirement for BS type 1-O and 2-O is specified in TS 38.104 [2], clause 12.5.6.2, and is defined as a directional requirement at the RIB and shall be met within the OTA coverage range. The test purpose defined in clause 6.5.3.3 shall apply. The method of test defined in clause 6.5.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.5.3.5 shall apply.

9.2.5.4		OTA time alignment error for SBFD

The minimum requirement for BS type 1-H is specified in TS 38.104 [2], clause 12.5.6.2, and is defined as a directional requirement at the RIB and shall be met within the OTA coverage range. The test purpose defined in clause 6.6.4.3 shall apply. The method of test defined in clause 6.6.4.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.6.4.5 shall apply.

==============Next change==============

### 9.2.6 	OTA unwanted emissions for SBFD

9.2.6.1		General

The general description for unwanted emissions as clause 6.7.1 shall be applied.

9.2.6.2		OTA occupied bandwidth

The minimum requirement for BS type 1-O and 2-0 is specified in TS 38.104 [2], clause 12.5.7.2, and is defined as a directional requirement and shall be met in the manufacturer's declared OTA coverage range at the RIB. The test purpose defined in clause 6.7.2.3 shall apply. The method of test defined in clause 6.7.2.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.7.2.5 shall apply.

9.2.6.3		OTA Adjacent Channel Leakage Power Ratio (ACLR) for SBFD

The minimum requirement for BS type 1-O and 2-O is specified in TS 38.104 [2], clause 12.5.7.2, and is defined as TRP. The test purpose defined in clause 6.7.3.3 shall apply. The method of test defined in clause 6.7.3.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.7.3.5 shall apply.

9.2.6.4		OTA operating band unwanted emissions for SBFD

The minimum requirement for BS type 1-O and 2-O is specified in TS 38.104 [2], clause 12.5.7.2, and apply as TRP per RIB, unless otherwise stated. The test purpose defined in clause 6.7.4.3 shall apply. The method of test defined in clause 6.7.4.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.7.4.5 shall apply.

9.2.6.5		OTA transmitter spurious emissions for SBFD

The minimum requirement for BS type 1-O and 2-O is specified in TS 38.104 [2], clause 12.5.7.2, and apply as TRP per RIB, unless otherwise stated. The test purpose defined in clauses 6.7.5.2.3, 6.7.5.3.3 and 6.7.5.4.3 shall apply. The method of test defined in clauses 6.7.5.2.4, 6.7.5.3.4 and 6.7.5.4.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clauses 6.7.5.2.5, 6.7.5.3.5 and 6.7.5.4.5 shall apply.

==============Next change==============

### 9.2.7	OTA transmitter intermodulation for SBFD

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], clause 12.5.8.1, and applies at each RIB supporting transmission in the operating band. The test purpose defined in clause 6.8.3 shall apply. The method of test defined in clause 6.8.4 shall apply, except that the test signals, test configurations, RF channels, and test models shall follow the SBFD-specific test conditions defined in clause 4.9.x. The test requirements defined in clause 6.8.5 shall apply.

==============Next change==============

### 9.2.8	Transient period between SBFD and non-SBFD operation

### 9.2.8.1	Definition and applicability

Transmitter transient period between SBFD and non-SBFD operation requirements apply only to SBFD-capable BS.

The transmitter transient period between SBFD and non-SBFD operation is the time period during which the transmitter is changing from the transmitter OFF period to the transmitter ON period or vice versa. The transmitter transient period between SBFD and non-SBFD operation is illustrated in figure 9.2.8.1-1.

Transmitter output power frequency regions for transition

Time

Transmitter ON period

(DL transmission)

Transmitter OFF

period

Transmitter OFF

period

Transient period between

SBFD and non-SBFD

OFF power level

ON power level

UL transmission

GP or UL transmission

Figure 9.2.8.1-1: Example of relations between transmitter ON period, transmitter OFF period and transmitter transient period between SBFD and non-SBFD operation

The transmitter transient period for SBFD shall be measured in the transmission bandwidth configuration for SBFD DL subband, UL subband and guardband, depending on different SBFD cases:

Table 9.2.8.1-1: Transmitter transient period between SBFD and non-SBFD operation

|  | Frequency regions for transition | BS transmitter behavior during transition |
| --- | --- | --- |
| SBFD symbol to DL symbol | SBFD UL subband and guardband(s) | BS transmitter OFF-to-ON |
| SBFD symbol to UL symbol | SBFD DL subband | BS transmitter ON-to-OFF |
| DL symbol to SBFD symbol | SBFD UL subband and guardband(s) | BS transmitter ON-to-OFF |
| UL symbol to SBFD symbol | SBFD DL subband | BS transmitter OFF-to-ON |

For BS type 1-O and 2-O this requirement shall be applied at each RIB supporting transmission in the operating band.

9.2.8.2	Minimum requirement

The minimum requirement for BS type 1-O is specified in TS 38.104 [2], clause 12.5.10.2.

The minimum requirement for BS type 2-O is specified in TS 38.104 [2], clause 12.5.10.3

9.2.8.3	Test purpose

The purpose of this test is to verify the OTA transmitter transient periods between SBFD and non-SBFD operation are within the limits of the minimum requirements.

9.2.8.4	Method of test

##### 9.2.8.4.1	Initial conditions

Test environment:

-	normal; see annex B.2.

RF channels to be tested for single carrier: see clause 4.9.1.

RF bandwidth positions to be tested for multi-carrier: see clause 4.9.1.

Directions to be tested:

-	The requirement for BS type 1-O is specified as co-location requirement. For general description of co-location requirements, refer to clause 4.12.

-	The requirement for BS type 2-O is verified by an EIRP measurement at a direction corresponding to the OTA peak directions set reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

##### 9.2.8.4.2	Procedure

9.2.8.4.2.1	General procedure

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

9.2.8.4.2.2	BS type 1-O

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Place the co-location test antenna as specified in clause 4.12.

5)	Configure the beam peak direction of the BS according to the declared beam direction pair.

6)	Set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test models or set of physical channels in clause 4.9.2.

For a BS declared to be capable of multi-carrier, use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test models or set of physical channels in clause 4.9.2 on all carriers configured.

7)	Measure the mean power spectral density at the output(s) of co-location test antenna as power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 10 μs to 35/N μs before start of next transmitter ON period - 10 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

9.2.8.4.2.3	BS type 2-O

3)	Set the BS in the direction of the declared beam peak direction of the beam direction pair, for the beam to be tested.

4)	Set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model NR-FR2-TM1.1_SBFD_DU/UD/DUD and set of physical channels in clause 4.9.2.

For a BS declared to be capable of multi-carrier, use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model NR-FR2-TM1.1_SBFD_DU/UD/DUD and set of physical channels in clause 4.9.2 on all carriers configured.

5)	Measure the mean EIRP spectral density as the power sum over two orthogonal polarizations over 70/N μs filtered with a square filter of bandwidth equal to the RF bandwidth of the NR BS centred on the central frequency of the RF bandwidth. 70/N μs average window centre is set from 35/N μs after end of one transmitter ON period + 3 μs to 35/N μs before start of next transmitter ON period - 3 μs. N = SCS/15, where SCS is Sub Carrier Spacing in kHz.

NOTE:	Make sure that the measurement receiver is not overloaded.

9.2.8.5	Test requirements

##### 9.2.8.5.1	SBFD-capable BS type 1-O

The mean power spectral density measured according to clause 6.5.2.4.2 shall be less than -102.6 dBm/MHz for carrier frequency f ≤ 3.0 GHz.

The mean power spectral density measured according to clause 6.5.2.4.2 shall be less than -102.4 dBm/MHz for carrier frequency 3.0 GHz < f ≤ 7.125 GHz.

##### 9.2.8.5.2	SBFD-capable BS type 2-O

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -33.1 + PSBFD,rated,c,EIRP - PSBFD,rated,c,TRP dBm/MHz for carrier frequency 24.15 GHz < f ≤ 29.5 GHz, where PSBFD,rated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -32.7 + PSBFD,rated,c,EIRP – PSBFD,rated,c,TRP dBm/MHz for carrier frequency 37 GHz < f ≤ 43.5 GHz, where PSBFD,rated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -32.4 + Prated,c,EIRP - Prated,c,TRP dBm/MHz for carrier frequency 43.5 GHz < f ≤ 48.2 GHz, where Prated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

The measured mean EIRP spectral density according to clause 6.5.2.4.2 shall be less than -30.4 + PSBFD,rated,c,EIRP – PSBFD,rated,c,TRP dBm/MHz for carrier frequency 52.6 GHz ≤ f ≤ 71 GHz, where PSBFD,rated,c,EIRP is the value declared for the reference beam direction pair (D.8) for the beam identifier (D.3) which provides the highest intended EIRP.

==============Next change==============

### 9.2.9	In-channel adjacent subband leakage for SBFD

9.2.9.1	Definition and applicability

In-channel adjacent subband leakage (ICASL) is the filtered mean power centred on the declared frequency region of SBFD UL subband within NR carrier.

The requirement shall be applied per RIB during the transmitter ON period.

9.2.9.2	Minimum requirement

The minimum requirement for SBFD-capable BS type 1-O is specified in TS 38.104 [2], clause 12.5.11.2.

The minimum requirement for SBFD capable BS type 2-O is specified in TS 38.104 [2], clause 12.5.11.3.

9.2.9.3	Test purpose

To verify that the OTA in-channel adjacent subband leakage requirement shall be met as specified by the minimum requirement.

9.2.9.4	Method of test

##### 9.2.9.4.1	Initial conditions

Test environment: Normal; see annex B.2.

RF channels to be tested for single carrier: see clause 4.9.1.

Base Station RF Bandwidth positions to be tested for multi-carrier: see clause 4.9.1.

##### 9.2.9.4.2	Procedure

The following procedure for measuring TRP is based on the directional power measurements as described in annexI. An alternative method to measure TRP is to use a characterized and calibrated reverberation chamber. If so, follow steps 1, 3, 4, 6, 8, 9, 10, 11, 12 and 13. When calibrated and operated within the guidance of 3GPP TR 37.941 [29] the measurement methods are applicable and selected depending on availability at the test facility.

1)	Place the BS at the positioner.

2)	Align the manufacturer declared coordinate system orientation (D.2) of the BS with the test system.

3)	The measurement devices characteristics shall be:

measurement filter bandwidth: defined in clause 6.7.3.5.

detection mode: true RMS voltage or true power averaging.

The emission power should be averaged over an appropriate time duration to ensure the measurement is within the measurement uncertainty in Table 4.1.2.2-1 for FR1 and Table 4.1.2.2-2 for FR2.

4)	For single carrier operation, set the BS to transmit according to the applicable test configuration in clause 4.8 using the corresponding test model(s) in clause 4.9.2 at manufacturers declared rated carrier output power (PSBFD,rated,c,TRP).

For a BS declared to be capable of multi-carrier, use the applicable test signal configuration and corresponding power setting specified in clauses 4.7.2 and 4.8 using the corresponding test model(s) in clause 4.9.2 on all carriers configured.

5)	Orient the positioner (and BS) in order that the direction to be tested aligns with the test antenna such that measurements to determine TRP can be performed (see annex I).

6)	Measure the absolute power of the assigned channel frequency and the (adjacent channel frequency).

7)	Repeat step 5-6 for all directions in the appropriated TRP measurement grid needed for TRPEstimate (see annex I).

8)	Calculate TRPEstimate for the absolute total radiated power of the wanted channel and the adjacent channel using the measurements made in Step 7.

9)	Measure OTA ICASL for the frequency offsets both side of channel frequency as specified in table 6.7.3.5.1-1 for BS type 1-O or table 6.7.3.5.2-1 for BS type 2-O respectively. In multiple carrier case only offset frequencies below the lowest and above the highest carrier frequency used shall be measured.

10) Repeat the test with the channel set-up using NR- FR1-TM1.2_SBFD_DU/UD/DUD defined in clause 4.9.2 in TS 38.141-1 [3] for BS type 1-O.

#### 9.2.9.4.3	Test requirements

##### 9.2.9.4.3.1	BS type 1-O

The ICASL shall be less than or equal to a basic limit defined in table 9.2.9.4.3.1-1 + X (where X = 9 dB).

Table 9.2.9.4.3.1-1: SBFD-capable BS type 1-O ICASL absolute basic limit

| SBFD-capable BS category / BS class | OTA ICASL absolute limit |
| --- | --- |
| Category A Wide Area BS | -13 dBm/MHz |
| Category B Wide Area BS | -15 dBm/MHz |
| Medium Range BS | -25 dBm/MHz |
| Local Area BS | -32 dBm/MHz |

Table 9.2.9.4.3.1-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

##### 9.2.9.4.3.2	BS type 2-O

The ICASL shall be less than or equal to the value specified in table 9.2.9.4.3.2-1.

Table 9.2.9.4.3.2-1: SBFD-capable BS type 2-O ICASL absolute basic limit

| SBFD-capable BS category / BS class | OTA ICASL absolute basic limit |
| --- | --- |
| Wide Area BS | -13 dBm/MHz |
| Medium Range BS | -20 dBm/MHz |
| Local Area BS | -20 dBm/MHz |

Table 9.2.9.4.3.1-2: Filter parameters for the UL subband

| RAT | Filter on the assigned UL subband and corresponding filter bandwidth |
| --- | --- |
| NR | Square (the declared bandwidth of UL subband) |

==============End of change==============