3GPP TSG-RAN WG4 Meeting #117	R4-2522948

Dallas, USA, 17th – 21st November 2025

Title: 	Way Forward for SBFD BS conformance

Agenda item:	6.13

Source:	Ericsson

Document for:	Approval

# Introduction

The agreements in this WF follow the ad-hoc discussions and online discussion. It also states the updated work-split as reminder for the companies to provide draft CRs in RAN4#118.

# Way Forward

## Existing Tx-Rx requirements

Issue 1-1-1: General

Agreement:

- Conformance testing shall be performed separately for both SBFD and non-SBFD slots.

Issue 1-1-2: Manufacture declaration related to SBFD capable BSs

Agreement:

- Add the below declarations -

| D.129 | Subband full duplex | Declaration of subband full duplex support, i.e., SBFD support or no SBFD support |  | x |
| --- | --- | --- | --- | --- |
| D.130 | Operating bands and frequency ranges for SBFD operation of SBFD-capable BS | List of NR operating band(s) supported by single-band connector(s) of the BS and if applicable, frequency range(s) within the operating band(s) that the BS can operate in for SBFD operation. Declarations shall be made per TAB connector for BS type 1-H. |  | x |
| D.131 | NR supported channel bandwidth and subband configurations for SBFD operation of SBFD-capable BS | List of supported channel bandwidths for SBFD operation. List of supported subband configurations per supported channel bandwidth for SBFD operation, which includes: - The SBFD pattern (e.g., DUD, DU/UD). - The transmission bandwidth of the uplink subband (UL). - The transmission bandwidth of the downlink subbands (DL). - The bandwidth of the guard bands between subbands. - All RF channels that implement analogue - filters. Declare how these configurations correspond to the supported channel bandwidth(s). Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.132 | Single SBFD carrier operation or multi-carrier operation for SBFD-capable BS | Capable of operating with a single SBFD carrier (only), or multiple carriers, i.e., an SBFD carrier is operated in conjunction with one or more TDD carriers. For multi-carrier operation, declare the association between the SBFD carrier and each TDD carrier, providing configuration details and operational conditions. Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.133 | Maximum number of supported carriers per operating band in single band SBFD operation of SBFD-capable BS | For multi-carrier operation, maximum number of supported carriers per supported operation band in single band operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 2) |  | x |
| D.134 | Rated SBFD carrier output power (PSBFD,rated,c,TABC) for SBFD operation of SBFD-capable BS | For single carrier operation, conducted rated SBFD carrier output power per single band connector for SBFD operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 1, 2, 7) |  | x |
| D.135 | Rated carrier output power for SBFD operation of SBFD-capable BS | For multi-carrier operation: - Conducted rated carrier output power (PSBFD,rated,c,TABC) of SBFD carrier per single band connector for SBFD operation. - Conducted rated carrier output power (Prated,c,TABC) of each TDD carrier per single band connector. |  | x |
| D.136 | Rated total output power (PSBFD,rated,t,TABC) for SBFD operation of SBFD-capable BS | For multi-carrier operation, conducted total rated output power for SBFD operation. Declared per supported operating band, per TAB connector for BS type 1-H. (Note 1, 2, 7) |  | x |
| D.137 | Maximum supported power difference between carriers for SBFD operation of SBFD-capable BS | Maximum supported power difference between carriers including SBFD carrier and TDD carrier(s). Declared per supported operating band, per TAB connector for BS type 1-H. |  | x |
| D.138 | Relation between supported maximum RF bandwidth, number of carriers and rated total output power for SBFD operation of SBFD-capable BS | If the rated total output power and total number of supported carriers are not simultaneously supported, the manufacturer shall declare the following additional parameters: - The reduced number of supported carriers at the rated total output power; - The reduced total output power at the maximum number of supported carriers. |  | x |
| NOTE 1: If a BS is capable of 1024QAM DL operation then up to three rated output power declarations may be made. One declaration is applicable when configured for 1024QAM transmissions, a different declaration is applicable when configured 256QAM transmissions and the other declaration is applicable when configured neither for 256QAM nor 1024QAM transmissions. NOTE 2: Parameters for contiguous or non-contiguous spectrum operation in the operating band are assumed to be the same unless they are separately declared. When separately declared, they shall still use the same declaration identifier. NOTE 3: If BS is declared to support Band n20 (D.3), the manufacturer shall declare if the BS may operate in geographical areas allocated to broadcasting (DTT). Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 4: This manufacturer declaration is optional. NOTE 5: This manufacturer may declare two values, one with a minimum of +6dB and the other with a minimum of +3dB. NOTE 6: If BS is declared to support Band n24 (D.3), the manufacturer shall declare if the BS may operate in geographical areas where FCC regulations apply. Additionally, related declarations of the emission levels and maximum output power shall be declared. NOTE 7: If a BS is capable of 256QAM DL operation but not capable of 1024QAM DL operation then up to two rated output power declarations may be made. One declaration is applicable when configured for 256QAM transmissions, and the other declaration is applicable when not configured for 256QAM transmissions NOTE 8: If a BS is declared capable of SBFD operation: For SBFD operation, the manufacturer shall declare parameters D.129 – D.138; For non-SBFD operation, the manufacturer shall declare parameters D.1 – D.128 which applicable. |  |  |  |  |

Issue 1-1-3: RF channels to be tested for configuration-flexible SBFD capable BS

Agreement:

- For configuration-flexible SBFD capable BS,

  - For conformance testing of SBFD slots, conformance tests at the RF channels B (bottom), M (middle) and T (top) of the operating band are needed.

Issue 1-1-4: Test signal configuration for FR1 and FR2-1

Agreement:

  - For the test signal SCS configuration, confirm to use 30 kHz for FR1 and 120 kHz for FR2-1.

  - Adopt the below test signal configuration:

| SBFD-capable BS type |  | Configuration-limited |
| --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |
|  | Subcarrier spacing | 30 kHz for FR1 120 kHz for FR2-1 |
|  | Subband configuration | All declared subband configurations |

| SBFD-capable BS type |  |  | Configuration-flexible |  |
| --- | --- | --- | --- | --- |
| TC signal characteristics | BWchannel | All declared channel bandwidths |  |  |
|  | Subcarrier spacing | 30 kHz for FR1 120 kHz for FR2-1 |  |  |
|  | Subband configuration | DUD |  | DU/UD |
|  |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL,1/2. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL,1/2. |  | - The configuration with the narrowest UL subband size, and the largest NRB,SBFD,DL. - The configuration with the widest UL subband size, and the largest NRB,SBFD,DL. |

Issue 1-1-5: Test Model including common physical channel parameters and NR-FR1-TM1.1 for existing requirements

Agreement on using the following modified version of NR-FR1-TM1.1 as baseline for PAPR simulation:

- DU case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |

- DUD case

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL,1 - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 |
| # of PRBs PDSCH nRNTI=3 | NRB, SBFD, DL,2 |
| Modulation PDSCH nRNTI=3 | QPSK |
| Starting RB location of PDSCH nRNTI=3 | NRB - NRB, SBFD, DL, 2 |

- UD case:

  - The difference is on the PDCCH location. Companies are expected to provide PAPR simulations to further down-select from the below two options.

    - Option 1:

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB,SBFD,DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | 3 + NRB,SBFD,UL + NRB,SBFD,GB |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | 0 + NRB,SBFD,UL + NRB,SBFD,GB |

    - Option 2:

| Parameter | Value |
| --- | --- |
| # of PRBs PDSCH nRNTI=0 | NRB, SBFD, DL - 3 |
| Modulation PDSCH nRNTI=0 | QPSK |
| Starting RB location of PDSCH nRNTI=0 | NRB - NRB, SBFD, DL |
| Modulation of PDSCH nRNTI=2 | QPSK |
| Starting RB location of PDSCH nRNTI=2 | NRB - 3 |

On common physical channel parameters:

For DU:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For UD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL – 3 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

For DUD:

Table 4.9.2.2-2: Common physical channel parameters for PDCCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| # of symbols used for control channel | 2 |
| Starting symbol number for control channel | 0 |
| # of CCEs allocated to PDCCH | 1 |
| Starting RB location for PDCCH | 0 |
| # of available REGs | 6 |
| Aggregation level | 1 |
| # of RBs not allocated for PDCCH in each symbol | NRB,SBFD,DL,1 – 3 + NRB,SBFD,DL,2 |
| Ratio of PDCCH EPRE to DM-RS EPRE | 0 dB |
| Boosting level of control channel | 0 dB |

Table 4.9.2.2-3: Common physical channel parameters for PDSCH for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| Mapping type | PDSCH mapping type A |
| dmrs-TypeA-Position for the first DM-RS symbol | pos2 |
| dmrs-AdditionalPosition for additional DM-RS symbol(s) | 1 |
| dmrs-Type for comb pattern | Configuration type 1 |
| maxLength | 1 |
| Ratio of PDSCH EPRE to DM-RS EPRE | 0 dB |

For DU:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |

For UD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 + NRB,SBFD,UL + NRB,SBFD,GB |
| Number of PRBs | 3 |

For DUD:

Table 4.9.2.2-4: Common physical channel parameters for PDSCH by RNTI for BS type 1-H test models

| Parameter | Value |
| --- | --- |
| PDSCH nRNTI=0 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=1 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| PDSCH nRNTI=2 |  |
| Starting symbol | 2 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |
| Starting PRB location | 0 |
| Number of PRBs | 3 |
| PDSCH nRNTI=3 |  |
| Starting symbol | 0 |
| Ratio of PDSCH EPRE to PDCCH EPRE | 0 dB |

Issue 1-1-6: Test procedure

Agreement:

- Clause 4.9 to indicate the differences in test procedure related to SBFD.

- Re-use existing test procedures as much as possible.

Issue 1-1-7: Test configuration

Agreement:

- NRTC1 test configuration only

  - For single-carrier and multi-carrier SBFD operation, a new test configuration for contiguous spectrum operation shall be created including one SBFD carrier and the associated non-SBFD carriers

Issue 1-1-8: Measurement uncertainty (MU) and test tolerance (TT) for both Tx and Rx for existing requirements

Agreement:

- Re-use existing MU for SBFD symbols testing

- Re-use existing TT for SBFD symbols testing

Issue 1-1-9: Receiver status during conformance testing of transmitter

Agreement:

- No need to mention any requirement on RX when conformance testing TX.

## New requirements

Issue 1-2-2: Transient period between SBFD and Non-SBFD operation

Agreement:

- For testing configuration, testing procedure, MU and TT, re-use from Transmitter transient period performance requirement

  - Add SBFD modifications to test configuration such as time domain configuration.

- For test model,

  - For switching from SBFD to DL or from DL to SBFD, use two test models: the existing TM1.1 to represent non-SBFD DL operation, and a modified version of TM1.1 to represent SBFD operation.

  - For switching from SBFD to UL or from UL to SBFD, use one test model, i.e., the modified version of TM1.1.

  - Note – modified TM1.1 is subject to agreements on Issue 1-1-5

Issue 1-2-3: In-channel adjacent subband leakage conformance testing

Agreement:

- For test procedure, MU and TT, re-use the existing onesfor absolute ACLR limit as baseline and make the necessary modification for it.

- For test model,

  - Use the modified version of TM1.1

    - Note – modified TM1.1 is subject to agreements on Issue 1-1-5

  - FFS on TM1.2

Issue 1-2-4: In-channel adjacent subband blocking conformance testing

Agreement:

- For test procedure, MU and TT, re-use the existing ones for in-band blocking requirement as baseline and make the necessary modification for it.

- For test model,

  - Use the modified version of TM1.1

    - Note – modified TM1.1 is subject to agreements on Issue 1-1-5

# Work-split for CRs drafting

The current work-split is for both existing and new requirements.

| Content for both 38.141-1 and 38.141-2 | Current/ New clauses | Responsible company |
| --- | --- | --- |
| References Definitions, symbols and abbreviations Operating bands and channel arrangement | 2, 3, 5 | CATT |
| General conducted and radiated test conditions and declarations | 4 | Ericsson |
| Conducted and Radiated Transmitter characteristics - General - Base station output power - Output power dynamics - Transmit ON/ OFF power | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Samsung |
| Conducted and Radiated Transmitter characteristics - Transmitted signal quality - Unwanted emissions - Transmitter intermodulation - Transient period - In-channel adjacent subband leakage | Old clauses: 6.5 to 6.7 New potential clauses to align how it was done with Core specs: 9.5 to 9.9 | Nokia |
| Conducted and Radiated Receiver characteristics - General - Reference sensitivity level - Dynamic range - In-band selectivity and blocking | Old clauses: 6.1 to 6.4 New potential clauses to align how it was done with Core specs: 9.1 to 9.4 | Huawei |
| Conducted and Radiated Receiver characteristics - Out of band blocking - Receiver spurious emissions - Receiver intermodulation - In-channel selectivity - In-channel adjacent subband blocking | Old clauses: 6.5 to 6.8 New potential clauses to align how it was done with Core specs: 9.5 to 9.9 | ZTE |
| Big CR 38.141-1 and 38.141-2 |  | Moderator (Ericsson) |