3GPP TSG-RAN WG4 Meeting #117	R4-2523092

# Dallas, US, 17th – 21st Nov, 2025

| CR-Form-v12.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR | 6032 | rev | 1 | Current version: | 19.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at https://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | (NR_NTN_Ph3-Core) CR on RRM core maintenance for NTN Phase 3 |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | CATT, ZTE corporation, Sanechips |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_NTN_Ph3-Core |  |  |  |  |  | Date: |  |  | 2025-11-07 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-19 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) … Rel-17 (Release 17) Rel-18 (Release 18) Rel-19 (Release 19) Rel-20 (Release 20) |  |
| Reason for change: |  | - HD-FDD related impact was missed for Random access for RedCap UE with satellite access in clause 6.2E.2. - The format needs to be adjusted in clauses 7.1E.2.1 and 7.2E.2. - HD-FDD related impact was missed for CSI-RS based candidate beam detection for RedCap UE with satellite access in clause 8.5E.6. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add HD-FDD related impact for Random access for RedCap UE with satellite access in clause 6.2E.2. - Adjust the format in clauses 7.1E.2.1 and 7.2E.2. - Add HD-FDD related impac for CSI-RS based candidate beam detection for RedCap UE with satellite access in clause 8.5E.6. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The HD-FDD related requirements for RedCap with NTN would be unclear. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 6.2E.2, 7.1E.2.1, 7.2E.2, 8.5E.3.2, 8.5E.6 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |
| This CR's revision history: |  | R4-2520106 |  |  |  |  |  |  |  |  |

==============First change==============

6.2E.2	Random access for RedCap UE with satellite access

6.2E.2.1	Introduction

This clause contains requirements on the RedCap UE regarding random access procedure. The random access procedure is initiated to establish uplink time synchronization for a UE which either has not acquired or has lost its uplink synchronization, or to convey UE’s request Other SI, or for beam failure recovery. The random access is specified in clause 8 of TS 38.213 [3] and the control of the RACH transmission is specified in clause 5.1 of TS 38.321 [7]. Two types of procedure are defined for the random access, the 4-step RA type, and the 2-step RA type [7]. The decision on which type of procedure to adopt is as described in clause 5.1.1 of TS 38.321 [7].

The 1 Rx RedCap UE for performing the random access procedure applies:

-	rsrp-ThresholdSSB as the signaled value of rsrp-ThresholdSSB [2] + 1 dB.

-	msgA-RSRP-ThresholdSSB as the signaled value of msgA-RSRP-ThresholdSSB [2] + 1 dB.

-	msgA-RSRP-Threshold as the signaled value of msgA-RSRP-Threshold [2] + 1 dB.

-	rsrp-ThresholdMsg3 as the signaled value of rsrp-ThresholdMsg3 [2] - 1 dB.

6.2E.2.2	Requirements

The requirements for the 4-step RA type procedure described in clause 6.2C.2.2 and the requirements for the 2-step RA type procedure described in the clause 6.2C.2.3 are applicable for TDD and FDD RedCap UEs. The 4-step and 2-step RA requirements for contention based random access defined in clause 6.2C.2.2 and 6.2C.2.3 respectively apply to HD-FDD UE with the following conditions:

- The RedCap UE operating in HD-FDD mode is not expected to perform PRACH transmission on a PRACH resource of a cell if UE has not received at least one SSB associated with that PRACH resource during the last Tp period in the cell, where Tp=160 ms.

- The RedCap UE operating in HD-FDD mode shall meet the PRACH requirements when performing PRACH transmission on a PRACH resource of a cell provided that the UE has received at least one SSB associated with that PRACH resource during the last Tp period before the PRACH transmission, where Tp=160 ms.

==============Next change==============

7.1E.2.1	Gradual timing adjustment

The requirements in clause 7.1C.2.1 for FR1-NTN shall apply.

7.2	UE timer accuracy

7.2.1	Introduction

UE timers are used in different protocol entities to control the UE behaviour.

==============Next change==============

7.2E.2	Requirements

The requirements in clause 7.2C.2 for FR1-NTN shall apply.

7.3	Timing advance

7.3.1	Introduction

The timing advance is initiated from gNB to UE in EN-DC, NR-DC, NE-DC and NR SA operation modes, with MAC message that implies the adjustment of the timing advance, as defined in clause 5.2 of TS 38.321 [7].

==============Next change==============

8.5E.3.2	Minimum requirement

The requirements in clause 8.5C.3.2 shall apply when FD-FDD and TDD RedCap UE is capable of 2 Rx. When FD-FDD and TDD RedCap UE is only required to support 1 Rx antenna, the requirements defined in clause 8.5C.3.2 shall apply except that:

-	TEvaluate_BFD_SSB_RedCap as defined in table 8.5E.3.2-1 for FR1.

Table 8.5E.3.2-1: Evaluation period TEvaluate_BFD_CSI-RS_RedCap for 1 Rx RedCap

| Configuration | TEvaluate_BFD_CSI-RS_RedCap (ms) |
| --- | --- |
| no DRX | Max(50, Ceil(2MBFD P PBFD) TCSI-RS) |
| DRX cycle ≤ 320 ms | Max(50, Ceil(21.5 × MBFD P PBFD) Max(TDRX, TCSI-RS)) |
| DRX cycle > 320 ms | Ceil(2MBFD P PBFD) TDRX |
| NOTE: TCSI-RS is the periodicity of CSI-RS resource in the set q0. TDRX is the DRX cycle length. |  |

For HD-FDD UE indicating CollisionHandlingOfHDFDDOperation, the requirements in clause 8.5E.3.2 shall also apply, provided that the following conditions are met:

- there is no collision between the CSI-RS resources for BFD measurements and UL transmission, or

- in case of collision between the CSI-RS resources for BFD measurements and UL transmission except PRACH, and ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision is not indicated by network.

In case of collision between the CSI-RS resources to be measured and PRACH, if UE indicates CollisionHandlingOfHDFDDOperation, it is up to HD-FDD UE implementation to either receive the CSI-RS or transmit the PRACH, according to TS 38.213.

==============Next change==============

8.5E.6	Requirements for CSI-RS based candidate beam detection for RedCap UE with satellite access

The requirements in clause 8.5C.6 shall apply when RedCap UE is capable of 2 Rx and 1 Rx FD-FDD and TDD RedCap UE.

For HD-FDD UE indicating CollisionHandlingOfHDFDDOperation, the requirements in clause 8.5C.3.2 shall also apply, provided that the following conditions are met:

- there is no collision between the CSI-RS resources for CBD measurements and UL transmission, or

- in case of collision between the CSI-RS resources for CBD measurements and UL transmission except PRACH, and ntn-PrioritizeUl-semiStatichalfDuplexRedCapcollision is not indicated by network.

In case of collision between the CSI-RS resources to be measured and PRACH, if UE indicates CollisionHandlingOfHDFDDOperation, it is up to HD-FDD UE implementation to either receive the CSI-RS or transmit the PRACH, according to TS 38.213.

==============End of change==============