TSG-RAN Working Group 4 (Radio) meeting #72	R4-144701

Dresden, Germany, 18 - 22 August, 2014

Source:	Nokia Networks, Nokia Corporation

Title:	Clarification of UL and DL CA configuration

Agenda item:	4.2.1.2 [LTE_CA-Core]

Document for:	Discussion

# Introduction

It has been pointed out in [1-6] that UL CA configuration is not clear enough in the current revision of TS36.101.

In [3] it is proposed to add a new column in Tables 5.6A.1-1, 5.6A.1-2, and 5.6A.1-3 to indicate whether UL CA is supported or not for each E-UTRA CA configuration. In [4-6], it is proposed to clarify that the current tables apply only for DL CA configurations; at the same time it is proposed to add another table for UL CA configurations.

This paper proposes another way forward, which can be used even for 5DL/5UL CAs, where each DL configuration has many possible UL CA configurations.

# Discussion

E-UTRA CA configurations and bandwidth combination sets are defined in Tables 5.6A.1-1, 5.6A.1-2, and 5.6A.1-3 of TS 36.101. In these tables, the bandwidth combination sets are defined for each E-UTRA CA configuration. The supported bandwidth combination sets are not signaled separately for downlink and uplink in the UE capability signaling. Currently it is understood that the configuration is designed for the downlink CA. The uplink configuration are explained in the notes of Tables 5.6A.1-1 and 5.6A.1-2 indicating “For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal.” This explains what uplink configuration can be used with downlink. However it is uncler if all the UL configurations are possible or not.

The proposal [3] is to add clarification whether UL CA is supported or not for each bandwidth combination set. Some CA band combinations are specified without UL CA in RAN4; thus this proposal is useful to identify which CA supports UL CA. However, this means that two bandwidth combination sets shall be defined for the same DL CA configuration with and without UL CA. This is redundant since information about the bandwidth combination set is essentially the same regardless of UL CA. The UE capability signaling already supports UL CA capability by signaling the supported band combinations and CA bandwidth classes. Therefore it is not necessary to include another UE capability information signaled as the supported bandwidth combination sets. Furthermore, it is expected there are many UL CA configurations in the future because of the increasing number CCs in uplink. Therefore it will be necessary to indicate which UL CAs are possible in RAN4 rather than to indicate all or nothing.

The proposal [4-6] is to clarify that the current tables are for downlink and add new tables for uplink. This might make sense if the supported bandwidth combination sets are signaled separately for uplink and downlink; however, they are not signaled separately. In the current RRC specification, supportedBandwidthCombinationSet is signaled for each CA band combination, thus it is applied to both uplink and downlink. Furthermore, the bandwidth combination set table is not expected to be different from downlink to uplink. Thus adding another table for uplink is unnecessary. Therefore, it is recommended to have one table only as the RRC signaling is designed in such a way.

In conclusion, what to clarify in the current TS36.101 is

- The bandwidth combination set tables are designed for downlink CA and resued for uplink CA; however this relationship is not well presented.

- It is unclear which UL CAs are supported in RAN4 as some UL CAs are not yet specified.

Therefore we propose that E-UTRA CA configuration is split to DL and UL in the bandwidth combination set tables. For UL, the highest order UL CA configuration(s) are included to show which CAs are specified in RAN4. All the lower order fallback CAs (including non-CA in UL) are assumed specified as well; it is not necessary to list all the lower order UL CAs. The proposed new tables for Rel-12 are presented in Annex A. Modifications from the current revision of TS36.101 (v12.4.0) are highlighted in red.

The proposed new table structure has the following benefits.

- The changes to the existing specifications are minimum; there is no impact to high layer.

- DL and UL CA configurations together with the bandwidth combination sets are presented in one table, which is consistent with UE capability signaling in RRC.

- It is easy to clarify UL CA restrictions. For example, CA band 1, band 41, and band 41 [7] is proposed to support only Band 1 for the uplink. The band 41 uplink can be supported in intra-band CA but is not supported in inter-band CA. The new table structure can easily clarify this behavior by not including band 41 in the uplink inter-band CA configuration.

A question is whether the table gets too crowded when more CA configurations are introduced in future RAN4 meetings. We have made exercises in Annex B to illustrate how the table look like for 5CC CAs up to 5 DL bands and 2 UL bands. In the example tables, it is assumed all UL band combinations are possible. This might be an extreme assumption because some UL band restrictions (like CA band 1, band 41 and band 41) would be expected for an increasing number of UL bands. If there is any limitation in uplink bands, the number of UL CAs  would be further reduced and thus the tables in Annex B are more simplified. Though it is not presented in Annex B, the maximum number of UL CAs for 3UL and 4UL bands is similar to 2UL bands. The proposed table structure is expected to be futureproof.

# Conclusion

We propose a new structure for the bandwidth combination sets that clarify both DL and UL CA configurations in the bandwidth combination set tables. The proposed new tables for Rel-12 (TS36.101 v12.4.0) are presented in Annex A, for intra-band contiguous, inter-band, and non-contiguous intra-band CAs.

The new structure can be used for 5CC CAs for any number of DL and UL bands as some examples are exercised in Annex B. It is also useful to introduce uplink band restrictions in the new table, which are expected in future CA band combinations.

CRs based on the proposal in this paper are available in [8-10].

# Reference

- R4-141816	Designation of UL and DL CA classes and configurations, 36.101 CR-2242 (Rel-12), Qualcomm Incorporated

- R4-142427	Designation of UL and DL CA classes and configurations, 36.101 CR-2242rev1 (Rel-12), Qualcomm Incorporated

- R4-142926	Designation of UL and DL CA configurations, 36.101 CR-2346 (Rel-12), Qualcomm Incorporated

- R4-142973	Clarification on UL and DL CA, 36.101 CR-2366 (Rel-10), Ericsson

- R4-142974	Clarification on UL and DL CA, 36.101 CR-2367 (Rel-11), Ericsson

- R4-142975	Clarification on UL and DL CA, 36.101 CR-2368 (Rel-12), Ericsson

- RP-140963	New WI: LTE Advanced 3 Band Carrier Aggregation (3DL/1UL) of Band 1, Band 41 and Band 41, KDDI

- R4-144702	Clarification of UL and DL CA configuration (Rel-10), Nokia Networks, Nokia Corporation

- R4-144703	Clarification of UL and DL CA configuration (Rel-11), Nokia Networks, Nokia Corporation

- R4-144704	Clarification of UL and DL CA configuration (Rel-12), Nokia Networks, Nokia Corporation

# Annex A: The proposed new tables for the bandwidth combination sets

Table 5.6A.1-1: E-UTRA CA configurations and bandwidth combination sets defined for intra-band contiguous CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration |  | Component carriers in order of increasing carrier frequency |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| Downlink | Uplink |  |  |  |  |  |
| CA_1C | CA_1C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
| CA_3C | CA_3C | 5, 10, 15 | 20 |  | 40 | 0 |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_7C | CA_7C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
|  |  | 10 | 20 |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
| CA_23B | CA_23A | 10 | 10 |  | 20 | 0 |
|  |  | 5 | 15 |  |  |  |
| CA_27B | CA_27A | 1.4, 3, 5 | 5 |  | 13 | 0 |
|  |  | 1.4, 3 | 10 |  |  |  |
| CA_38C | CA_38C | 15 | 15 |  | 40 | 0 |
|  |  | 20 | 20 |  |  |  |
| CA_39C | CA_39C | 5,10,15 | 20 |  | 35 | 0 |
|  |  | 20 | 5, 10, 15 |  |  |  |
| CA_40C | CA_40C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15 |  |  |  |
|  |  | 20 | 10, 20 |  |  |  |
| CA_41C | CA_41C | 10 | 20 |  | 40 | 0 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 10, 15, 20 |  |  |  |
|  |  | 5, 10 | 20 |  | 40 | 1 |
|  |  | 15 | 15, 20 |  |  |  |
|  |  | 20 | 5, 10, 15, 20 |  |  |  |
| CA_40D | CA_40C | 10, 20 | 20 | 20 | 60 | 0 |
|  |  | 20 | 10 | 20 |  |  |
|  |  | 20 | 20 | 10 |  |  |
| CA_41D | CA_41C | 10 | 20 | 15 | 60 | 0 |
|  |  | 10 | 15, 20 | 20 |  |  |
|  |  | 15 | 20 | 10, 15 |  |  |
|  |  | 15 | 10, 15, 20 | 20 |  |  |
|  |  | 20 | 15, 20 | 10 |  |  |
|  |  | 20 | 10, 15, 20 | 15, 20 |  |  |
| CA_42C | CA_42C | 5, 10, 15, 20 | 20 | 5, 10, 15, 20 | 40 | 0 |
|  |  | 20 | 5, 10, 15 | 20 |  |  |
| NOTE 1: The CA configuration refers to an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. |  |  |  |  |  |  |

Table 5.6A.1-2: E-UTRA CA configurations and bandwidth combination sets defined for inter-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| Downlink | Uplink |  |  |  |  |  |  |  |  |  |
| CA_1A-5A | CA_1A, CA_5A | 1 |  |  |  | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  |  | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-8A | CA_1A, CA_8A | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-11A | CA_1A, CA_11A | 1 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 11 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-18A | CA_1A, CA_18A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 18 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-19A | CA_1A, CA_19A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-20A | CA_1A, CA_20A | 1 |  |  | Yes | Yes | Yes |  | 25 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_1A-21A | CA_1A, CA_21A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_1A-26A | CA_1A, CA_26A | 1 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 1 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-4A | CA_2A, CA_4A | 2 | Yes | Yes | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 4 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-5A | CA_2A, CA_5A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-12A | CA_2A, CA_12A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-13A | CA_2A, CA_13A | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_2A-17A | CA_2A, CA_17A | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_2A-29A | CA_2A | 2 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 2 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-5A | CA_3A, CA_5A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-7A | CA_3A, CA_7A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_3A-8A | CA_3A, CA_8A | 3 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  |  | Yes |  |  | 20 | 1 |
|  |  | 8 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-19A | CA_3A, CA_19A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 19 |  |  | Yes | Yes | Yes |  |  |  |
| CA_3A-20A | CA_3A, CA_20A | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_3A-26A | CA_3A, CA_26A | 3 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 26 |  |  | Yes | Yes | Yes |  |  |  |
|  |  | 3 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 26 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-27A | CA_3A, CA_27A | 3 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 27 |  |  | Yes | Yes |  |  |  |  |
| CA_3A-28A | CA_3A, CA_28A | 3 |  |  | Yes | Yes | Yes | Yes | 40 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-5A | CA_4A, CA_5A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 5 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-7A | CA_4A, CA_7A | 4 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_4A-12A | CA_4A, CA_12A | 4 | Yes | Yes | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 | Yes | Yes | Yes | Yes | Yes | Yes | 30 | 1 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-13A | CA_4A CA_13A | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 13 |  |  |  | Yes |  |  |  |  |
| CA_4A-17A | CA_4A CA_17A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_4A-29A | CA_4A | 4 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
|  |  | 4 |  |  | Yes | Yes | Yes | Yes | 30 | 2 |
|  |  | 29 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-7A | CA_5A, CA_7A | 5 | Yes | Yes | Yes | Yes |  |  | 30 | 0 |
|  |  | 7 |  |  |  | Yes | Yes | Yes |  |  |
| CA_5A -12A | CA_5A, CA_12A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 12 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-17A | CA_5A, CA_17A | 5 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 17 |  |  | Yes | Yes |  |  |  |  |
| CA_5A-25A | CA_5A, CA_25A | 5 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-20A | CA_7A, CA_20A | 7 |  |  |  | Yes | Yes | Yes | 30 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 7 |  |  |  | Yes | Yes | Yes | 40 | 1 |
|  |  | 20 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_7A-28A | CA_7A, CA_28A | 7 |  |  | Yes | Yes | Yes | Yes | 35 | 0 |
|  |  | 28 |  |  | Yes | Yes | Yes |  |  |  |
| CA_8A-20A | CA_8A, CA_20A | 8 |  |  | Yes | Yes |  |  | 20 | 0 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
|  |  | 8 |  | Yes | Yes | Yes |  |  | 20 | 1 |
|  |  | 20 |  |  | Yes | Yes |  |  |  |  |
| CA_8A-40A | CA_8A, CA_40A | 8 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 40 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_11A-18A | CA_11A, CA_18A | 11 |  |  | Yes | Yes |  |  | 25 | 0 |
|  |  | 18 |  |  | Yes | Yes | Yes |  |  |  |
| CA_12A-25A | CA_12A, CA_25A | 12 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 25 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_19A-21A | CA_19A, CA_21A | 19 |  |  | Yes | Yes | Yes |  | 30 | 0 |
|  |  | 21 |  |  | Yes | Yes | Yes |  |  |  |
| CA_20A-32A | CA_20A | 20 |  |  | Yes | Yes |  |  | 30 | 0 |
|  |  | 32 |  |  | Yes | Yes | Yes | Yes |  |  |
| CA_23A-29A | CA_23A | 23 |  |  | Yes | Yes | Yes | Yes | 30 | 0 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
|  |  | 23 |  |  | Yes | Yes |  |  | 20 | 1 |
|  |  | 29 |  | Yes | Yes | Yes |  |  |  |  |
| CA_39A-41A | CA_39A, CA_41A | 39 |  |  |  | Yes | Yes | Yes | 40 | 0 |
|  |  | 41 |  |  |  |  |  | Yes |  |  |
| NOTE 1: The CA Configuration refers to a combination of an operating band and a CA bandwidth class specified in Table 5.6A-1 (the indexing letter). Absence of a CA bandwidth class for an operating band implies support of all classes. NOTE 2: For each band combination, all combinations of indicated bandwidths belong to the set. NOTE 3: For the supported CC bandwidth combinations, the CC downlink and uplink bandwidths are equal. |  |  |  |  |  |  |  |  |  |  |

Table 5.6A.1-3: E-UTRA CA configurations and bandwidth combination sets defined for non-contiguous intra-band CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration |  | Component carriers in order of increasing carrier frequency |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| Downlink | Uplink |  |  |  |  |
| CA_2A-2A | CA_2A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_3A-3A | CA_3A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_4A-4A | CA_4A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |
| CA_7A-7A | CA_7A | 5 | 15 | 40 | 0 |
|  |  | 10 | 10, 15 |  |  |
|  |  | 15 | 15, 20 |  |  |
|  |  | 20 | 20 |  |  |
| CA_23A-23A | CA_23A | 5 | 10 | 15 | 0 |
| CA_25A-25A | CA_25A | 5, 10 | 5, 10 | 20 | 0 |
| CA_41A-41A | CA_41A | 10, 15, 20 | 10, 15, 20 | 40 | 0 |
|  |  | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 1 |
| CA_41A-41C | CA_41C | 5, 10, 15, 20 | See Table 5.6A.1-1 | 60 | 0 |
| CA_41C-41A | CA_41C | See Table 5.6A.1-1 | 5, 10, 15, 20 | 60 | 0 |
| CA_42A-42A | CA_42A | 5, 10, 15, 20 | 5, 10, 15, 20 | 40 | 0 |

# Annex B: Examples of CA up to 5DL/5UL

Example 1: 5DL/5UL intra-band contiguous CA

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA configuration |  | Component carriers in order of increasing carrier frequency |  |  |  |  | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
|  |  | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] | Allowed channel bandwidths for carrier [MHz] |  |  |
| Downlink | Uplink |  |  |  |  |  |  |  |
| CA_xF | CA_xF | 10, 15, 20 | 20 | 20 | 20 | 20 | 40 | 0 |
|  |  | 20 | 20 | 20 | 20 | 10, 15, 20 |  |  |

Example 2: 5DL/5UL inter-band CA (2 UL/DL bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| Downlink | Uplink |  |  |  |  |  |  |  |  |  |
| CA_xA-yE | CA_xA-yE | x |  |  |  | Yes | Yes | Yes | 100 | 0 |
|  |  | y | See Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_xC-yD | CA_xC-yD | x | See Table 5.6A.1-1 |  |  |  |  |  | 100 | 0 |
|  |  | y | See Table 5.6A.1-1 |  |  |  |  |  |  |  |

Example 3: 5DL/4UL inter-band CA (3 DL bands and 2 UL bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| Downlink | Uplink |  |  |  |  |  |  |  |  |  |
| CA_xA_yA_zD | CA_xA-yA, CA_xA-zD, CA_yA-zD | x |  |  |  | Yes | Yes | Yes | 100 | 0 |
|  |  | y |  |  |  | Yes | Yes | Yes |  |  |
|  |  | z | See Table 5.6A.1-1 |  |  |  |  |  |  |  |
| CA_xA_yC_zC | CA_xA-yC, CA_xA-zC, CA_yC-zC | x |  |  |  | Yes | Yes | Yes | 100 | 0 |
|  |  | y | See Table 5.6A.1-1 |  |  |  |  |  |  |  |
|  |  | z | See Table 5.6A.1-1 |  |  |  |  |  |  |  |

Example 4: 5DL/3UL inter-band CA (4DL bands and 2 UL bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| Downlink | Uplink |  |  |  |  |  |  |  |  |  |
| CA_xA_yA_zA_wC | CA_xA-yA, CA_xA-zA, CA_xA-wC, CA_yA-zA, CA_yA-wC, CA_zA-wC | x |  |  |  | Yes | Yes | Yes | 100 | 0 |
|  |  | y |  |  |  | Yes | Yes | Yes |  |  |
|  |  | z |  |  |  | Yes | Yes | Yes |  |  |
|  |  | w | See Table 5.6A.1-1 |  |  |  |  |  |  |  |

Example 5: 5DL/2UL inter-band CA (5DL bands and 2 UL bands)

| E-UTRA CA configuration / Bandwidth combination set |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA CA Configuration |  | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth combination set |
| Downlink | Uplink |  |  |  |  |  |  |  |  |  |
| CA_xA_yA_zA_wA_qA | CA_xA-yA, CA_xA-zA, CA_xA-wA, CA_xA-qA, CA_yA-zA, CA_yA-wA, CA_yA-qA, CA_zA-wA, CA_zA-qA, CA_wA-qA | x |  |  |  | Yes | Yes | Yes | 100 | 0 |
|  |  | y |  |  |  | Yes | Yes | Yes |  |  |
|  |  | z |  |  |  | Yes | Yes | Yes |  |  |
|  |  | w |  |  |  | Yes | Yes | Yes |  |  |
|  |  | q |  |  |  | Yes | Yes | Yes |  |  |