3GPP TSG-RAN WG4 Meeting #75	R4- 153220

# Fukuoka, Japan, 25-29 May. 2015

Title:	Discussion on Pcell mandatory Support

Source:	CMCC

Agenda Item:	5.1

Document for:	Approval

- Introduction

During RAN#67, a Way forward on “Mandating Pcell support in UE for all bands of LTE-CA band combinations” was agreed [1]. And in RAN4#74bis, another way forward was agree [2]. This document propose the request for some band combination based on the latter way forward.

- Discussion on Pcell mandatory

2.1 Agreed Way Forward

In RAN4#74bis, following way forward was agree [2].

1. For the existing CA combinations, PCell mandatory support is applied to the constituting bands of a CA combination which received “request” and/or “exception” (Refer to RP-150476 about “request” and “exception”). Mandatory PCell support is applied in following manner

1-1:    Bands are considered to have mandatory Pcell support for inter band FDD-FDD CA and TDD-TDD CA if not captured in the following three Tables in TS36.101.

- Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

- Table 7.3.1A-0bA: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for two bands)

- Table 7.3.1A-0bC: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions for three bands)

1-2:    Operator specific CA combinations such as CA_18A-28A bands are considered to have mandatory PCell support regardless of the above 1-1 and CA types such as FDD-FDD, TDD-TDD and TDD-FDD CA

1-3:    CA combinations where a “request” and “exception info” is provided, a decision on PCell mandatory support in all aggregated carriers will be discussed in case-by-case basis.

- The CA combinations not covered by the above cases will be discussed in RAN4#75.

- How to capture the results into the 3GPP specification is addressed in the next meetings.

2.2 TDD-FDD CA

The reference architecture and complexity of TDD-FDD CA of Class A1 combination (Low-High combination without harmonic or intermodulation) is similar to other FDD-FDD A1 combinations.

Proposal#1: Bands are considered to have mandatory Pcell support for inter band TDD-FDD CA of class A1 combination.

2.3 Class A2 CA

If we check Table 7.3.1A-0a in TS 36.101 Rel-12, we will find that no requirement is needed for CA_3A-8A, CA_26A-41A and CA_26A-41C (note 4 and note 8). That’s because these A2 combinations do not consider HTF, the RF reference architecture keeps the same as A1 combinations.

Proposal#2: Bands are considered to have mandatory Pcell support for inter band FDD-FDD CA, TDD-TDD and TDD-FDD CA of A2 combination if no HTF is considered.

Table 7.3.1A-0a: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (exceptions)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_1A-28A5,6 | 1 |  |  | [-89.8] | [-89.4] | [-89] | [-88.7] | FDD |
|  | 28 |  |  | -98.3 | -95.3 | -93.5 | -90.8 |  |
| CA_3A-8A4 | 3 |  |  |  | N/A | N/A | N/A | FDD |
|  | 8 |  |  | N/A | N/A |  |  |  |
| CA_4A-12A5,6 | 4 | -89.2 | -89.2 | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-17A5,6 | 4 |  |  | -90 | -89.5 |  |  | FDD |
|  | 17 |  |  | -96.5 | -93.5 |  |  |  |
| CA_2A-4A-12A5,6 | 2 |  |  | -97.7 | -94.7 | -92.9 | -91.7 | FDD |
|  | 4 |  |  | -90 | -89.5 | -89 | -88.5 |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-4A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-5A-12A5,6 | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 5 |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-7A-12A5,6 | 4 |  |  | [-90] | [-89.5] | [-89] | [-88.5] | FDD |
|  | 7 |  |  | -97.5 | -94.5 |  |  |  |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_4A-12B5,6 | 4 |  |  | -90 | -89.5 | -89 | -88.5 | FDD |
|  | 12 |  |  | -96.5 | -93.5 |  |  |  |
| CA_26A-41A8 | 26 |  |  | N/A | N/A | N/A |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_26A-41C8 | 26 |  |  | N/A | N/A | N/A |  | FDD |
|  | 41 |  |  | N/A | N/A | N/A | N/A | TDD |
| CA_7A-8A5,6 | 7 |  |  |  | -87.4 | -87 | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |
| CA_7A-8A-20A5,6 | 7 |  |  |  | -87.4 | -87 | -86.7 | FDD |
|  | 8 |  | -99 | -96.8 | -93.8 |  |  |  |
|  | 20 |  |  | [-96.8] | [-93.8] |  |  |  |
| NOTE 1: The transmitter shall be set to PUMAX as defined in subclause 6.2.5A. NOTE 2: Reference measurement channel is A.3.2 with one sided dynamic OCNG Pattern OP.1 FDD/TDD as described in Annex A.5.1.1/A.5.2.1 NOTE 3: The signal power is specified per port NOTE 4: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 2nd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). NOTE 5: These requirements apply when there is at least one individual RE within the uplink transmission bandwidth of a low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of a high band. NOTE 6: The requirements should be verified for UL EARFCN of a low band (superscript LB) such that in MHz and with the carrier frequency of a high band in MHz and the channel bandwidth configured in the low band. NOTE 7: Void. NOTE 8: No requirements apply when there is at least one individual RE within the uplink transmission bandwidth of the low band for which the 3rd transmitter harmonic is within the downlink transmission bandwidth of the high band. The reference sensitivity is only verified when this is not the case (the requirements specified in clause 7.3.1 apply). |  |  |  |  |  |  |  |  |

- Conclusion

Proposal#1: Bands are considered to have mandatory Pcell support for inter band TDD-FDD CA of class A1 combination.

Proposal#2: Bands are considered to have mandatory Pcell support for inter band FDD-FDD CA, TDD-TDD and TDD-FDD CA of A2 combination if no HTF is considered.

- Reference

- [1] RP-150476, “Way forward on Mandating Pcell support in UE for all bands of LTE-CA band combinations”, Vodafone, TeliaSonera, Orange, Telecom Italia, Deutsche Telekom, Telefónica, NTT DOCOMO, INC., CHTTL, Dish Network, China Mobile, Sprint

[2] R4-152433, “Way forward on PCell mandatory support”, NTT DOCOMO, INC., Telecom Italia, Vodafone, Orange