3GPP TSG RAN WG4 Meeting #76	R4-154252

Beijing, China, 24 – 28 August, 2015

Agenda Item:	9.4.5

Source:	Huawei, HiSilicon

Title:	Discussion on PRACH performance under high speed scenario

Document for:	Discussion

Introduction

In this contribution we will further discuss the potential issue for PRACH in the high speed scenario. According our analysis, the existing restrict set of PRACH sequence when High-speed-flag is set to one could not fully support high speed scenario, and the enhancement would be needed.

Background

More and more high-speed-railways are being constructed, and the speed is higher and higher. Nowadays, the designed highest speed train could be beyond 400km/h. The higher and higher speed brings challenges to the current LTE mobile communication system which is because that higher speed will cause higher Doppler frequency shifts to the wireless signal over the air, and the higher Doppler frequency shifts will result in system performance degradation.

Furthermore, assuming the frequency shift of the downlink caused by Doppler frequency is fd, the frequency shifts caused by the UE speed in the uplink will be two times of the Doppler shifts in the downlink due to that the UE will synchronize on an frequency that the carrier plus the Doppler frequency and transmit uplink signal at the synchronized frequency, and an extra Doppler frequency will be added to the uplink signal received by the base station.

One of the functions impacted is the random access. In the current LTE system, the random access is design to cope with the Doppler frequency shift no larger than the preamble signal subcarrier spacing. But with higher speed or higher carrier frequency, the Doppler frequency shifts will be larger than one subcarrier spacing of the preamble signal.

The above figure shows the uplink Doppler frequency shifts along with the carrier frequency and the terminal moving speed. For example for carrier 2.6GHz, speed of 350km/h and 400km/h will result in the Doppler frequency of 1685 and 1925 Hz at the eNB side which is 1.34 and 1.54 times of the preamble subcarrier spacing (1250Hz). The Doppler frequency shifts @2.6GHz with speed beyond 350km/h will exceed the design of the LTE system for PRACH capability. More analysis will be in the following sections.

Table 1: the Doppler frequency along with the carrier frequency and UE speed

| UE speed (km/h) | Doppler frequency |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | 1.9GHz carrier |  | 2.3 GHz carrier |  | 2.6 GHz carrier |  | 3.5 GHz carrier |  |
| DL/UL | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink | Downlink | Uplink |
| 150 | 264 | 528 | 319 | 639 | 361 | 722 | 486 | 972 |
| 200 | 352 | 704 | 426 | 852 | 481 | 963 | 648 | 1296 |
| 250 | 440 | 880 | 532 | 1065 | 602 | 1204 | 810 | 1620 |
| 300 | 528 | 1056 | 639 | 1278 | 722 | 1444 | 972 | 1944 |
| 350 | 616 | 1231 | 745 | 1491 | 843 | 1685 | 1134 | 2269 |
| 400 | 704 | 1407 | 852 | 1704 | 963 | 1926 | 1296 | 2593 |
| 450 | 792 | 1583 | 958 | 1917 | 1083 | 2167 | 1458 | 2917 |

Doppler frequency impact on LTE random access

For the random access of LTE, one preamble is sent by the UE who is about to access the network, and at the base station side, the eNB will detect the uplink preamble by correlation between the local preamble and the received uplink preamble signal. However, higher frequency shifts will result in timing ambiguity due to that multiple correlation peaks may be caused by the frequency shifts. In LTE spec, High-speed-flag is used to support high speed UE to access the network, where 3 detection windows [1, 2] for one UE is employed to resolve the timing and frequency offset ambiguity, so that the correct timing and preamble identity can be detected.

In [1], the root sequence and its preambles by cyclic shifting is selected and calculated according to

Cv is the cyclic shifts for each preamble sequence using the same root sequence.

For, the parameters are given by

# For, the parameters are given by

For all other values of, there are no cyclic shifts in the restricted set.

The variable  is the cyclic shift corresponding to a frequency shift of one preamble subcarrier spacing. Different root sequence may have different value of du.

For preamble root sequence u=702, du= 49 and Ncs= 49, it can be calculated that Cv=130v, v=0,……5.

The maximum frequency offset supported by the current spec is limited to around one subcarrier spacing of the preamble signal which is 1.25 KHz. However, when higher speed (above 350 Km/h) or higher carrier frequency (band 22/42/43) is applied, the timing and frequency offset ambiguity still exists, which will degrade the uplink random access performance (higher random access failure rate, higher false alarm rate and increased random access latency). Two examples are presented as below where one UE is using preamble sequence with Cv=0 for the random access.

Figure 1: correlation of the local preamble sequence and the received preamble signal with frequency shift of 1.4 times of the preamble subcarrier spacing

The above figure shows an example of the correlation results of the local preamble sequence (u=702, du= 49) and the received preamble signal with a frequency shift of 1.4 times of the preamble subcarrier spacing which is 1.75 KHz. From the correlation result, two main peaks emerge at du+delay and 2×du+delay. At this correlation result, the eNB will have large probability of wrong detection of the preamble sequence. And high probability of random access failure if another UE is using the preamble sequence with Cv=1 for random access due to the high interference from the first UE with Cv=dstart;

Figure 2: correlation of the local preamble sequence and the received preamble signal with frequency shift of 1.9 times of the preamble subcarrier spacing

The above figure shows an example of the correlation results of the local preamble sequence and the received preamble signal with a frequency shift of 1.9 times of the preamble subcarrier spacing which is 2.37 KHz. And it can be seen that the strongest peak is at the point of 2×du+delay which is out of the detection window for the root sequence and acting as the interference to the UE using the preamble with Cv=dstart. And the peaks in the detection window of the preamble for the root sequence ( Cv=0) is too small to be detected, and the UEs using the preamble sequence with Cv=0 and  Cv=dstart will both encounter random access failure due to the interference.

Observation : The higher Doppler frequency will cause random access failure of the due to correlation shifted out of the detection window or due to high interference from other users with high Doppler frequency.

Proposal : PRACH enhancement should be considered for the scenario with the higher UE speed beyond 350km/h or with the higher frequency band.

Conclusion

Observation : higher Doppler frequency will cause random access failure of the due to correlation shifted out of the detection window or due to high interference from other users with high Doppler frequency.

Proposal : PRACH enhancement should be considered for the scenario with the higher UE speed beyond 350km/h or with the higher frequency band.

References

36.211, E-UTRA: Physical channels and modulation, version 8.9.0

R1-133569 , “ clarification on Rel-8 PRACH related operation, ” 3GPP TSG RAN WG1 #74