3GPP TSG-RAN WG4 Meeting #77	R4-157459

# Anaheim, US, 16-20 Nov. 2015

Title:	Discussion on BS RF requirement for LAA

Source:	CMCC

Agenda Item:	7.13.3

Document for:	Approval

- Introduction

In the last two meeting, BS RF requirement for LAA was extensively discussed. The controversial issues mainly focus on ACLR requirement and spurious requirement under one/sub-band filter assumption.

In this paper, we show the difference between LAA band and tradition TDD band on co-existence aspects. And practical scenario is also given to deduce the reasonable ACLR and other RF requirement for co-existence.

- Difference between LAA band and traditional TDD band

Currently, LAA band is proposed to operate in TDM manner, but it is different from traditional TDD system operation. For TDD system in licensed band, UL-DL interference needs to be avoided. Two existing solutions are implemented in the practical network:

    - Synchronization operation between adjacent channels

    - Guard band + stringent co-existence requirement

In unlicensed band, for LAA with both DL and UL transmission will face the same problem as traditional TDD system. Besides of this, although it is agreed that in Rel-13 only DL is specified, LAA BS with LBT scheme has to perform a clear channel assessment (CCA) to determine if the channel is free for use. The CCA operation can be viewed as a kind of UL receiving operation, which can also be affected by adjacent DL transmission. If the interference from different channel is higher enough, the BS will view the channel as being used. This kind of interference can only be avoided by appropriate co-existence requirement since neither synchronization operation nor guard band is feasible for unlicensed band. In other words, two nearby LAA BSs operating in adjacent frequency may not have the possibility to transmit simultaneously if co-existence requirement is not sufficient.

Obersvation#1: For LAA system, the CCA operation can be affected by adjacent channel interference even Rel-13 LAA only considers DL operation.

Obersvation#2: Two nearby LAA BSs operating in different channel may not have the possibility to transmit simultaneously if co-existence requirements such as ACLR, additional co-existence requirement and co-location requirement are not sufficient.

- Adjacent channel requirements

  - ACLR requirement

In the last two RAN4 meeting, ACLR requirement for LAA BS was discussed, but no conclusion was made. Some companies proposed to maintain the E-UTRA 45dB requirement while others wanted to relax it. The main argument of supporting relaxed requirement is that if the same band is shared with other devices with much looser RF requirement, those devices will be the bottleneck in terms of interference. However, pure LAA-LAA co-existence scenario is also a typical case.

    - In some country or region, part of the 5GHz has the potential to be LAA only or majority, especially for the one that not allocated yet or rarely used. Take China as an example, 5150-5250 and 5250-5350MHz was allocated in 2012, but these bands are quite clean till now. Most of the WLAN APs (owned by operators, enterprises or individuals) use 5.8GHz band. When operators decide to deploy LAA, these clean bands are quite attractive.

    - In a certain well-controlled area, such as big office, factory, the owner have the power to deploy LAA BS only in terms of high performance.

Observation#3: LAA-LAA co-existence scenario is typical case in practical network.

Here we define minimal separation distance for CCA, as the minimal distance between the two BSs operating in different channel in unlicensed band do not affect LBT CCA procedure of each other. If the minimal separation distance is too large, it is hard for operators to deploy efficiently.

For the LAA-LAA adjacent channel co-existence scenario, we calculate the minimal separation distance under different ACLR requirement. The parameters were collected in table 1. Since different frequency blocks has different maximum EIRP regulatory requirement as well as the path loss formula also depends on frequency, we calculate the minimal separation distance respectively for these three frequency blocks. For LBT, CCA-ED threshold is assumed in the range between -62dBm and -82dBm [1], and here we assume the upper bound value -62dBm, i.e. the minimal separation distance is the most conservative one.

Table 1. Parameters for unlicensed band

| Frequency Blocks（MHz） | 5150-5350 | 5470-5725 | 5725-5850 |
| --- | --- | --- | --- |
| Max EIRP (dBm/20MHz) | 23 | 30 | 36 |
| Assumed antenna gain (dB) | 6 | 6 | 6 |
| CCA-ED threshold (dBm/20MHz) | -62 | -62 | -62 |

Note: Max EIPR requirement for Europe is assumed

Figure 1. Minimal separation distance under different ACLR requirement

In Figure 1, we can see that even for the most conservative value, 5-20m separation distances is needed to guarantee the received adjacent channel leakage being not over the CCA-ED threshold when ACLR is set to 30dB.  Larger ACLR requirement can help reduce the Minimal separation distance a lot. Especially, only 1-4m separation distance is needed when ACLR maintains 45dB. To maintain 45dB ACLR requirement for LAA BS will increase the system performance and spectrum efficiency for LAA-LAA co-existence scenario.

Obersvation#4: Larger ACLR requirement can help reduce the minimal separation distance a lot.

Proposal#1: 45dB ACLR should be maintained for LAA BS.

  - ACS requirement

Adjacent channel interference not only depends on transmitter ACLR requirement but also ACS requirement for receiver. ACS requirement should be at the similar level adapting to 45dB ACLR requirement. In current specification, ACS requirement is also about 45dB. Therefore, legacy ACS requirement should be maintained for LAA BS.

Proposal#2:  Legacy ACS requirement should be maintained for LAA BS.

- Additional co-existence and co-location requirement

  - Spurious emission requirement for the co-location

In the last meeting, whether to introduce sub-band definition and sub-band RF requirements for LAA BS were discussed. If the LAA BS uses one single band filter (5150-5925), only operating band unwanted emission could be applied for each sub-band (5150-5250, 5250-5350, 5470-5725, 5725-5925) to protect each other. In Table 2 we can see that, by applying operating band unwanted emission requirement, minimal separation distance for CCA cannot satisfy co-location deployment scenario or even for co-existence scenario. In a practical scenario, operators always share the limited BS sites or use nearby BSs. If co-located or additional co-existence requirement doesn’t apply, no matter how large the operating frequency separation is, two co-located LAA BSs cannot transmit simultaneously.

Table 2 minimal separation distance of operating band unwanted emission requirement for CCA

| BS Class | Operating band unwanted emission 10.05 MHz f_offset < f_offsetmax for 20MHz bandwidth | Minimal separation distance |  |
| --- | --- | --- | --- |
|  |  | CCA threshold -62dBm/20MHz | CCA threshold -82dBm/20MHz |
| Local Area | -37dBm/100kHz | 4.36m | 43.6m |
| Home BS (20dBm) | -32dBm/1MHz | 2.45m | 24.5m |
| Medium Range (30dBm) | -29dBm/1MHz | 3.46m | 34.6m |

If sub-band filter is assumed, spectrum efficiency and network flexibility could be increased. Four co-located or nearby LAA BSs operating in different sub-band could work independently serving different UEs when co-located requirement is well defined.

For LAA DL only BS, co-location requirement only depends on CCA detection threshold. Assuming the MCL between two BS is 30dB, spurious emission requirement for the co-location is listed in table 3.

Table 3 Required spurious emission requirement for the co-location

| CCA threshold | -62dBm/20MHz | -82dBm/20MHz |
| --- | --- | --- |
| Spurious emission requirement for the co-location | -55dBm/100kHz | -75dBm/100kHz |

For LAA BS with UL, the spurious emission requirement for the co-location depends on the reference sensitivity. It is propose to reuse the Medium Range, Local Area and Home BS requirement.

Proposal#3: For LAA DL only BS, spurious emission requirement for co-location is in the range of [-75 to -55] dBm depending on CCA threshold agreed in RAN1

Proposal#4: For LAA BS with UL, the spurious emission requirement for co-location could reuse the Medium Range, Local Area and Home BS requirement.

As 5150-5250 and 5250-5350, 5470-5725 and 5725-5925 are two pairs of adjacent band, such tight requirement is hard to applied at the band edge. Therefore, a certain excluded frequency range such as 10MHz could also be defined.

  - Spurious emission requirement for the additional co-existence

As LAA band is in TDM manner, additional co-existence depends on the tighter value of inference to BS and UE. For interfering UE case, the requirement can be calculated as:

Additional co-existence to UE = UE thermal noise – MCL = [-101, -100] – (-53) = [-48, -47]

Note: thermal noise range is base on [-90, -89] REFSENSE proposed in the last meeting.

For interfering BS case, especially for LAA DL only BS, the requirement can be calculated as:

Additional co-existence to DL only BS = CCA threshold – MCL = [-82, -62] – (-67) = [-15, +5]

For LAA BS with UL, current additional co-existence requirement of -49dBm can be reused.

Proposal#5: For LAA DL only BS, spurious emission requirement for additional co-existence is in the range of [-48 to -47] dBm depending on UE REFSENSE

Proposal#6: For LAA BS with UL, spurious emission requirement for additional co-existence could be reused.

Similar as co-location requirement, certain excluded frequency range such as 10MHz could also be defined at the adjacent band edge.

  - Blocking requirement for the co-location

The blocking requirement for co-location depends on the maximum output power.

Blocking level = maximum output power of LAA BS – 30dB

For LAA DL only BS, the criterion should be “CCA clear detection” instead of 6dB sensitivity degradation. For LAA BS with UL, the criterion for blocking requirement for the co-location could reuse that for Medium Range, Local Area and Home BS.

Proposal#7 For LAA DL only BS, blocking requirement for the co-location should be define as the LAA BS doesn’t alarm channel being used after CCA while the interfering signal mean power “maximum output power of LAA BS – 30dB”.

Proposal#8: For LAA DL BS with UL, blocking emission requirement for the co-location should reuse the same wanted signal mean power for Medium Range, Local Area and Home BS, while the interfering signal mean power be “maximum output power of LAA BS – 30dB”.

- Conclusion

Obersvation#1: For LAA system, the CCA operation can be affected by adjacent channel interference even Rel-13 LAA only considers DL operation.

Obersvation#2: Two nearby LAA BSs operating in different channel may not have the possibility to transmit simultaneously if co-existence requirement is not sufficient.

Observation#3: LAA-LAA co-existence scenario is typical case in practical network.

Obersvation#4: Larger ACLR requirement can help reduce the minimal separation distance a lot.

Proposal#1: 45dB ACLR should be maintained for LAA BS.

Proposal#2:  Legacy ACS should be maintained for LAA BS.

Proposal#3: For LAA DL only BS, spurious emission requirement for the co-location is in the range of [-75 to -55] dBm depending on CCA threshold agreed in RAN1

Proposal#4: For LAA BS with UL, the spurious emission requirement for co-location could reuse the Medium Range, Local Area and Home BS requirement.

Proposal#5: For LAA DL only BS, spurious emission requirement for additional co-existence is in the range of [-48 to -47] dBm depending on UE REFSENSE.

Proposal#6: For LAA BS with UL, spurious emission requirement for additional co-existence could be reused.

Proposal#7 For LAA DL only BS, blocking requirement for the co-location should be define as the LAA BS doesn’t alarm channel being used after CCA while the interfering signal mean power “maximum output power of LAA BS – 30dB”.

Proposal#8: For LAA DL BS with UL, blocking emission requirement for the co-location should reuse the same wanted signal mean power for Medium Range, Local Area and Home BS, while the interfering signal mean power be “maximum output power of LAA BS – 30dB”

- Reference

[1] TR 36.889, Study on Licensed-Assisted Access to Unlicensed Spectrum