3GPP TSG RAN4 Meeting #77	R4-157563

Anaheim, USA, 16th – 20th Nov, 2015

Title: 	MSD for 2UL/3DL Inter-band CA

Source: 	Huawei, HiSilicon

Agenda item:	7.36.2

Document for:	Discussion

# 1	Introduction

This contribution provides our MSD analysis for CA_3A-7A-28A.

# 2	Discussion

Single antenna common diplexer based architecture is used as a reference to derive MSD for this CA combination. Furthermore a quadplexer was used to combine bands 3 and 7. Reference architecture illustrating main and diversity paths is shown in Figure 1 below.

Figure 1 Reference architecture

This combination is quite unique in the sense that each 2UL/2DL combination creates IMD on top the third DL.

| Case # | UL combination | IMD# | Victim DL |
| --- | --- | --- | --- |
| 1 | B3+B7 | IMD2 | B28 |
| 2 | B3+B28 | IMD3 | B7 |
| 3 | B7+B28 | IMD2 | B3 |

Table 1Intermodulation cases

We analyze only the total IMD levels in this contribution. Due to lack of time CF is not analyzed. The following parameters are used in this analysis:

Antenna switch IP2			  		112

Antenna switch IP3					68

Quadplexer IP2						113

Quadplexer IP3						74

B3 forward mixing IP2				27

B3 forward mixing IP3				30

B7 forward mixing IP2				27

B7 forward mixing IP3				30

B28 forward mixing IP2				27

B28 forward mixing IP3				30

B3 reverse mixing IP2				38

B3 reverse mixing IP3				28

B7 reverse mixing IP2				38

B7 reverse mixing IP3				28

B28 reverse mixing IP2				38

B28 reverse mixing IP3				28

B3 LNA IIP2							5

B7 LNA IIP2							5

B28 LNA IIP2						5

B3 LNA IIP3							-5

B7 LNA IIP3							-5

B28 LNA IIP3						-5

Antenna isolation 					10

PCB isolation PA-PA 				60

PCB isolation TX-RX				65

RFIC Pout B3/B7/B28				>0

Quadplexer cross-band isolation		50

Quadplexer attenuation at B28 RX   30

B28 Duplexer rejection at B3 RX     28

The MSD results are as follows

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B28 | B3 | IMD2 | fB7 - fB3 | 1747 | 5 | 25 | 796.0 | 5 |  | 36.4 |
|  | B7 |  |  | 2543 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB28 | 1712.5 | 5 | 25 | 2679.5 | 5 |  | 20.7 |
|  | B28 |  |  | 745.5 | 5 | 25 |  |  |  |  |
|  | B7 | IMD2 | fB7 - fB28 | 2543 | 5 | 25 | 1832.5 | 5 |  | 35.3 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |

The most dominant IMD mechanisms are:

B28 MSD (IMD2): B3/B7 PA forward mixing

-B3/B7 PA forward IMD at B28 RX frequency is attenuated only by 30dB by quadplexer and further 15dB by diplexer

B7 MSD (IMD3): B3 PA forward mixing

-B3 PA forward IMD at B7 RX frequency is attenuated by 50dB by quadplexer cross-band isolation

B3 MSD (IMD2): B28 PA forward mixing

-B28 forward IMD at B3 RX frequency is attenuated only by 28dB by B28 duplexer and further 15dB by diplexer

# 3	Conclusion

According to our analysis the MSD for CA_3A-7A-28A is as follows (not including CF which lowers the numbers a bit).

| DL CA | UL CA | IMD |  | UL Fc (MHz) | UL BW (MHz) | UL RB # | 3rd DL Fc (MHz) | DL BW (MHz) | CF (dB) | MSD (dB) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| B3+B7+B28 | B3 | IMD2 | fB7 - fB3 | 1747 | 5 | 25 | 796.0 | 5 |  | 36.4 |
|  | B7 |  |  | 2543 | 5 | 25 |  |  |  |  |
|  | B3 | IMD3 | 2*fB3 - fB28 | 1712.5 | 5 | 25 | 2679.5 | 5 |  | 20.7 |
|  | B28 |  |  | 745.5 | 5 | 25 |  |  |  |  |
|  | B7 | IMD2 | fB7 - fB28 | 2543 | 5 | 25 | 1832.5 | 5 |  | 35.3 |
|  | B28 |  |  | 710.5 | 5 | 25 |  |  |  |  |

# 6	References