3GPP TSG-RAN WG4 Meeting #78 	R4-160500

# St. Julian’s, Malta, 15 Feb-19 Feb, 2016

Source:	Ericsson

Title:	TP to 36.854-13, Co-existence analysis for CA Combinations with LAA band 46

Agenda item:				5.5

Document for:	Approval

# 1. Introduction

This is a text proposal for 36.854-13 for co-existence analysis for 4DL/1UL CA combinations with LAA operation, that is, CA between Band 2, Band 4 or Band 7 with band 46.

# 2. Text Proposal for TR 36.854-13

----- Unchanged sections omitted -----

### 6.x		LTE Advanced Carrier Aggregation of Band 2 and Band 46 (4DL/1 UL)

----- Unchanged sections omitted -----

##### 6.x.1.2	Co-existence studies for CA_2A-46D (4DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 2 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 43 and that 3rd harmonics falls into the band 46.

Table 6.x.1.2-1: Band 2 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1850 | 1910 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3700 to 3820 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5550 to 5730 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 2 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 2 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 1930 | 1990 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3860 to 3980 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5790 to 5970 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 3160 to 3995 |  | 7080 to 7915 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 1170 to 2065 9010 to 9905 |  | 8310 to 9920 12230 to 13840 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 1910 to 2010 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that none of 2rd harmonics fall into the any E-UTRA operating BS receive band, and it can be seen that 3rd harmonics fall into the E-UTRA operating BS receive band 46.

It can also be seen that two-tone 3rd order IMD products may fall into Band 22, 42 and 43,  and that the three-tone 3rd order IMD products may fall into Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 1, 2, 3, 4, 9, 10, 11, 21, 23, 24, 25, 33, 34, 35, 36, 37, 39, 65 and 66 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 2 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

----- Unchanged sections omitted -----

### 6.x	LTE Advanced Carrier Aggregation of Band 4 and Band 46 (4DL/1 UL)

----- Unchanged sections omitted -----

##### 6.x.1.2	Co-existence studies for CA_4A-46D (4DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 4 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 22 and 42, and that 3rd harmonics falls into band 46.

Table 6.x.1.2-1: Band 4 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 1710 | 1755 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 3420 to 3510 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 5130 to 5265 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 4 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 4 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2110 | 2155 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 4220 to 4310 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 6330 to 6465 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 2995 to 3815 |  | 7260 to 8080 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 840 to 1705 9370 to 10235 |  | 8145 to 9740 12410 to 14005 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2090 to 2175 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that none of the 2nd or 3rd harmonics fall into any of the E-UTRA operating BS receive bands.

It can also be ssen that two-tone 2nd order IMD products may fall into Band 22, 42 and 43, and that two-tone 3rd order IMD products may fall into Band 5, 8, 11, 19, 20, 21, 24 and 26. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 8, 11, 19, 20, 21, 22, 24, 26, 42 and 43 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 4 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

----- Unchanged sections omitted -----

### 6.x	LTE Advanced Carrier Aggregation of Band 7 and Band 46 (4DL/1 UL)

----- Unchanged sections omitted -----

##### 6.x.1.2	Co-existence studies for CA_7A-46D (4DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 7 + band 46 with 1UL. It can be seen that the 2nd or 3rd harmonics don’t fall into any E-UTRA operating band.

Table 6.x.1.2-1: Band 7 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2500 | 2570 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 5000 to 5140 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7500 to 7710 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 7 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 7 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2620 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 5240 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7860 to 8070 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 2460 to 3305 |  | 7770 to 8615 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 230 to 685 10390 to 11305 |  | 7610 to 9230 12920 to 14540 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2600 to 2710 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that the 2nd harmonics fall into Band 46 of the E-UTRA operating BS receive bands, and that none of the 3rd harmonics fall into anyof the E-UTRA operating BS receive bands

It can also be seen that two-tone 2nd order IMD products may fall into Band 7, 38 and 41, and that the two-tone 3rd order IMD products may fall into Band 31. The three tone 3rd order products may fall into Band 7, 38 and 41.

With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 7, 31, 38 and 41 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 7 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

----- Unchanged sections omitted -----

### 6.x	LTE Advanced Carrier Aggregation of Band 41 and Band 46 (4DL/1 UL)

----- Unchanged sections omitted -----

##### 6.x.1.2	Co-existence studies for CA_41A-46A (4DL/1UL)

Table 6.x.1.2-1 gives the UL to DL harmonic products for CA band 41 + band 46 with 1UL. It can be seen that 2nd harmonics fall into band 46.

Table 6.x.1.2-1: Band 41 and Band 46 1UL harmonic products

| UE UL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| UL frequency (MHz) | 2496 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 4992 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7488 to 8070 |  | 15450 to 17775 |  |

The 2nd and 3rd order DL to UL harmonics and IMD products caused in the BS by transmitting of band 41 and band 46 DL carriers is calculated and shown in table 6.x.1.2-2 below:

Table 6.x.1.2-2: Band 41 and Band 46 DL harmonics and IMD products

| BS DL carriers | f1_low | f1_high | f2_low | f2_high |
| --- | --- | --- | --- | --- |
| DL frequency (MHz) | 2496 | 2690 | 5150 | 5925 |
| 2nd order harmonics frequency range (MHz) | 4992 to 5380 |  | 10300 to 11850 |  |
| 3rd order harmonics frequency range (MHz) | 7488 to 8070 |  | 15450 to 17775 |  |
| Two-tone 2nd order IMD products | f2_low – f1_high | f2_high – f1_low | f2_low + f1_low | f2_high + f1_high |
| IMD frequency range (MHz) | 2460 to 3429 |  | 7646 to 8615 |  |
| Two-tone 3rd order IMD products | 2*f1_low – f2_high & (2*f1_low + f2_low) | 2*f1_high – f2_low & (2*f1_high + f2_high) | 2*f2_low – f1_high & (2*f2_low + f1_low) | 2* f2_high – f1_low & (2*f2_high + f1_high) |
| IMD frequency range (MHz) | 230 to 933 10142 to 11305 |  | 7610 to 9354 12796 to 14540 |  |
| Three-tone 3rd order IMD products | (f1_low – max BW f2) | (f1_high + max BW f2) | (f2_low – max BW f1) | (f2_high + max BW f1) |
| IMD frequency range (MHz) | 2476 to 2710 |  | 5130 to 5945 |  |

It can be seen from Table 6.x.1.2-2 that the 2rd harmonics fall into the E-UTRA operating BS receive band 46. It can also be seen none of the 3rd harmonics any E-UTRA operating BS receive bands.

It can also be seen that two-tone 3rd order IMD products may fall into Band 7, 22, 38, 41 and 42 and that the three-tone 3rd order IMD products may fall into Band 5, 6, 8, 12, 13, 14, 15, 16, 17, 18, 19, 20, 26, 27, 28, 29, 31, 32, 44, 45, 67 and 68. With the performances of the current BS antenna system, transmit and receive path components, amplifiers, pre-distortion algorithms and filters, it is expected that the harmonics and IMD interference generated within the Band 5, 6, 7, 8, 12, 13, 14, 15, 16, 17, 18, 19, 22, 20, 26, 27, 28, 29, 31, 32, 38, 41, 42, 44, 45, 67 and 68 receiver would be well below the receiver noise floor eliminating the possibility of receiver desensitization, provided that Band 41 and Band 46 BS transmitters do not share the same antenna with any of these receivers.

----- Unchanged sections omitted -----