3GPP TSG-RAN WG4 Meeting #81	R4-1610955

# Reno, Nevada, USA, 14th – 18th Nov 2016

Agenda item:	8.19.1.1

Source: 	Qualcomm Incorporated, LG Electronics

Title: 	Simulation Results for A-MPR of V2V Waveforms

Document for:	Discussion

# Introduction

In this paper, we propose simulation assumptions for A-MPR to meet additional Region 1 requirements.

# Simulation Assumption

## Basic simulation assumption and parameters

  - Channel BW : 10MHz

  - Modulation schemes : QPSK, 16QAM

  - Center frequency FC = 5860, 5870, 5880, 5890, 5900, 5910, 5920

  - Modulator impairments

    - I/Q imbalance	: 25 dBc

    - Carrier leakage: 25 dBc

    - Counter IM3 : 60dBc

## General Requirements

  - EVM

    - 17.5% for QPSK

    - 12.5% for 16QAM

  - ACLR

Table 1: ACLR for V2X UE

| Channel arrangement | Minimum channel spacing with 1 MHz Guard band |
| --- | --- |
| E-UTRAACLR for V2X UE | 30 dB |
| Adjacent channel centre1 frequency offset (in MHz) | +10 (20) / -10 (20) |
| E-UTRA channel1 Measurement bandwidth | 9 (18) MHz |

  - General SEM

Table 2: General SEM for V2X UE

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| ΔfOOB (MHz) | 1.4 MHz | 3.0 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Measurement bandwidth |
| 0-1 | -10 | -13 | -15 | -18 | -20 | -21 | 30 kHz |
| 1-2.5 | -10 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.5-2.8 | -25 | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 2.8-5 |  | -10 | -10 | -10 | -10 | -10 | 1 MHz |
| 5-6 |  | -25 | -13 | -13 | -13 | -13 | 1 MHz |
| 6-10 |  |  | -25 | -13 | -13 | -13 | 1 MHz |
| 10-15 |  |  |  | -25 | -13 | -13 | 1 MHz |
| 15-20 |  |  |  |  | -25 | -13 | 1 MHz |
| 20-25 |  |  |  |  |  | -25 | 1 MHz |

  - General SE requirement

Table 4: General SE for V2X UE

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz f < 12.75 GHz | -30 dBm | 1 MHz |  |
| 12.75 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz | 1 |
| 12.75 GHz - 26 GHz | -30 dBm | 1 MHz | 2 |
| NOTE 1: Applies for Band 22, Band 42 and Band 43. NOTE 2: Applies for Band 47. |  |  |  |

The A-MPR results are provided w/o MPR level, for each waveform.

## Region 1 Requirements

  - Antenna gain: 0dBi

  - Additional SEM requirement for 10MHz

Table 3: Additional SEM for V2X UE in region 1

| Spectrum emission limit (dBm)/ Channel bandwidth |  |  |
| --- | --- | --- |
| ΔfOOB (MHz) | 10 MHz | Measurement bandwidth |
| 0-0.5 | [-13-12∆fOOBMHz] | 100 kHz |
| 0.5-5 | [-19-169∆fOOBMHz-0.5] | 100 kHz |
| 5-10 | [-27-2∆fOOBMHz-5.0] | 100 kHz |
| NOTE: Additional SEM for V2X overrides any other requirements in frequency range 5855-5950MHz. |  |  |

  - Additional SE requirement (SE UE-to-UE coexistence)

Table 5: Additional SE for V2X UE in TR36.785 v1.0.0

| E-UTRA Band | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| … |  |  |  |  |  |  |  |
| 47 | E-UTRA Band 1, 3, 5, 7, 8, 22, 26, 28, 34, 39, 40, 41, 42, 44, 45, 65 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Frequency range | 5925 | - | 5950 | -30 | 1 | 1 |
|  | Frequency range | 5815 | - | 5855 | -30 | 1 | 1 |
| NOTE 1: Applicable when NS_XX is signalled by the network. NOTE 2: Applicable for an intended antenna installation>2m above the ground level. The EIRP requirement will be convert to conducted requirement. NOTE 3: Applicable for an intended antenna installation≤2m above the ground level. The EIRP requirement will be convert to conducted requirement. |  |  |  |  |  |  |  |

## V2X RB Allocations

### Adjacent

Resource pool that PSCCH and the corresponding PSSCH are in adjacent resource blocks is illustrated in Figure 1. The candidate size of a sub-channel is {5, 6, 10, 15, 20, 25, 50}.

Figure 1 PSCCH/PSSCH adjacent allocation scheme

NOTE1: following RAN1 specification to determine the number of RB for PSSCH.

if a pool is (pre)configured such that a UE always transmits PSCCH and the corresponding PSSCH in adjacent resource blocks in a subframe,  and  is the largest integer that fulfils

where  is a set of non-negative integers

NOTE 2: PSSCH is always at the lower part of each channel.

### Non Adjacent

Resource pool that PSCCH and the corresponding PSSCH are in non-adjacent resource blocks is illustrated in Figure 2. The candidate size of a sub-channel is {4,5,6,8,9,10,12,15,16,18,20,30,48,72,96. It is noted that in the simulation, it is assumed that PSCCH and PSSCH resource are not overlapped with each other.

Figure 2 PSCCH/PSSCH non-adjacent allocation scheme

NOTE1: following RAN1 specification to determine the number of RB for PSSCH.

if a pool is (pre)configured such that a UE may transmit PSCCH and the corresponding PSSCH in non-adjacent resource blocks in a subframe,  and , subjected to the constraint

LsubCH= 2α13α25α3

where  is a set of non-negative integers

NOTE 2: PSSCH is always at the lower part of each channel.