3GPP TSG-RAN WG4 Meeting #83	R4-1705241

Hangzhou, China, 15 - 19 May, 2017

Agenda item:	7.2.2

Source:	Dish Network

Title:	TP for TR 36.714-02-01: MSD for CA_66A-70A

Document for:	Approval

# 1	Introduction

This contribution provides a TP to capture MSD for CA_66A-70A.

# 2	Discussion

The frequency arrangements are shown in below.

B66: UL 1710-1780MHz, DL 2110-2200MHz

B70: UL 1695-1710MHz, DL 1995-2020MHz

The uplinks are adjacent with no gap, thus a triplexer instead of a quadplexer is used to combine these bands.

We contacted several filter vendors to get triplexer data. The data is collected in tables below. Please note that some of the data is in typical conditions (TYP) and some is worst case (WC).

Table 1 Isolation for CA_66A-70A

We note that one of the vendors provided information for both conditions, one provided isolation for typical only, and one provided information for worst case only. We have calculated averages for typical conditions and worst case conditions separately. These averages are not directly comparable with each other because they use different data mixes.

TX isolation needs to be above 50dB to avoid MSD for this band while 50dB RX isolation is sufficient. Both the typical and worst case averages show all isolation values are at least 52.5dB. According to our analysis this should be sufficient to warrant no MSD is needed for B66, B70, and B66+B70 CA.

Conclusion: No MSD requirement is needed for B66, B70, and B66+B70 CA

# 3	TP

## ----- Start of Text proposal -----

### 6.52.5	Reference sensitivity

Isolation values are collected in table below.

Table 6.52.5-1 Isolation for CA_66A-70A

TX isolation needs to be >50dB to avoid MSD for this band while 50dB RX isolation is sufficient. Both the typical and worst case averages show all isolation values are at least 52.5dB. According to our analysis this should be sufficient to warrant no MSD is needed for B66, B70, and B66+B70 CA.

----- End of Text proposal -----

# References

- RP-160945, “Revised WID: LTE Advanced inter-band CA Rel-14 for 2DL/1UL”, Qualcomm, 3GPP RAN#81, Reno, Nevada, USA, November 2016

[2]  R4-174092, “TP for TR 36.714-02-01:  ΔTIB,c  ΔRIB,c, and MSD for CA_66A-70A”, DISH Network, 3GPP  RAN4#82bis, Spokane, USA, April 2017