3GPP TSG-RAN WG4 Meeting #84                                      R4-1707099

Berlin, Germany, 21st – 25th August, 2017

Agenda item:			7.1.2

Source:			Samsung

Title:	LTE BHH TRP TRS Measurement Results

Document for:			Discussion

# 1	Introduction

In RAN4 #83 (Hangzhou) meeting, the LTE TRP/TRS work item was making progress by approving the detailed framework to derive LTE BHH TRP/TRS requirement [1]. With the extension of this WI in RAN plenary, in this paper, we will provide our measurement results to further complete RAN4 TRP/TRS data pool [2]. Particularly, we would like to provide DUT’s measurement data from as many bands as possible, in order to achieve a more representative RAN4 data pool.

# Discussion

For this submission of data, 14 commercially available LTE handsets were measured, which is marked by DUT 1 through 14. The tables below provide the measured TRP and TRS results of the LTE handsets for the BHH test case in Bands 1, 2, 3, 4, 5, 7, 8, 12, 13, 17, 19, 20, 21, 28, 30 and 41. Dashes in the tables indicate that the device does not support the given band.

Table 1: LTE handset TRP/TRS measurements for the BHH test case in Band 1

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 12.2 | 11.6 | -91.9 | -90.2 |
| 002 | 14.1 | 13.4 | -89.9 | -89.9 |
| 003 | 14.3 | 12.5 | -90.2 | -89.0 |
| 004 | 13.5 | 11.3 | -90.8 | -89.6 |
| 005 | 14.7 | 13.3 | -90.3 | -89.3 |
| 006 | 14.7 | 14.0 | -92.6 | -91.6 |
| 007 | 15.4 | 14.9 | -90.3 | -89.6 |
| 008 | 13.7 | 12.4 | -89.7 | -89.6 |
| 009 | 14.0 | 12.7 | -93.4 | -93.2 |
| 010 | 14.3 | 12.6 | -91.5 | -91.0 |
| 011 | - | - | - | - |
| 012 | 12.7 | 11.4 | -91.7 | -90.8 |
| 013 | 15.3 | 13.6 | -89.2 | -87.8 |
| 014 | 15.0 | 14.9 | -90.7 | -89.7 |

Table 2: LTE handset TRP/TRS measurements for the BHH test case in Band 2

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | 14.5 | 13.3 | -88.6 | -85.5 |
| 004 | 14.6 | 12.8 | -90.9 | -89.3 |
| 005 | 13.8 | 12.7 | -91.5 | -90.6 |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | 14.0 | 12.8 | -89.0 | -88.0 |
| 009 | 13.3 | 12.5 | -91.4 | -91.0 |
| 010 | 13.8 | 12.5 | -91.0 | -90.2 |
| 011 | 14.3 | 14.0 | -93.0 | -92.4 |
| 012 | 13.7 | 12.2 | -91.6 | -91.2 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 3: LTE handset TRP/TRS measurements for the BHH test case in Band 3

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 11.9 | 11.7 | -91.8 | -90.9 |
| 002 | 13.9 | 13.6 | -90.7 | -90.6 |
| 003 | 14.3 | 13.3 | -91.0 | -89.7 |
| 004 | 13.8 | 12.4 | -89.9 | -87.8 |
| 005 | 13.2 | 11.8 | -91.2 | -91.0 |
| 006 | 13.3 | 12.7 | -91.5 | -91.1 |
| 007 | 13.7 | 13.3 | -91.1 | -91.0 |
| 008 | 14.7 | 12.7 | -89.6 | -88.0 |
| 009 | 13.7 | 13.7 | -93.3 | -92.9 |
| 010 | 14.3 | 13.8 | -92.1 | -91.3 |
| 011 | 12.0 | 10.9 | -88.3 | -88.1 |
| 012 | 13.0 | 12.3 | -92.1 | -91.5 |
| 013 | 13.6 | 13.0 | -91.6 | -90.4 |
| 014 | 14.2 | 13.8 | -90.7 | -90.5 |

Table 4: LTE handset TRP/TRS measurements for the BHH test case in Band 4

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | 12.4 | 11.1 | -88.5 | -87.7 |
| 005 | 12.7 | 11.1 | -88.1 | -87.8 |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | 13.8 | 11.8 | -88.9 | -88.9 |
| 009 | 13.0 | 13.0 | -93.0 | -92.8 |
| 010 | 13.7 | 13.3 | -90.9 | -90.6 |
| 011 | 13.4 | 12.6 | -93.1 | -93.0 |
| 012 | 14.2 | 12.4 | -91.5 | -91.4 |
| 013 | 10.3 | 8.5 | -90.2 | -90.0 |
| 014 | - | - | - | - |

Table 5: LTE handset TRP/TRS measurements for the BHH test case in Band 5

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 9.3 | 8.9 | -86.9 | -86.3 |
| 002 | 9.6 | 8.6 | -88.1 | -87.4 |
| 003 | 12.1 | 11.3 | -87.0 | -86.3 |
| 004 | 9.9 | 9.8 | -86.4 | -85.9 |
| 005 | 7.3 | 5.3 | -88.0 | -87.3 |
| 006 | 13.8 | 13.3 | -87.6 | -86.8 |
| 007 | 12.3 | 11.8 | -84.3 | -83.9 |
| 008 | 8.0 | 6.1 | -86.2 | -85.1 |
| 009 | 12.2 | 12.1 | -88.6 | -88.5 |
| 010 | 11.4 | 11.0 | -88.1 | -88.0 |
| 011 | 10.7 | 10.1 | -86.8 | -86.6 |
| 012 | 11.4 | 11.2 | -87.7 | -87.7 |
| 013 | 10.2 | 9.0 | -89.3 | -88.6 |
| 014 | 12.1 | 11.8 | -86.3 | -86.3 |

Table 6: LTE handset TRP/TRS measurements for the BHH test case in Band 7

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 13.7 | 12.4 | -91.1 | -90.7 |
| 002 | 15.2 | 14.9 | -93.0 | -92.3 |
| 003 | 14.9 | 14.8 | -90.1 | -88.7 |
| 004 | 16.1 | 15.6 | -91.8 | -90.7 |
| 005 | 11.9 | 11.1 | -87.9 | -86.9 |
| 006 | 14.4 | 12.7 | -89.9 | -88.5 |
| 007 | 15.5 | 14.6 | -92.0 | -90.5 |
| 008 | 16.1 | 14.8 | -91.6 | -91.4 |
| 009 | 15.8 | 14.8 | -91.3 | -89.6 |
| 010 | 15.4 | 14.8 | -92.9 | -92.0 |
| 011 | 17.1 | 17.0 | -92.1 | -90.6 |
| 012 | 14.4 | 14.3 | -91.5 | -90.8 |
| 013 | 13.1 | 12.0 | -90.7 | -89.4 |
| 014 | 15.0 | 14.9 | -90.9 | -90.6 |

Table 7: LTE handset TRP/TRS measurements for the BHH test case in Band 8

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 10.2 | 9.8 | -86.7 | -86.6 |
| 002 | 9.7 | 8.6 | -89.5 | -86.1 |
| 003 | 11.7 | 11.5 | -87.7 | -86.1 |
| 004 | 10.0 | 9.9 | -87.4 | -87.3 |
| 005 | 8.0 | 5.1 | -88.2 | -87.2 |
| 006 | 12.8 | 12.3 | -87.6 | -86.5 |
| 007 | 11.3 | 11.0 | -83.8 | -83.0 |
| 008 | 7.0 | 4.6 | -87.7 | -85.2 |
| 009 | 11.5 | 10.7 | -88.1 | -88.0 |
| 010 | 11.3 | 11.0 | -88.5 | -88.3 |
| 011 | 8.8 | 8.1 | -85.3 | -84.4 |
| 012 | 9.5 | 9.4 | -87.1 | -86.2 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 8: LTE handset TRP/TRS measurements for the BHH test case in Band 12

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | 9.5 | 9.2 | -88.0 | -87.6 |
| 005 | - | - | - | - |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | 2.0 | 0.9 | -89.1 | -88.2 |
| 009 | 9.3 | 8.9 | -90.5 | -90.4 |
| 010 | 4.9 | 4.4 | -87.5 | -86.8 |
| 011 | - | - | - | - |
| 012 | 10.1 | 9.7 | -88.9 | -87.8 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 9: LTE handset TRP/TRS measurements for the BHH test case in Band 13

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | - | - | - | - |
| 005 | - | - | - | - |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | 9.1 | 8.3 | -88.4 | -88.2 |
| 010 | 8.7 | 8.0 | -88.1 | -88.1 |
| 011 | 11.5 | 11.4 | -87.6 | -87.1 |
| 012 | 11.0 | 10.8 | -88.1 | -87.9 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 10: LTE handset TRP/TRS measurements for the BHH test case in Band 17

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | 9.4 | 9.1 | -85.6 | -85.3 |
| 005 | 8.8 | 8.7 | -84.2 | -84.0 |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | 2.1 | 0.6 | -85.7 | -84.8 |
| 009 | 9.4 | 9.2 | -87.9 | -87.7 |
| 010 | 4.7 | 4.0 | -85.3 | -84.6 |
| 011 | - | - | - | - |
| 012 | 10.2 | 10.0 | -87.7 | -87.4 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 11: LTE handset TRP/TRS measurements for the BHH test case in Band 19

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | - | - | - | - |
| 005 | - | - | - | - |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | - | - | - | - |
| 010 | - | - | - | - |
| 011 | - | - | - | - |
| 012 | - | - | - | - |
| 013 | 11.8 | 10.8 | -91.4 | -91.0 |
| 014 | 12.5 | 12.3 | -88.4 | -88.3 |

Table 12: LTE handset TRP/TRS measurements for the BHH test case in Band 20

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | 10.7 | 10.1 | -86.3 | -84.3 |
| 002 | 11.8 | 10.3 | -87.7 | -85.8 |
| 003 | 12.4 | 11.7 | -89.0 | -88.7 |
| 004 | 10.2 | 10.1 | -87.6 | -87.3 |
| 005 | 7.7 | 5.3 | -85.9 | -85.3 |
| 006 | 13.3 | 12.9 | -89.2 | -89.0 |
| 007 | 11.5 | 11.3 | -88.4 | -87.3 |
| 008 | 9.5 | 7.5 | -87.0 | -86.1 |
| 009 | 12.7 | 12.3 | -88.8 | -88.4 |
| 010 | 11.8 | 11.5 | -88.3 | -88.1 |
| 011 | 9.6 | 8.9 | -86.5 | -86.2 |
| 012 | 10.1 | 9.7 | -85.2 | -84.3 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 13: LTE handset TRP/TRS measurements for the BHH test case in Band 21

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | - | - | - | - |
| 005 | - | - | - | - |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | - | - | - | - |
| 010 | - | - | - | - |
| 011 | - | - | - | - |
| 012 | - | - | - | - |
| 013 | 12.2 | 11.4 | -87.3 | -86.7 |
| 014 | 13.5 | 13.4 | -88.3 | -88.1 |

Table 14: LTE handset TRP/TRS measurements for the BHH test case in Band 28

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | 7.4 | 6.6 | -83.8 | -83.1 |
| 005 | 8.2 | 7.6 | -85.3 | -84.7 |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | 9.0 | 8.5 | -87.9 | -87.8 |
| 010 | 9.7 | 9.5 | -88.2 | -88.1 |
| 011 | - | - | - | - |
| 012 | 7.8 | 7.7 | -87.1 | -87.1 |
| 013 | 9.1 | 7.7 | -88.0 | -87.5 |
| 014 | 8.3 | 7.6 | -83.1 | -81.8 |

Table 15: LTE handset TRP/TRS measurements for the BHH test case in Band 30

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | - | - | - | - |
| 005 | - | - | - | - |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | - | - | - | - |
| 010 | - | - | - | - |
| 011 | - | - | - | - |
| 012 | 14.1 | 13.3 | -90.0 | -89.0 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Table 16: LTE handset TRP/TRS measurements for the BHH test case in Band 41

| DUT | TRP avg | TRP min | TRS avg | TRS max |
| --- | --- | --- | --- | --- |
| 001 | - | - | - | - |
| 002 | - | - | - | - |
| 003 | - | - | - | - |
| 004 | - | - | - | - |
| 005 | 10.7 | 8.7 | -84.0 | -82.6 |
| 006 | - | - | - | - |
| 007 | - | - | - | - |
| 008 | - | - | - | - |
| 009 | 14.6 | 13.7 | -89.3 | -88.4 |
| 010 | 15.7 | 15.4 | -89.3 | -88.7 |
| 011 | 19.1 | 18.2 | -88.6 | -87.8 |
| 012 | 14.4 | 14.3 | -87.7 | -86.9 |
| 013 | - | - | - | - |
| 014 | - | - | - | - |

Next, in the following Table 17, we provide the CA support and number of supported bands information for each DUT.

Table 17: CA support and number of supported bands information

| DUT | CA support? | Num. bands |
| --- | --- | --- |
| 001 | NO | 15 |
| 002 | YES | 15 |
| 003 | YES | 16 |
| 004 | YES | 16 |
| 005 | NO | 16 |
| 006 | NO | 14 |
| 007 | NO | 13 |
| 008 | YES | 21 |
| 009 | YES | 31 |
| 010 | YES | 33 |
| 011 | YES | 34 |
| 012 | YES | 39 |
| 013 | YES | 23 |
| 014 | YES | 22 |

# Conclusion

In this paper, we would like to provide our measurement results to further complete RAN4 TRP/TRS data pool, for further alignment on LTE handset TRP/TRS requirements.

# Reference

[1] R4-1706096, “Proposals on completion of OTA TRP/TRS WI”, Telecom Italia, Intel, OPPO, Xiaomi, Sony Mobile, Coolpad, 3GPP RAN4#83, May 2017

[2] R4-1706093, “TRP/TRS data pool and joint band passing rate worksheet”, Intel, 3GPP RAN4#83, May 2017