3GPP TSG-RAN4 Meeting #84	R4-1707337

# Berlin, Germany, 21st Aug 2017 - 25th Aug 2017

| CR-Form-v11.2.1 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.101 | CR | 4521 | rev | - | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on band definition and duplex mode of sidelink operation in band 47. |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Qualcomm Inc. |  |  |  |  |  |  |  |  |  |
| Source to TSG: |  |  |  |  |  |  |  |  |  |  |
| Work item code: | LTE_SL_V2V-Core |  |  |  |  |  | Date: |  |  | 2017-08-09 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | - Current band 47 has to use Frame Structure 2 which does not allow for 100% DL allocation despite there is no TDD network deployment in this band. - V2X Sidelink comminucation duplex mode is TDD, which is incorrect. |  |  |  |  |  |  |  |  |
| Summary of change: |  | - Add a note to allow band 47 to use Frame Struture 1, which allows for 100% DL allocation. - Fix V2X Sidelink comminucation duplex mode to HD. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | - V2X communication cannot use all spectrum avaliable in band 47, which leads to porr spectral effecency. - V2X Sidelink comminucation duplex mode is incorrect. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 5.5, 5.5G, 7.3.1G |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  | X |  | Test specifications |  |  |  | TS 36.521-1 |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Begin of Change 1>

## 5.5	Operating bands

E-UTRA is designed to operate in the operating bands defined in Table 5.5-1.

Table 5.5-1 E-UTRA operating bands

| E-UTRA Operating Band |  | Uplink (UL) operating band BS receive UE transmit |  |  |  |  |  | Downlink (DL) operating band BS transmit UE receive |  |  |  |  |  | Duplex Mode |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  |  |  |  | FDL_low – FDL_high |  |  |  |  |  |  |  |
| 1 |  | 1920 MHz |  | – |  | 1980 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | FDD |  |
| 2 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | FDD |  |
| 3 |  | 1710 MHz |  | – |  | 1785 MHz |  | 1805 MHz |  | – |  | 1880 MHz |  | FDD |  |
| 4 |  | 1710 MHz |  | – |  | 1755 MHz |  | 2110 MHz |  | – |  | 2155 MHz |  | FDD |  |
| 5 |  | 824 MHz |  | – |  | 849 MHz |  | 869 MHz |  | – |  | 894MHz |  | FDD |  |
| 61 |  | 830 MHz |  | – |  | 840 MHz |  | 875 MHz |  | – |  | 885 MHz |  | FDD |  |
| 7 |  | 2500 MHz |  | – |  | 2570 MHz |  | 2620 MHz |  | – |  | 2690 MHz |  | FDD |  |
| 8 |  | 880 MHz |  | – |  | 915 MHz |  | 925 MHz |  | – |  | 960 MHz |  | FDD |  |
| 9 |  | 1749.9 MHz |  | – |  | 1784.9 MHz |  | 1844.9 MHz |  | – |  | 1879.9 MHz |  | FDD |  |
| 10 |  | 1710 MHz |  | – |  | 1770 MHz |  | 2110 MHz |  | – |  | 2170 MHz |  | FDD |  |
| 11 |  | 1427.9 MHz |  | – |  | 1447.9 MHz |  | 1475.9 MHz |  | – |  | 1495.9 MHz |  | FDD |  |
| 12 |  | 699 MHz |  | – |  | 716 MHz |  | 729 MHz |  | – |  | 746 MHz |  | FDD |  |
| 13 |  | 777 MHz |  | – |  | 787 MHz |  | 746 MHz |  | – |  | 756 MHz |  | FDD |  |
| 14 |  | 788 MHz |  | – |  | 798 MHz |  | 758 MHz |  | – |  | 768 MHz |  | FDD |  |
| 15 |  | Reserved |  |  |  |  |  | Reserved |  |  |  |  |  | FDD |  |
| 16 |  | Reserved |  |  |  |  |  | Reserved |  |  |  |  |  | FDD |  |
| 17 |  | 704 MHz |  | – |  | 716 MHz |  | 734 MHz |  | – |  | 746 MHz |  | FDD |  |
| 18 |  | 815 MHz |  | – |  | 830 MHz |  | 860 MHz |  | – |  | 875 MHz |  | FDD |  |
| 19 |  | 830 MHz |  | – |  | 845 MHz |  | 875 MHz |  | – |  | 890 MHz |  | FDD |  |
| 20 |  | 832 MHz |  | – |  | 862 MHz |  | 791 MHz |  | – |  | 821 MHz |  | FDD |  |
| 21 |  | 1447.9 MHz |  | – |  | 1462.9 MHz |  | 1495.9 MHz |  | – |  | 1510.9 MHz |  | FDD |  |
| 22 |  | 3410 MHz |  | – |  | 3490 MHz |  | 3510 MHz |  | – |  | 3590 MHz |  | FDD |  |
| 231 |  | 2000 MHz |  | – |  | 2020 MHz |  | 2180 MHz |  | – |  | 2200 MHz |  | FDD |  |
| 24 |  | 1626.5 MHz |  | – |  | 1660.5 MHz |  | 1525 MHz |  | – |  | 1559 MHz |  | FDD |  |
| 25 |  | 1850 MHz |  | – |  | 1915 MHz |  | 1930 MHz |  | – |  | 1995 MHz |  | FDD |  |
| 26 |  | 814 MHz |  | – |  | 849 MHz |  | 859 MHz |  | – |  | 894 MHz |  | FDD |  |
| 27 |  | 807 MHz |  | – |  | 824 MHz |  | 852 MHz |  | – |  | 869 MHz |  | FDD |  |
| 28 |  | 703 MHz |  | – |  | 748 MHz |  | 758 MHz |  | – |  | 803 MHz |  | FDD |  |
| 29 |  | N/A |  |  |  |  |  | 717 MHz |  | – |  | 728 MHz |  | FDD2 |  |
| 30 |  | 2305 MHz |  | – |  | 2315 MHz |  | 2350 MHz |  | – |  | 2360 MHz |  | FDD |  |
| 31 |  | 452.5 MHz |  | – |  | 457.5 MHz |  | 462.5 MHz |  | – |  | 467.5 MHz |  | FDD |  |
| 32 |  |  |  | N/A |  |  |  | 1452 MHz |  | – |  | 1496 MHz |  | FDD2 |  |
| 33 |  | 1900 MHz |  | – |  | 1920 MHz |  | 1900 MHz |  | – |  | 1920 MHz |  | TDD |  |
| 34 |  | 2010 MHz |  | – |  | 2025 MHz |  | 2010 MHz |  | – |  | 2025 MHz |  | TDD |  |
| 35 |  | 1850 MHz |  | – |  | 1910 MHz |  | 1850 MHz |  | – |  | 1910 MHz |  | TDD |  |
| 36 |  | 1930 MHz |  | – |  | 1990 MHz |  | 1930 MHz |  | – |  | 1990 MHz |  | TDD |  |
| 37 |  | 1910 MHz |  | – |  | 1930 MHz |  | 1910 MHz |  | – |  | 1930 MHz |  | TDD |  |
| 38 |  | 2570 MHz |  | – |  | 2620 MHz |  | 2570 MHz |  | – |  | 2620 MHz |  | TDD |  |
| 39 |  | 1880 MHz |  | – |  | 1920 MHz |  | 1880 MHz |  | – |  | 1920 MHz |  | TDD |  |
| 40 |  | 2300 MHz |  | – |  | 2400 MHz |  | 2300 MHz |  | – |  | 2400 MHz |  | TDD |  |
| 41 |  | 2496 MHz |  |  |  | 2690 MHz |  | 2496 MHz |  |  |  | 2690 MHz |  | TDD |  |
| 42 |  | 3400 MHz |  | – |  | 3600 MHz |  | 3400 MHz |  | – |  | 3600 MHz |  | TDD |  |
| 43 |  | 3600 MHz |  | – |  | 3800 MHz |  | 3600 MHz |  | – |  | 3800 MHz |  | TDD |  |
| 44 |  | 703 MHz |  | – |  | 803 MHz |  | 703 MHz |  | – |  | 803 MHz |  | TDD |  |
| 45 |  | 1447 MHz |  | – |  | 1467 MHz |  | 1447 MHz |  | – |  | 1467 MHz |  | TDD |  |
| 46 |  | 5150 MHz |  | – |  | 5925 MHz |  | 5150 MHz |  | – |  | 5925 MHz |  | TDD8 |  |
| 47 |  | 5855 MHz |  | – |  | 5925 MHz |  | 5855 MHz |  | – |  | 5925 MHz |  | TDD11 |  |
| 48 |  | 3550 MHz |  | – |  | 3700 MHz |  | 3550 MHz |  | – |  | 3700 MHz |  | TDD |  |
| … |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 64 |  | Reserved |  |  |  |  |  |  |  |  |  |  |  |  |  |
| 65 |  | 1920 MHz |  | – |  | 2010 MHz |  | 2110 MHz |  | – |  | 2200 MHz |  | FDD |  |
| 66 |  | 1710 MHz |  | – |  | 1780 MHz |  | 2110 MHz |  | – |  | 2200 MHz |  | FDD4 |  |
| 67 |  |  |  | N/A |  |  |  | 738 MHz |  | – |  | 758 MHz |  | FDD2 |  |
| 68 |  | 698 MHz |  | – |  | 728 MHz |  | 753 MHz |  | – |  | 783 MHz |  | FDD |  |
| 69 |  | N/A |  |  |  |  |  | 2570 MHz |  | – |  | 2620 MHz |  | FDD2 |  |
| 70 |  | 1695 MHz |  | – |  | 1710 MHz |  | 1995 MHz |  | – |  | 2020 MHz |  | FDD10 |  |
| NOTE 1: Band 6, 23 is not applicable NOTE 2: Restricted to E-UTRA operation when carrier aggregation is configured. The downlink operating band is paired with the uplink operating band (external) of the carrier aggregation configuration that is supporting the configured Pcell. NOTE 3: A UE that complies with the E-UTRA Band 65 minimum requirements in this specification shall also comply with the E-UTRA Band 1 minimum requirements. NOTE 4: The range 2180-2200 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured. NOTE 5: A UE that supports E-UTRA Band 66 shall receive in the entire DL operating band NOTE 6: A UE that supports E-UTRA Band 66 and CA operation in any CA band shall also comply with the minimum requirements specified for the DL CA configurations CA_66B, CA_66C and CA_66A-66A. NOTE 7: A UE that complies with the E-UTRA Band 66 minimum requirements in this specification shall also comply with the E-UTRA Band 4 minimum requirements. NOTE 8: This band is an unlicensed band restricted to licensed-assisted operation using Frame Structure Type 3 NOTE 9: In this version of the specification, restricted to E-UTRA DL operation when carrier aggregation is configured. NOTE 10: The range 2010-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 300 MHz The range 2005-2020 MHz of the DL operating band is restricted to E-UTRA operation when carrier aggregation is configured and TX-RX separation is 295 MHz. NOTE 11: This band is unlicensed band used for V2X communication. There is no expected network deployment in this band so Frame Structure Type 1 can be used. If there is network deployed in this band, only Frame Structure 2 shall be used. |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |

<End of Change 1>

<Begin of Change 2>

## 5.5G	Operating bands for V2X Communication

E-UTRA V2X Communication is designed to operate in the the operating bands defined in Table 5.5G-1.

Table 5.5G-1 V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
|  | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | HD | PC5 |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD | Uu |
| 7 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD | Uu |
| 8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD | Uu |
| 39 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD | Uu |
| 41 | 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD | Uu |

E-UTRA V2X communication is designed to operate concurrent with E-UTRA uplink/downlink on the operating bands combinations listed in Table 5.5G-2.

Table 5.5G-2 Inter-band con-current V2X operating bands

| V2X con-current band configuration | E-UTRA or V2X Operating Band | Interface | Uplink (UL) operating band BS receive UE transmit |  |  | Downlink (DL) operating band BS transmit UE receive |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| V2X_3-47 | 3 | Uu | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_7-47 | 7 | Uu | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_8-47 | 8 | Uu | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_39-47 | 39 | Uu | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |
| V2X_41-47 | 41 | Uu | 2496 MHz | – | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD |
|  | 47 | PC5 | 5855 MHz | – | 5925 MHz | 5855 MHz |  | 5925 MHz | HD |

E-UTRA V2X communication is also designed to operate for intra-band multi-carrier operation in the operating bands defined in Table 5.5G-3.

Table 5.5G-3: V2X intra-band multi-carrier operation

| V2X multi-carrier Band configuration | V2X operating Band | Interface |
| --- | --- | --- |
| V2X_47 | 47 | PC5 |

<End of Change 2>

<Begin of Change 3>

### 7.3.1G	Minimum requirements (QPSK) for V2X

When UE is configured for E-UTRA V2X reception non-concurrent with E-UTRA uplink transmissions for E-UTRA V2X operating bands specified in Table 5.5G-1, the throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8.2 with parameters specified in Table 7.3.1G-1.

Table 7.3.1G-1: Reference sensitivity

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode | Interface |
| 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |
| NOTE 1: Reference measurement channel is defined in A.8.2. NOTE 2: The signal power is specified per port. |  |  |  |  |  |  |  |  |

When UE is configured for E-UTRA V2X reception on V2X carrier con-current with E-UTRA uplink for inter-band E-UTRA V2X / E-UTRA bands specified in Table 5.5G-2, E-UTRA V2X sidelink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.8.2 with parameters specified in Table 7.3.1G-2. Also the E-UTRAdownlink throughput shall be ≥ 95% of the maximum throughput of the reference measurement channels as specified in Annexes A.3.3.2.

For the UE which supports V2X in an operating band as specified in Table 5.5G-2, and the UE also supports a E-UTRA downlink inter-band con-current configuration in Table 7.3.1G-2A, the minimum requirement for reference sensitivity in Table 7.3.1G-1 and Table 7.3.1G-2 shall be increased by the amount given in ΔRIB,c in Table 7.3.1G-2A for the corresponding E-UTRA V2X band.

Table 7.3.1G-2: Reference sensitivity for V2X Communication QPSK PREFSENS

| Inter-band V2X reception |  | Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X Band | E-UTRA or V2X band | E-UTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex Mode | Interface |
| Band 47 | Band 3 | 3 | -101.7 | -98.7 | -97 | -94 | -92.2 | -91 | FDD | Uu |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |
| Band 47 | Band 7 | 7 |  |  | -98 | -95 | -93.2 | -92 | FDD | Uu |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |
| Band 47 | Band 8 | 8 | -102.2 | -99.2 | -97 | -94 |  |  | FDD | Uu |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |
| Band 47 | Band 39 | 39 |  |  | -100 | -97 | -95.2 | -94 | TDD | Uu |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |
| Band 47 | Band 41 | 41 |  |  | -98 | -95 | -93.2 | -92 | TDD | Uu |
|  |  | 47 |  |  |  | -90.4 |  | -87.5 | HD | PC5 |

Table 7.3.1G-2A: ΔRIB,c (two bands)

| V2X inter-band con-current band Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| V2X_3A-47A | 3 | 0 |
| V2X_7A-47A | 7 | 0 |
| V2X_8A-47A | 8 | 0 |
| V2X_39A-47A | 39 | 0 |
| V2X_41A-47A | 41 | 0 |

The reference sensitivity is defined to be met with E-UTRA uplink assigned to one band (that differs from the V2X operating band) and all E-UTRA downlink carriers active. The E-UTRA uplink resource blocks shall be located as close as possible to E-UTRA V2X operating band but confined within the transmission bandwidth configuration for the channel bandwidth (Table 5.6-1). The uplink configuration for the E-UTRA operating band is specified in Table 7.3.1G-3 and 7.3.1G-4.

Table 7.3.1G-3: Uplink configuration for REFSENS of E-UTRA V2X Bands (PC5)

| Inter-band E-UTRA V2X /E-UTRA configuration |  | E-UTRA UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X band (PC5) | E-UTRA or V2X operating band (Uu) | E-UTRA or V2X UL band (Uu) | Channel Bandwidth (MHz) | NRB | Duplex Mode |  |
| 47 | 3 | 3 | 10 | 50 | HD | FDD |
| 47 | 7 | 7 | 10 | 50 | HD | FDD |
| 47 | 8 | 8 | 10 | 50 | HD | FDD |
| 47 | 39 | 39 | 10 | 50 | HD | TDD |
| 47 | 41 | 41 | 10 | 50 | HD | TDD |

Table 7.3.1G-4: Uplink configuration for REFSENS of E-UTRA V2X Bands (Uu)

| Inter-band E-UTRA V2X /E-UTRA configuration |  | E-UTRA UL band / Channel BW / NRB / Duplex mode |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
| E-UTRA V2X band (PC5) | E-UTRA or V2X operating band (Uu) | E-UTRA or V2X band (PC5) | Channel Bandwidth (MHz) | NRB | Duplex Mode |  |
| 47 | 3 | 47 | 10 | 50 | HD | FDD |
| 47 | 7 | 47 | 10 | 50 | HD | FDD |
| 47 | 8 | 47 | 10 | 50 | HD | FDD |
| 47 | 39 | 47 | 10 | 50 | HD | TDD |
| 47 | 41 | 47 | 10 | 50 | HD | TDD |

For intra-band contiguous multi-carrier operation, the above reference sensitivity requirement shall apply for each component carrier with both carriers active. The requirement is applied for 2 carrier intra-concurrent recpetions when 2 carrier transmission are activated at the same time.

<End of Change 3>