3GPP TSG-RAN WG4 Meeting #84	R4-1707679

Berlin, Germany, 21 – 25 August 2017

Agenda Item:	8.25.3.2

Source:	Ericsson

Title:	Simulation assumption for eFeMTC SI acquisition time improvement

Document for:	Discussion

# 1	Introduction

This contribution proposes the simulation assumption for eFeMTC SI acquisition time improvement.

# 2	Simulation assumption

Table 1 is the simulation assumption for SIB1-BR acquisition. Table 2 is the simulation assumption for SIB2-BR acquisition. We propose to use this assumption for the study of system acquisition time improvement for eFeMTC WI.

Table 1	Parameter for SIB1-BR acquisition time evaluation.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channels | ETU1, EPA1 |
| TBS for SIB1-BR | 208bits |
| SIB1-BR repetition number within 80ms | 16 |
| SIB1-BR accumulation period | 80ms, 160ms, 320ms, 640ms |
| UE receive antennas | 1 |
| UE frequency offset | 100Hz |
| Test point (SNR) | -15dB |
| Evaluation criteria | 99% of SIB1-BR decoding success rate |

Table 2	Parameter for SIB2-BR acquisition time evaluation.

| Parameters | CE Mode B |
| --- | --- |
| System bandwidth | 10MHz |
| Transmit antennas | 2 |
| Propagation channels | ETU1, EPA1 |
| TBS for SIB1-BR | 208bits |
| si-WindowLength-BR | 40ms |
| si-RepetitionPattern | everyRF |
| si-Periodicity | 160ms (rf16) |
| SIB2-BR accumulation period | 160ms, 320ms, 640ms, 1280ms |
| UE receive antennas | 1 |
| UE frequency offset | 100Hz |
| Target SNR | -15dB |

# 3	Reference

R4-1707678, “SI acquisition time improvement for eFeMTC”, Ericsson.