3GPP TSG-WG RAN4 Meeting #84 	R4-1707684

# Berlin, Germany, 21st – 25th Aug, 2017

Source: 	Nokia, Nokia Shanghai Bell, Verizon

Title: 					TP to TR 36.715-03-01: CA_3DL_2A-13A-46A_1UL_BCS0

Agenda Item: 			8.3.2

Document for:	Approval

# Introduction

This TP introduces 3DL/1UL CA configuration CA_2A-13A-46A in to the TR 36.715-03-01

**************************** Start of changes ********************************************

## 6.X	CA_2A-13A-46A_BCS0

### 6.X.1	Operating bands for CA

Table 6.X.1-1: 3DL Inter-band CA operating bands

| E-UTRA CA Band | E-UTRA Band | Uplink (UL) operating band |  |  | Downlink (DL) operating band |  |  | Duplex Mode |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | BS receive / UE transmit |  |  | BS transmit / UE receive |  |  |  |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |
| CA_2-13-46 | 2 | 1850 MHz | – | 1910 MHz | 1930 MHz | – | 1990 MHz | FDD |
|  | 13 | 777 MHz | – | 787 MHz | 746 MHz | – | 756 MHz | FDD |
|  | 46 | 5150 MHz | – | 5925 MHz | 5150 MHz | – | 5925 MHz | TDD |

### 6.X.2	Channel bandwidths per operating band for CA

Table 6.X.2-1: Supported E-UTRA bandwidths per CA configuration for 3DL inter-band CA

| CA operating / Channel bandwidth |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CA Configuration | E-UTRA Bands | 1.4 MHz | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | Maximum aggregated bandwidth [MHz] | Bandwidth Combination Set |
| CA_2A-13A-46A | 2 |  |  | Yes | Yes | Yes | Yes | 50 | 0 |
|  | 13 |  |  | Yes | Yes |  |  |  |  |
|  | 46 |  |  |  |  |  | Yes |  |  |

### 6.X.3	Co-existence studies

Table 6.X.3-3 summarizes frequency ranges where harmonics occur due to Band 2, Band 13 and Band 46 CA with 1 UL.

Table 6.X.3-3: Impact of 1 UL Harmonic Interference

|  |  |  | 2nd Harmonic |  | 3rd Harmonic |  | 7th harmonic |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Band | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge | UL Low Band Edge | UL High Band Edge |  |  |
| 2 | 1850 | 1910 | 3700 | 3820 | 5550 | 5730 |  |  |
| 13 | 777 | 787 | 1554 | 1574 | 2331 | 2361 | 5439 | 5509 |
| 46 | 5150 | 5925 | 10300 | 11850 | 15450 | 17775 |  |  |

It can be seen from Table 6.X.3-3 for CA_2A-13A-46A that there is the 3rd harmonic interterference from band 2 uplink towards band 46 downlink and the 7th harmonic interference from band 13 uplink towards band 46 downlink.

### 6.X.4	∆TIB,c and ∆RIB,c values

For three simultaneous DLs and one UL of Band 2, Band 13 and Band 46 for 1UL/3DL CA_2A-13A-46A, the TIB,c and RIB,c values are shown in Table 6.X.4-1 and Table 6.X.4-2, respectively.

Table 6.X.4-1: ΔTIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔTIB,c [dB] |
| --- | --- | --- |
| CA_2A-13A-46A | 2 | 0.3 |
|  | 13 | 0.3 |

Table 6.X.4-2: ΔRIB,c for 3DL aggregation

| Inter-band CA Configuration | E-UTRA Band | ΔRIB,c [dB] |
| --- | --- | --- |
| CA_2A-13A-46A | 2 | 0 |
|  | 13 | 0 |
|  | 46 | 0 |

### 6.X.5	REFSENS requirements

Reference sensitivity for CA_2A-13A-46A is specified in Table 6.X.5-1 for the uplink configuration specified in Table 7.3.1-2 and Table 7.3.1A-0eC of TS36.101. The exclusion zones for band 46 due to harmonics are specified in Table 7.3.1A-0eC of TS36.101.

Table 6.X.5-1: Reference sensitivity for carrier aggregation QPSK PREFSENS, CA (CA with band 46)

| Channel bandwidth |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| EUTRA CA Configuration | EUTRA band | 1.4 MHz (dBm) | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | Duplex mode |
| CA_2A-13A-46A | 2 |  |  | -98 | -95 | -93.2 | -92 | FDD |
|  | 13 |  |  | -97 | -94 |  |  |  |
|  | 46 |  |  |  |  |  | -90 | TDD |

**************************** End of changes ********************************************