3GPP TSG-RAN WG4 #84	R4-1707691

Berlin, Germany, August 21 – 25, 2017

Agenda Item:	7.7.2

Source: 	Nokia, Nokia Shanghai Bell

Title:  	Designated Operating Bands for V2X Services via Uu?

Document for:	Discussion

# Discussion

In TS 36.101, section 5.5G, table 5.5G-1, designated operating bands for V2X Communication have been defined, both for reference point Uu and PC5.

Table 5.5G-1 V2X operating band

| E-UTRA Operating Band | E-UTRA V2X Operating Band | V2X UE transmit |  |  | V2X UE receive |  |  | Duplex Mode | Interface |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | FUL_low – FUL_high |  |  | FDL_low – FDL_high |  |  |  |  |
| 47 | 47 | 5855 MHz |  | 5925 MHz | 5855 MHz |  | 5925 MHz | TDD | PC5 |
| 3 | 3 | 1710 MHz | – | 1785 MHz | 1805 MHz | – | 1880 MHz | FDD | Uu |
| 7 | 7 | 2500 MHz | – | 2570 MHz | 2620 MHz | – | 2690 MHz | FDD | Uu |
| 8 | 8 | 880 MHz | – | 915 MHz | 925 MHz | – | 960 MHz | FDD | Uu |
| 39 | 39 | 1880 MHz | – | 1920 MHz | 1880 MHz | – | 1920 MHz | TDD | Uu |
| 41 | 41 | 2496 MHz |  | 2690 MHz | 2496 MHz |  | 2690 MHz | TDD | Uu |

Messages from V2X Services are transparently sent over Uu, similar to other end-to-end services (ftp, http…). The use of the Uu reference point for V2X Services does not impact RF characteristics and minimum performance requirements for E-UTRA User Equipment (UE), i.e. there are no band specific requirements. There is therefore no technical reason why V2X Communication via Uu cannot be offered via all E-UTRA operating bands listed in table 5.5-1 of TS 36.101.  	
We were wondering whether table 5.5G shown above would imply for a network implementation, that V2X messages exchanged between an application running on a UE and a V2X Control Function/V2X Application Server in the EPC (reference points V3/V5) are restricted to operating bands 3, 7, 8, 39, and 41, and whether this restriction is intended and desirable.

We kindly ask RAN4 to confirm that from RAN4 point of view

- V2X Communication via Uu can and shall be supported by every E-UTRA operating band;

- If the first bullet point has been confirmed, we kindly ask RAN4 to discuss our CR proposal R4-1707692.