3GPP TSG-WG RAN4 Meeting #84 	R4-1707693

# Berlin, Germany, 21st – 25th Aug, 2017

Source:	Nokia, Nokia Shanghai Bell

Title:	Band specific UE channel bandwidth

Agenda item:		9.3.1.2

Document for:		Approval

# 1	Introduction

In RAN4 NR adhoc in Qingdao, the WF on band specific UE channel bandwidth is agreed in [1] as summarized below. In this paper, we preset our view on how to specify UE channel bandwidths in Rel-15.

Proposal

- Supported channel bandwidth for UE should be decided in a band specific manner

- Channel bandwidths in slide #4-5 should be supported for UE

  - Orange color means CBW candidate with square bracket

- Square bracket should be removed regardless of data channel SCS from the following channel bandwidths:

  - 40MHz for 3.3-4.2 GHz and 4.4-4.99 GHz bands

  - 200MHz for 24.25-29.5 GHz and 37-40 GHz

- FFS on how to choose the set of CBW to mandate UE to support

LTE refarming bands

NR band

- Sub6

NR band Sub6

- mmW

# 2	Discussion

The tables above are not finalized yet with square bracket. In addition, mandatory set of bandwidths for UE is FFS. A motivation to reduce the set of UE bandwidths is to simplify the scope of UE testing. The other bandwidths can be supported by the carrier aggregation of the reduced set of UE bandwidths. For example, the bandwidths between 10 and 100MH with the granularity of 5MHz can be supported in the following manner

Bandwidth	CA (with 10, 15, 20)

10			10

15			15

20			20

25			10 + 15

30			15 + 15

35			20 + 15

40			20 + 20

45			15 + 15 + 15

50			20 + 20 + 10

55			20 + 20 + 15

60			20 + 20 + 20

65			20 + 20 + 15 + 10

70			20 + 20 + 20 + 10

75			20 + 20 + 20 + 15

80			20 + 20 + 20 + 20

85			20 + 20 + 20 + 15 + 10

90			20 + 20 + 20 + 20 + 10

95			20 + 20 + 20 + 20 + 15

100			20 + 20 + 20 + 20 + 20

Observation 1: Any bandwidth between 10 and 100MHz with the granularity of 5MHz can be supported with the minimum set of 10, 15, and 20MHz assuming up to 5 CC CAs.

Therefore, it makes sense to mandate these three bandwidths for the LTE refarming bands and the NR bands 3.3-3.8 and 3.3-4.2GHz, where 5MHz granularity may be expected.

Proposal 1: For LTE refarming bands and NR bands 3.3-3.8GHz and 3.3-4.2GHz, 10, 15, and 20MHz UE channel bandwidth shall be mandated for any SCS.

Support of 5MHz channel bandwidth should be a band specific decision. If there is a deployment scenario to use 5MHz channel bandwidth in a particular NR band, then 5MHz channel bandwidth should be included only with 15kHz SCS for the band. UE shall support 5MHz bandwidth as mandatory requirement.

Proposal 2: The inclusion of 5MHz channel bandwidth (only applicable to 15kHz SCS) is a band specific decision. If 5MHz is decided to be included, it is mandatory for UE supporting the bandwidth.

Additionally, we can introduce larger channel bandwidths to reduce the number of CCs. This would simplify the CA scenarios for the bands with wide contiguous spectrum. For example, for band 41 and NR bands 3.3-3.8GHz and 3.3-4.2GHz, a single operator may have larger holding more than 60MHz, which requires 4 or 5 CCs as presented above with the minimum set of channel bandwidths. If we add 50 and 80MHz in the UE bandwidth set, then all the bandwidths can be supported with 3 or less CCs.

BW		CA (with 10, 15, 20, 50, 80)

10		10

15		15

20		20

25		10 + 15

30		15 + 15

35		20 + 15

40		20 + 20

45		15 + 15 + 15

50		50

55		20 + 20 + 15

60		50 + 10

65		50 + 15

70		50 + 20

75		50 + 15 + 10

80		80

85		50 + 20 + 15

90		80 + 10

95		80 + 15

100		50 + 50

Observation 2: For the bands with allocation larger than 60MHz, the additional UE channel bandwidths such as 50 and 80MHz are useful to reduce the number of CCs of carrier aggregation.

It has been discussed which bandwidth should be mandated for UE. The problem in making UE bandwidth optional is that the base station scheduler will be more complicated if it must handle many varieties of UE bandwidth capabilities. In addition, the optimality of UE channel bandwidth requires the UE capability signaling bits, which shall be minimized to reduce the protocol overhead.

Observation 3: Fragmented UE bandwidth capabilities are not recommended from the network control efficiency point of view.

Observation 4: The UE bandwidth capability signaling should be minimized to reduce the higher layer overhead.

Optional UE bandwidth may reduce the burden UE testing, but increases the network overhead. Therefore, we propose not to introduce any optional UE bandwidths. All required channel bandwidths shall be mandated in Rel-15. If there is a new market need after Rel-16 to have new channel bandwidth, then, introduction of optional bandwidth can be further discussed.

Therefore, in Rel-15, we only propose to introduce the maximum UE channel bandwidth in its UE capability. All channel bandwidth equal to or narrower than the maximum shall be mandated for UE. The maximum UE channel bandwidth may be SCS dependent due to the limited FFT size.

Proposal 3: Optional UE bandwidth in Rel-15 shall be only based on the SCS dependent maximum UE channel bandwidth. All narrower channel bandwidths specified for the band shall be mandatory support for UE.

# 3	Conclusion

In this paper, we have discussed band specific UE channel bandwidth. Our observations and proposals are summarized below.

Observation 1: Any bandwidth between 10 and 100MHz with the granularity of 5MHz can be supported with the minimum set of 10, 15, and 20MHz assuming up to 5 CC CAs.

Observation 2: For the bands with allocation larger than 60MHz, the additional UE channel bandwidths such as 50 and 80MHz are useful to reduce the number of CCs of carrier aggregation.

Observation 3: Fragmented UE bandwidth capabilities are not recommended from the network control efficiency point of view.

Observation 4: The UE bandwidth capability signaling should be minimized to reduce the higher layer overhead.

Proposal 1: For LTE refarming bands and NR bands 3.3-3.8GHz and 3.3-4.2GHz, 10, 15, and 20MHz UE channel bandwidth shall be mandated for any SCS.

Proposal 2: The inclusion of 5MHz channel bandwidth (only applicable to 15kHz SCS) is a band specific decision. If 5MHz is decided to be included, it is mandatory for UE supporting the bandwidth.

Proposal 3: Optional UE bandwidth in Rel-15 shall be only based on the SCS dependent maximum UE channel bandwidth. All narrower channel bandwidths specified for the band shall be mandatory support for UE.

# 4	References

- R4-1706982 	WF on band specific UE channel bandwidth, NTT DOCOMO