3GPP TSG-RAN4 Meeting #84	R4-1707792

# Berlin, DE, 21st – 25th August 2017

Source: 	Qualcomm Incorporated

Title: 	PCmax computation and evaluation for intra-band CA with sTTI

Agenda item:	8.27.2

Document for:	Approval

# Introduction

In [1], PC max computation and evaluation for inter-band CA have been discussed. Intra-band CA presents new set of challenges as a single transmit chain is used for all the carriers in typical UE architectures. So, when the power of any of the sTTIs within a sub-frame needs to be increased, the gain of the transmit chain might need to be increased accordingly. This might result in a gain and phase transient for the longer TTI within the sub-frame.

# Discussion

It is our opinion that for intra-band CA, with the current mechanisms in place to handle power control, phase continuity cannot be maintained across TTIs when:

- TTIs are of different length

- TTIs of the same length have a different TAG within or across PUCCH groups.

Proposal 1: Intra-band CA shall be supported only among TTIs of the same length and TAG within and across different PUCCH groups.

Proposal 2: For intra-band CA among TTIs of the same length and TAG, PC max computations proposed in [1] should be applied.

# Conclusion

In this paper, the concerns with supporting intra-band CA among different TTI lengths have been presented. And the following proposal are made to support intra-band CA:

Proposal 1: Intra-band CA shall be supported only among TTIs of the same length and TAG within and across different PUCCH groups.

Proposal 2: For intra-band CA among TTIs of the same length and TAG, PC max computations proposed in [1] should be applied.

# References

[1] R4-1707791, PCmax computation and evaluation for inter-band CA with sTTI, 3GPP TSG-RAN4 Meeting #84, Berlin, DE, 21st – 25th August 2017.