GP TSG-RAN WG4 Meeting #84	R4-1707875

Berlin, Germany, 21st - 25th August 2017

Agenda item:			9.6.4

Source:	MediaTek Inc.

Title:	Simulation Results for RSRP Measurement Accuracy of NR

Document for:			Discussion

1	Introduction

In [1], it was agreed to conduct link-level simulation studies for measurements. Besides, the simulation assumptions for SS block measurements were agreed [2]. In this contribution, the RSRP accuracy for NR is investigated. The corresponding simulation results are provided as well.

2	RSRP simulation assumptions

The simulation assumptions are given in Table 1:

| Parameters | Value |
| --- | --- |
| Carrier frequency | 5 GHz |
| System bandwidth | 20 MHz |
| Subcarrier spacing | 15 kHz |
| SS burst periodicity | 5ms |
| Number of SS blocks per SS burst set, K | 1 |
| L1 sampling interval | 40ms |
| Measurement period | 200ms |
| L3 filtering | Disabled |
| SNR | -6dB |
| CP length | Normal |
| Channel type | Static，EPA 5Hz |

Table 1: RSRP simulation assumptions.

3	RSRP simulation results

In this section, we compare the following 2 measurement methods:

- RSRP based on NR-SSS

- RSRP based on NR-SSS and PBCH DMRS

Table 2 provides the delta RSRP statics based on different sample number (N), with any 2 samples separated by at least 40ms (L1 sampling interval) within 200ms measurement period. According to the simulation results, we can see that when N ≥ 2:

- The delta RSRP of both measurement methods are in the -2dB to +2dB range.

- The RSRP absolute accuracy difference between two measurement methods is approximately within 0.23dB and 0.3dB, in AWGN and EPA 5Hz channel, respectively.

| SNR = -6dB |  | CDF of delta RSRP (dB) |  |  |  |  |  | Absolute accuracy |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Channel Type | N | 5% |  | 50% |  | 95% |  | SSS | SSS & PBCH DMRS |
|  |  | SSS | SSS & PBCH DMRS | SSS | SSS & PBCH DMRS | SSS | SSS & PBCH DMRS |  |  |
| AWGN | 1 | -1.283 | -0.953 | 0.788 | 0.659 | 2.354 | 2.109 | 2.114 | 1.829 |
|  | 2 | -0.840 | -0.601 | 0.600 | 0.512 | 1.772 | 1.548 | 1.564 | 1.331 |
|  | 3 | -0.625 | -0.457 | 0.504 | 0.441 | 1.494 | 1.296 | 1.303 | 1.113 |
|  | 4 | -0.513 | -0.364 | 0.446 | 0.390 | 1.322 | 1.150 | 1.156 | 0.986 |
|  | 5 | -0.456 | -0.316 | 0.406 | 0.362 | 1.210 | 1.051 | 1.042 | 0.901 |
| EPA5 | 1 | -2.210 | -1.796 | 0.197 | 0.112 | 2.132 | 1.840 | 2.157 | 1.823 |
|  | 2 | -1.652 | -1.305 | 0.105 | 0.069 | 1.607 | 1.330 | 1.626 | 1.320 |
|  | 3 | -1.413 | -1.085 | 0.061 | 0.044 | 1.380 | 1.099 | 1.396 | 1.091 |
|  | 4 | -1.255 | -0.972 | 0.049 | 0.036 | 1.253 | 0.978 | 1.254 | 0.973 |
|  | 5 | -1.158 | -0.857 | 0.032 | 0.032 | 1.145 | 0.876 | 1.155 | 0.864 |

Table 2: delta RSRP based on 1 to 5 samples.

Observation 1: When sample number is larger than 2, the delta RSRP of both measurement methods are in the -2dB to +2dB range and the RSRP absolute accuracy difference between two measurement methods is small.

Besides, the CDF curves of 2 measurement methods are also compared in Figure 1-a,b,c,d,e and Figure 2-a,b,c,d,e. We can also see that the gain we obtain from adding the PBCH DMRS into RSRP calculation is limited. Consider that the SSS RSRP accuracy is good enough. We suggest that SSS is sufficient enough for the RSRP measurement.

Proposal 1: RSRP measurement requirement is specified based on SSS only.

Besides, the CDF curves of SSS measurement accuracy with different N are also compared in Figure 1-f and Figure 2-f. We can see that the measurement accuracy has no significant improvement when N≥3.

Observation 2: The measurement accuracy has no significant improvement when sample number N≥3.

According to the observation 2, we think that number of samples N should be at least no smaller than 3 in order to guaranteed acceptable RSRP measurement accuracy.

Proposal 2: Sample number should be at least no smaller than 3 for RSRP measurement.

Figure 1-a, delta RSRP in AWGN, with N =1.          Figure 1-b, delta RSRP in AWGN, with N =2.
   
Figure 1-c, delta RSRP in AWGN, with N =3.          Figure 1-d, delta RSRP in AWGN, with N =4.

Figure 1-e, delta RSRP in AWGN, with N =5.          Figure 1-f, delta RSRP based on 1 to 5 samples.

Figure 2-a, delta RSRP in EPA 5, with N =1.          Figure 2-b, delta RSRP in EPA 5, with N =2.
   
  Figure 2-c, delta RSRP in EPA 5, with N =3.          Figure 2-d, delta RSRP in EPA 5, with N =4.                    Figure 2-e, delta RSRP in EPA 5, with N =5.          Figure 2-f, delta RSRP based on 1 to 5 samples.

4	Measurement Period and Sample Number

In this section, we discuss the possible options for measurement period and sample number.

Consider that we may observe correlated samples in slow fading channel, e.g. Doppler frequency 5Hz. To guarantee independent samples for L3 input (i.e. L1 output), we need at least 1/5 =0.5s = 200ms separation between two consecutive samples. That indicates the minimum requirement of measurement period.

As we know, NR system may apply different SS burst set periodicity, which implies measurement periods should be SS burst set periodicity dependent. For example, when SS burst set periodicity equals to 160ms, roughly only one sample can be observed within 200ms. The measurement periods should be extended to obtain more samples. Therefore, we suggest to categorize different periodicities into three groups. Below we list two options for future evaluations:

|  | Option 1 |  | Option2 |  |
| --- | --- | --- | --- | --- |
| SS burst set periodicity (ms) | Sample number N | L1 measurement period (ms) | Sample number N | L1 measurement period (ms) (close to 200ms) |
| 5, 10, 20, 40 | N (e.g. N = 5) | 40 N (e.g. 200) | 5 | 200 |
| 80 | N (e.g. N = 5) | 80 N (e.g. 400) | 3 | 240 |
| 160 | N (e.g. N = 5) | 160 N (e.g. 800) | 3 | 480 |

Table 3: measurement period in different SS burst set periodicity.

The measurement period is mainly decided by the sample number N in option 1, whereas it is set as close as possible to 200ms in option 2, with no less than 3 samples assumed. Please note that some modification for extending to inter-frequency cases should be further discussed. Consider that the reporting delay might be further scaled by the number of frequency layer to be monitored, we slightly prefer option 2 to avoid too long reporting delay.

Proposal 3: RAN4 to dsicuss the measurement period and sample number used in one L1 output.

5	Summary

In this contribution, the simulation results for NR measurement accuracy are presented. It is observed that

Observation 1: When sample number is larger than 2, the delta RSRP of both measurement methods are in the -2dB to +2dB range and the RSRP absolute accuracy difference between two measurement methods is small.

Observation 2: The measurement accuracy has no significant improvement when sample number N≥3.

And we propose

Proposal 1: RSRP measurement requirement is specified based on SSS only.

Proposal 2: Sample number should be at least no smaller than 3 for RSRP measurement.

Proposal 3: RAN4 to dsicuss the measurement period and sample number used in one L1 output.

4	Reference

[1] R4-1704806, NR RRM way forward, Ericsson, May 2017.

[2] R4-1706904, Link-level simulation assumptions for SS block measurements, Ericsson, June 2017.