3GPP TSG-RAN WG4 Meeting #84 	R4-1709032

# Berlin, Germany, 21 – 25 August 2017

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 36.133 | CR | 5129 | rev | 1 | Current version: | 14.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | x | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR for NB-IoT Transmit Timing Test |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Neul |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NB_IOT-Perf |  |  |  |  |  |  | Date: |  | 2017-08-25 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-14 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | During the test in A.7.17.2, the test system applies a sudden timing change of up to 144 Ts. On the other hand, the max timing adjustment allowed for the UE is 58.33Ts. Hence it is very demanding to require the UE to handle a significantly bigger timing change than that for the base station. NB-IoT in-band and guardband operation in 5MHz LTE is supported in R14. RRM test cases need to be updtaed to also apply for 5MHz case. OCNG pattern is missing. |  |  |  |  |  |  |  |  |
| Summary of change: |  | The timing change is spread over a number of NPDCCH periods giving the UE opportunities to track the relatively smaller timing changes. Make the test also apply for 5MHz LTE system. Add OCNG pattern. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The requirement would be unrealistic for UEs. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.7.1.17 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  | x |  | Test specifications |  |  | TS 36.521-3 |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

<Start of Changes>

### A.7.1.17	E-UTRAN HD-FDD – UE Transmit Timing Accuracy Tests for Category NB1 UE In-Band mode under normal coverage

#### A.7.1.17.1	Test Purpose and Environment

The purpose of this test is to verify that the Category NB1 UE under normal coverage is capable of following the frame timing change of the connected eNode B and that the UE initial transmit timing accuracy, maximum amount of timing change in one adjustment, minimum and maximum adjustment rate are within the specified limits. This test will verify the requirements in clause 7.20.

For this test a single NB-IoT cell and a single LTE cell are used. Test parameters are given in Table A.7.1.17.1-1, Table A.7.1.17.1-2, and Table A.7.1.17.1-3. The transmit timing is verified by the UE transmitting NPUSCH.

Table A.7.1.17.1-1: General Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| NB-IoT Operation mode |  | In-band |
| DRX |  | OFF |
| NPRACH configuration |  | As specified in A.3.18 |
| NPDCCH repetition level |  | 1 |
| NPUSCH repetition level |  | 1 |

Table A.7.1.17.1-2: nCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| RF Channel Number |  | 1 |
| BWchannel | kHz | 200 |
| PRB location within eCell |  | eCell BWchannel 5MHz: 17 eCell BWchannel 10MHz:30 |
| NPDSCH parameter |  | eCell BWchannel 5MHz: R.17 HD-FDD eCell BWchannel 10MHz: R.15 HD-FDD |
| NPDCCH parameter |  | eCell BWchannel 5MHz: R.41 HD-FDD eCell BWchannel 10MHz: R.29 HD-FDD |
| NPBCH_RA | dB | -3 |
| NPBCH_RB |  |  |
| NPSS_RA |  |  |
| NSSS_RA |  |  |
| NPDCCH_RA |  |  |
| NPDCCH_RB |  |  |
| NPDSCH_RA |  |  |
| NPDSCH_RB |  |  |
| NOCNG_RA Note1 |  |  |
| NOCNG_RB Note1 |  |  |
|  | dBm/15 kHz | Specified in Table A.7.1.17.1-3 |
|  | dB | 4 |
|  | dB | 4 |
| Antenna Configuration |  | 2x1 |
| Propagation condition | - | AWGN |
| Note 1 NOCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

Table A.7.1.17.1-3: eCell specific Test Parameters for UE Transmit Timing Accuracy Tests for E-UTRAN HD-FDD Category NB1 UE in In-Band mode under normal coverage

| Parameter | Unit | Value |
| --- | --- | --- |
|  |  | Test 1 |
| E-UTRA RF Channel Number |  | 1 |
| BWchannel | MHz | 5 or 10 |
| NOCNG Pattern |  | - |  | BWchannel 5MHz: NOP.4 FDD BWchannel 10MHz: NOP.1 FDD |
| PBCH_RA | dB | 0 |
| PBCH_RB |  |  |
| PSS_RA |  |  |
| SSS_RA |  |  |
| PCFICH_RB |  |  |
| PHICH_RA |  |  |
| PHICH_RB |  |  |
| PDCCH_RA |  |  |
| PDCCH_RB |  |  |
| OCNG_RANote1 |  |  |
| OCNG_RBNote1 |  |  |
|  | dBm/15 kHz | -98 |
|  | dB | 4 |
| Antenna configuration |  | 1x1 |
| Propagation condition | - | AWGN |
| Note 1: OCNG shall be used such that the cell is fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. |  |  |

#### A.7.1.17.2	Test Requirements

For parameters specified in Tables A.7.1.17.1-1, A.7.1.17.1-2, and A.7.1.17.1-3, the initial transmit timing accuracy, the maximum amount of timing change in one adjustment, the minimum and the maximum adjustment rate shall be within the limits defined in clause 7.20.2.

The following sequence of events shall be used to verify that the requirements are met.

The test sequence shall be carried out in RRC_CONNECTED:

a) 	After a connection is set up with the cell, the test system sends NPDCCH including uplink grant for NPUSCH transmission and the test system shall measure the UE transmit timing offset (nTs) and verify that it is within Te (nTs ≤ NTA×TS   ± 80×TS) with respect to the first detected path (in time) of the corresponding downlink frame of NB-IoT cell 1.

b) 	Using the value of n measured in a), the test system adjusts the downlink transmit timing for the cell:

- if n < 0, by +(144 – |n|)TS compared to that in (a).

- if n ≥ 0, by -(144 – |n|)TS compared to that in (a).

The timing adjustment is performed monotonically in multiple steps of ∆T≤ 9×TS per NPDCCH period (∆T is to be defined in the test procedure) until the above required total timing change is achieved, during which no grant is transmitted for the UE.

c) 	Immediately after (b), the test system sends NPDCCH including uplink grant for NPUSCH transmission and immediately after receiving NPUSCH the test system repeatedly sends NPDCCH including uplink grant for NPUSCH transmission until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1. The test system shall verify that the adjustment step size and the adjustment rate shall be according to the requirements in clause 7.20.2 until the UE transmit timing offset is within NTA×TS  ± 80×TS with respect to the first detected path (in time) of the corresponding downlink frame of cell 1.

d) 	The test system the test system sends NPDCCH including uplink grant for NPUSCH transmission and shall verify that the UE transmit timing offset stays within NTA×TS  ± 80×TS with respect to the first detected path  (in time) of the corresponding downlink frame of NB-IoT cell 1.

<End of Changes>