3GPP TSG-RAN WG4 Meeting #84bis	R4-1711520

Dubrovnik, Croatia, 9th-13th October 2017

Source: 	Ericsson

Title:  	Conducted blocker levels for NR BS In-band blocking for FR2

Agenda Item:	9.5.4.3.2

Document for:	Discussion

# Introduction

BS receiver blocking has been studied in RAN4 in the Rel-14 NR SI phase. In one of our companion contributions [3], we provided our opinion on the methodology based on joint probability of receiver blocking where probability of both wanted signal and blocking signal above a level are considered.

In two of our previous contributions [5] and [6], we have provided details simulations results. In this contribution, we provide the summary of these simulations and provide our conclusion.

# Blocker levels in mmWave UMa scenario Summary

The following table summarizes the blocker levels that we have found from the simulations that we have run based on the methodology as described in [3].

|  | when array gain is considered |  | when only element gain is included |  |
| --- | --- | --- | --- | --- |
| Collocated UMa deployment | 1-P3 [%] | 96%-tile | 1-P3 [%] | 97.63%-tile |
|  | Blocking level [dBm] | [-70 -65] | Blocking level [dBm] | [-66 -63] |
| Non-collocated UMa deployment | 1-P3 [%] | 95.82%-tile | 1-P3 [%] | 97.63%-tile |
|  | Blocking level [dBm] | [-65 -59] | Blocking level [dBm] | [-55 -49] |

The range in the above table represents the system utilization levels, which spans from 50% to 90%.

# Proposal

In the above simulation results, we have obtained blocker level when joint probability of the wanted and blocker level is considered. In the above simulations, 200MHz CBW is considered, with 10dB NF, the wanted signal level becomes -75dBm. As we see from simulation results, we propose the following:

Proposal-1: Consider -49dBm as “conducted” blocker level for FR2.

The delta between wanted signal level and blocker level is around 26dB. Thus, we propose the following:

Proposal-2: Consider Delta between wanted level and blocker level as ~26dB.

It is worth noting here that the blocking levels in this contribution are considered for conducted levels.

Observation: The results in this contribution are strictly valid for conducted blocking levels.

This “conducted” blocker level can be converted to OTA level. We have discussed this in one of our companion contribution in [7].

# References

TR 38.803

R4-1700224, BS OTA sensitivity for mm-wave frequencies, Ericsson

R4-1709857, Methodology for mmWave NR BS receiver blocking investigation, Ericsson

R4-1708145, Consideration of joint probability for wanted and interference signals in mmWave NR BS receiver blocking investigation

R4-1706812, Blocking simulation results for NR BS by considering joint probability for receiver blocking, Ericsson

R4-1708146, In-band blocking simulation results for NR BS by considering joint probability for receiver blocking, Ericsson

R4-1711002, OTA Receiver blocking requirement, Ericsson