3GPP TSG-RAN4 Meeting #84bis	R4-171596

# Dubrovnik, HR, 9th – 13th October 2017

Title:	Draft response LS for time mask for sTTI

Response to:	Reply LS on implication of sTTI operation on UL ON/OFF time mask

Release:	Rel-15

Work Item:  	LTE_sTTIandPT-core

Source:	RAN4

To:	RAN1

CC:

Contact Person:

# Name:	Prashanth Akula

# E-mail Address:	pakula@qti.qualcomm.com

# Attachment:  	R4-1710622.

1. Overall Description:

RAN4 would like to thank RAN1 for the LS on implication of sTTI operation on UL ON/OFF time mask. RAN4 has discussed the issues mentioned in the LS, and have following feedbacks:

- Regarding the duration of the transient period the agreements in RAN4 are:

  - For ON-OFF mask for sTTI:

    - For Tx BW>1.4 MHz, duration of transient period=10 usec

    - For Tx BW=1.4 MHz, duration of transient period =20 usec

    - The transient period shall be outside the TTI duration

  - For consecutive sTTI symbols:

    - Transient periods =10+10 usec.

    - The location for the transient period is dependent on the contents of the TTI

- There are multiple cases for consecutive TTIs as listed below:

| Case | Current symbol/ TTI | Next symbol/TTI |
| --- | --- | --- |
| 1 | 2os/3os | 2os/3os |
| 2 | Intra sTTI 2os/3os/7os hopping |  |
| 3 | 2os/3os | 1msec/7os |
| 4 | 1msec/7os | 2os/3os |
| 5 | 1msec/7os | 7os/1msec |

Table 1: List of all possible cases for transient periods between consecutive symbols

Table 2: Logic to decide the location for the transient periods for each of the above cases

- Observation:

  - To fully utilize the above logic a new data pattern for sTTI 5 is needed: “DRD” which allows the transient period to be place completely in ‘R’ when SRS is scheduled in the last symbol. Placing the entire transient period in ‘R’ will protect ‘D’ and ‘SRS’. Currently only one data pattern “RDD” has been defined for sTTI5.

2. Actions:

ACTION: 	RAN4 kindly asks RAN1 to take above information in the work for shortened TTI and processing time for LTE.

3. Date of Next 3GPP TSG-RAN4 Meetings:

# 3GPP TSG-RAN WG4 Meeting #85		27 Nov - 1 Dec 2017		Reno, Nevada, US

# 3GPP TSG-RAN WG4 Meeting #86		26 Feb - 2 Mar 2018		Athens, GR