3GPP TSG-RAN WG4 Meeting #85	R4-1712458

Reno, US, 27th Nov – 1st Dec 2017

Source: 	Nokia, Nokia Shanghai Bell, T-Mobile USA

Title: 					TP to TR 38.817-01: Notation of NR-LTE contiguous intraband DC acronymn

Agenda Item: 			9.2.2.1

Document for:	Approval

# 1	Introduction

T-Mobile USA has LTE-NR DC configuration where both carriers belong to same physical band i.e. band 71 and n71[1].

| 71 | n71 | Nelson Ueng T-Mobile USA | neng-tsann.ueng@t-mobile.com | Ericsson, Samsung, Deutsche Telecom, Nokia, Qorvo, Skyworks | new |
| --- | --- | --- | --- | --- | --- |

It has been clarified by T-Mobile USA that inorder to complete the combination in time the allocation of LTE carrier and NR carrier form a contiguous block of spectrum with maximum bandwidth of 20 MHz. This would be quite similar scenario as LTE CA bandwidth class B with an obvious difference that carriers are from different RAT.

Due to agreement that LTE bands and NR bands are separated in DC acronym a way of defining contiguous intraband LTE-NR DC is needed. We propose following notation using band 71 as an example

DC_71b_n71b

Where small b after LTE band and NR band jointly indicate the CA bandwidth class i.e. ‘b’ + ‘b’ = ‘B’ and similarly ‘c’ + ‘c’ = ‘C’,… etc.)

# 2	References

[1] R4- 1711048, Update of DC band combination lists, RAN4#84bis

*************************** Start of the TP **********************************************

## 4.1	Operating bands

While requirements are often generic and band agnostic, there may in many cases be separate requirements for different bands. In some cases, these are identified directly using the band number. They will also be references to different frequency ranges covering many bands, identified as requirements for sub-6 GHz bands (Frequency Range 1) and mmWave bands (Frequency Range 2). These ranges are for the purpose of the specification defined as follows:

-	FR1:	450 – 6000 MHz

-	FR2:	24250 – 52600 MHz.

The lowest frequency in FR1 is selected as the lowest frequency identified for IMT by the ITU-R [2]. FR2 is defined starting from the lowest frequency under study for IMT under WRC Agenda item 1.13, up to the highest frequency within the scope of the present work item. In the future when new operating bands may be added outside the frequency ranges of FR1 and FR2, those can be incorporated by extending FR1 or FR2, or by adding new ranges (FR3, etc.).

The definitions of FR1 and FR2 are common the UE and BS and should be specified within clause 4 of the specifications.

As for UTRA and LTE, bands for NR are identified by band numbers. A number of different aspects are considered for developing an NR band numbering scheme:

1)	For NR in LTE “refarming” bands, whether to re-use the LTE band numbers for NR.

2)	For new NR bands, different alternative options are considered:

a)	To put new NR bands above 6 GHz in a new separate numbering range within existing signaling capability.

b)	To group new NR bands below 6 GHz with the LTE bands, resulting in separate band numbering ranges for below 6 GHz and above 6 GHz bands.

c)	To assign unused numbers to all new bands on a “first come first served” basis, regardless of frequency range, duplex mode or RAT (LTE, NR, etc.)

3)	How to consider “Duplex mode” for different types of bands

The final NR band numbering scheme is based on the following:

1)	For NR in LTE “refarming” bands, LTE band numbers are reused for NR. This will be consistent with the scheme for UTRA and LTE, where the same number was used ofr the same frequency range, while written in a different way. It also conserves band numbers.

2)	NR band numbers will be written with a prefix “n”, to distinguish from band numbers for other RATs. For example, NR Band n7 will correspond to LTE Band 7 and UTRA Band VII.

3)	New bands should be assigned band numbers on a “first come first served” basis in reserved frequency ranges as follows:

-	65 to 256 is reserved for new LTE and new NR bands in FR1.

-	257 to 512 is reserved for new NR bands in FR2. By avoiding LTE bands in this number range, the present LTE band definition is preserved and will most likely not need any changes in the future.

-	New LTE only TDD band can use band number from existing LTE TDD numbering space until all numbers up to 64 are used and after that from the reserved 65-256 space.

4)	For both re-farming LTE bands and new NR bands, duplex mode should be assigned and described in band defining table (e.g. in TS 8.101 and TS 38.104)

-	TDD for unpaired bands and FDD for paired bands

-	FFS identifies options under study, such as flexible duplexing for paired bands

-	If multiple duplexing modes are allowed in a specific frequency range, separate bands will be introduced with each duplexing mode

NR DC and CA band combination are defined using a notation similar to LTE, but using the distinct notations for the different RATs: A number (Arabic) for LTE and a number with prefix “n” for NR. Examples are given in Table 4.1-1.

Contiguous intraband LTE-NR DC will use following acronym (band 71 as an example) DC_71b_n71b where small b after LTE band and NR band jointly indicate the CA bandwidth class i.e. ‘b’ + ‘b’ = ‘B’ and similarly ‘c’ + ‘c’ = ‘C’, etc.). Non-contiguous intraband LTE-NR DC will follow the LTE notation i.e. DC_71A_n71A.

Table 4.1-1: Example notation for DC and CA.

| Representation | Corresponding functionality |
| --- | --- |
| CA_n77-n78 | NR interband CA of band n77 and band n78 |
| DC_1-2_n77 | NR-LTE DC with LTE interband CA of band 1 and band 2 and NR band n77. |
| DC_1-2_n77-n78 | NR-LTE DC with LTE interband CA of band 1 and band 2 and NR interband CA of band n77 and band n78. |
| DC_71b_n71b | NR-LTE Contiguous intraband DC configuration with two contiguously aggregated carriers of CA BW class B |
| DC_71A_n71A | NR-LTE Non-contiguous intraband DC configuration with two aggregated carriers |
| DC_2A-71b_n66A-n71b | NR-LTE DC involving contiguous intraband DC configuration |
| DC_2A-71A_n66A-n71A | NR-LTE DC involving non-contiguous intraband DC configuration |

Editor’s note: It is FFS to define CA bandwidth class (postfix of the bands) for NR:
-	How to handle the single carrier operation with larger than 100PRBs
-	How to handle different maximum bandwidth for each frequency range

Editor’s note:It is FFS how to handle BCS for NR
-	Reuse exiting BCS concept or adopt simplified BS concept or no BCS

For SUL and NR-LTE co-existence, example of notation is given in Table 4.1-2.

Table 4.1-2: Example notation for SUL and NR-LTE coexistence.

| Representation | Corresponding functionality |
| --- | --- |
| SUL_n78-n81 | Band combination of NR band n78 and band n81(SUL) for NR operation. |
| DC_3-SUL_n78-n80 | LTE-NR DC between LTE Band 3, and NR bands n80 (SUL) and n78 including NR-LTE coexistence with UL sharing. |

The definition of operating band can be inherited from LTE. In TS 38.101 and TS 38.104, there should be separate tables defining the band numbers in FR1 and FR2. This exploits the commonality within frequency ranges and reduces table size. For FR2, the table will only contain unpaired frequency ranges, assuming that there will be no FDD operation. To be consistent, all frequencies are defined in MHz.

For the first version of the spec, bands listed as part of the NR WID should be included. The new bands in Tables 4.1-3 and 4.1-4 are defined for NR.

Table 4.1-3: New NR bands in FR1.

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n1 | 1920 – 1980 MHz | 2110 – 2170 MHz | FDD |
| n2 | 1850 – 1910 MHz | 1930 – 1990 MHz | FDD |
| n3 | 1710 – 1785 MHz | 1805 – 1880 MHz | FDD |
| n5 | 824 – 849 MHz | 869 – 894MHz | FDD |
| n7 | 2500 – 2570 MHz | 2620 – 2690 MHz | FDD |
| n8 | 880 – 915 MHz | 925 – 960 MHz | FDD |
| n20 | 832 – 862 MHz | 791– 821MHz | FDD |
| n28 | 703 – 748 MHz | 758 – 803 MHz | FDD |
| n38 | 2570 – 2620 MHz | 2570 – 2620 MHz | TDD |
| n41 | 2496 – 2690 MHz | 2496 – 2690 MHz | TDD |
| n50 | 1432 – 1517 MHz | 1432 – 1517 MHz | TDD |
| n51 | 1427 – 1432 MHz | 1427 – 1432 MHz | TDD |
| n66 | 1710 – 1780 MHz | 2110 – 2200 MHz | FDD |
| n70 | 1695 – 1710 MHz | 1995– 2020 MHz | FDD |
| n71 | 663 – 698 MHz | 617 – 652 MHz | FDD |
| n74 | 1427 –1470 MHz | 1475 – 1518 MHz | FDD |
| n75 | N/A | 1432 – 1517 MHz | SDL |
| n76 | N/A | 1427 – 1432 MHz | SDL |
| n77 | 3.3 – 4.2 GHz | 3.3 – 4.2 GHz | TDD |
| n78 | 3.3 – 3.8 GHz | 3.3 – 3.8 GHz | TDD |
| n79 | 4.4 – 5.0 GHz | 4.4 – 5.0 GHz | TDD |
| n80 | 1710 – 1785 MHz | N/A | SUL |
| n81 | 880 – 915 MHz | N/A | SUL |
| n82 | 832 – 862 MHz | N/A | SUL |
| n83 | 703 – 748 MHz | N/A | SUL |
| n84 | 1920 – 1980 MHz | N/A | SUL |
| n85 | 2496 – 2690 MHz | N/A | SUL |

Table 4.1-4: New NR bands in FR2.

| Band number | UL | DL | Duplex mode |
| --- | --- | --- | --- |
| n257 | 26.5 –29.5 GHz | 26.5 –29.5 GHz | TDD |
| n258 | 24.25 – 27.5 GHz | 24.25 – 27.5 GHz | TDD |
| n259 | N/A | N/A | TDD |
| n260 | 37–40 GHz | 37–40 GHz | TDD |

*************************** End of the TP **********************************************