3GPP TSG-RAN WG4 Meeting #85                           	         R4-1712932

Reno, US, 27 November – 1 December, 2017

Source: 	Apple

Title:           TP on TR 37.863-01-01 for Single Tx Switched UL

Agenda Item:	  9.4.2

Document for:  Approval

# Introduction

In RAN4#84bis meeting, some agreements were made in [1] for single Tx switch UL. The major points comprise of:

- Inter-band combinations that have IM2 are categorized difficult and inter-band combinations that have IM3 for L-L and H-H combinations are categorized difficult.

- Formula’s are used to define which channel combinations within difficult inter-band combinations are not affected by IM2 and IM3.

  - Concurrent 2 UL is mandatory for these channel combinations.

Besides of the agreements, one issue is still FFS as following:

- RAN4 shall discuss the definition of UL and DL channels in the next meeting

•Alt.1. spectrum holdings of a operator in a band

•Alt.2. aggregated UE channel bandwidth configured by the network

•Alt.3. transmission bandwidth

In this paper, we elaborate on the FFS issue and provide corresponding proposal. In addition, in order to support this feature, the text proposal for TR37.863-01-01[2] is also presented.

# Discussion

Issue -Definition of DL and UL channels in the formulas

Regarding the definition of DL and UL channels used in the formulas, Alt.1 suggests to use the spectrum holdings of an operator in a band. However, normally speaking this information probably is not available at UEs. For Alt.2, aggregated UE channel bandwidth configured by the network is a little bit confusing, for it is not necessarily working as aggregated carriers. And for Alt.3, the transmission bandwidth is dynamic per each transmission, subject to eNB’s scheduling. To our understanding, the DL and UL channels should be static or semi-static rather than dynamic. Note that in RAN2, the terms “carrierBandwidthDL” and “carrierBandwidthUL” for NR are applied when network configures the NR carrier to UE. Since the definitions for them are still not crystal clear, we suggest following the conventional definition in E-UTRAN and using “channel bandwidth of the cell”, at least for now. Whenever RAN2 determines on these terms, RAN4 texts can be updated accordingly.

Quote from TS36.101: Channel bandwidth: The RF bandwidth supporting a single E-UTRA RF carrier with the transmission bandwidth configured in the uplink or downlink of a cell.

Proposal 1: Using the term “channel bandwidth of the cell” to define the DL and UL channels in the formulas. It is subject to update according to RAN2 progress.

# Text Proposal

------------ < First Change > -------------

# 1	Scope

The present document is a technical report for Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band under Rel-15 time frame. The purpose is to gather the relevant background information and studies in order to address Dual connectivity (DC) band combinations of LTE 1DL/1UL + one NR band for the Rel-15 band combinations in Table 1-1.

Table 1-1: Release 15 DC band combination of LTE 1DL/1UL + one NR band

| DC combination | REL independent from | Difficult for simultaneous UL(1) |
| --- | --- | --- |
| DC_1A-n77A | Rel-15 | Yes |
| DC_1A-n78A | Rel-15 | No |
| DC_1A-n79A | Rel-15 | No |
| DC_3A-n77A | Rel-15 | Yes |
| DC_3A-n78A | Rel-15 | Yes |
| DC_3A-n79A | Rel-15 | No |
| DC_11A-n77A | Rel-15 | No |
| DC_11A-n78A | Rel-15 | No |
| DC_11A-n79A | Rel-15 | No |
| DC_11A-n257A | Rel-15 | No |
| DC_18A-n77A | Rel-15 | No |
| DC_18A-n78A | Rel-15 | No |
| DC_18A-n79A | Rel-15 | No |
| DC_18A-n257A | Rel-15 | No |
| DC_19A-n77A | Rel-15 | No |
| DC_19A-n78A | Rel-15 | No |
| DC_19A-n79A | Rel-15 | No |
| DC_21A-n77A | Rel-15 | No |
| DC_21A-n78A | Rel-15 | No |
| DC_21A-n79A | Rel-15 | No |
| DC_26A-n77A | Rel-15 | No |
| DC_26A-n78A | Rel-15 | No |
| DC_26A-n79A | Rel-15 | No |
| DC_26A-n257A | Rel-15 | No |
| DC_28A-n77A | Rel-15 | No |
| DC_28A-n78A | Rel-15 | No |
| DC_28A-n79A | Rel-15 | No |
| DC_41A-n257A | Rel-15 | No |
| DC_42A-n77A | Rel-15 | No |
| DC_42A-n78A | Rel-15 | No |
| DC_42A-n79A | Rel-15 | No |
| DC_20A-n28A | Rel-15 | Yes |
| DC_39A-n78A | Rel-15 | No |
| DC_39A-n258A | Rel-15 | No |
| DC_1A_n257 | Rel-15 | No |
| DC_3A_n257 | Rel-15 | No |
| DC_3A-n258A | Rel-15 | No |
| DC_5A-n257A | Rel-15 | No |
| DC_7A-n78 | Rel-15 | No |
| DC_7A-n258A | Rel-15 | No |
| DC_8A-n78A | Rel-15 | No |
| DC_8A-n79A | Rel-15 | No |
| DC_8A-n258A | Rel-15 | No |
| DC_19A-n257 | Rel-15 | No |
| DC_20A-n78A | Rel-15 | No |
| DC_21A-n257 | Rel-15 | No |
| DC_28A-n257 | Rel-15 | No |
| DC_28A-n258A | Rel-15 | No |
| DC_38A-n78A | Rel-15 | No |
| DC_39A-n79A | Rel-15 | No |
| DC_41A-n77A | Rel-15 | No |
| DC_41A-n78A | Rel-15 | No |
| DC_41A-n79A | Rel-15 | No |
| DC_41A-n258A | Rel-15 | No |
| DC_42A-n257 | Rel-15 | No |
| DC_1A-n28 | Rel-15 | No |
| DC_3A-n7 | Rel-15 | No |
| DC_3A-n28 | Rel-15 | No |
| DC_5A-n78A | Rel-15 | No |
| DC_7A_n28 | Rel-15 | No |
| DC_7A-n257A | Rel-15 | No |
| DC_66A-n257A | Rel-15 | No |

Note(1): Details on how to support it can be found in Section 5.2.

This TR contains a general part and band specific combination part. The actual requirements are added to the corresponding technical specifications.

---------- < Second Change >----------

5  DC band combination of LTE 1DL/1UL + one NR band: General Part

--------<Unchanged texts are omitted>---------

5. X  Single Tx Switched UL

In some LTE-NR inter-band combinations, as indicated in the “Difficult for simultaneous UL” column in Table 1-1, it is difficult for UE to perform simultaneous LTE+NR UL transmission. The band combinations in which the 2nd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. And the band combinations for L-L (<1GHz) and H-H (1.695GHz-2.695GHz) in which the 3rd order IMD of the simultaneous UL bands fall into its own Rx band are categorized as difficult. For non-difficult band combinations, simultaneous UL transmission is mandatory.

The following formulas define which channel combinations within difficult band combinations are not affected by IM2 and IM3. Simultaneous UL transmission is mandatory for the unaffected channel combinations in difficult band combinations.

Interference bandwidth: IBW = |a| x CBW1 + |b| x CBW2

|a| + |b| = 2 (or 3)

CBW1 and CBW2 are UL channels

Center frequency of IBW:  fIBW = a x f1 + b x f2

f1 and f2 are center frequency of each UL channel

The range of IMD 2 (or 3):  [fIBW – IBW/2, fIBW + IBW/2]

Note: The DL channel and UL channels are the channel bandwidth of the cell.

---------- < End of Change >----------

# Reference

- R4-1711618,   WF on single Tx switched UL,        Apple,     RAN4#84bis meeting

- TR 37.863-01-01_V0.3.0_DC band combinations of LTE 1DL1UL+one NR band

- TS 36.101       User Equipment (UE) radio transmission and reception