3GPP TSG-RAN WG4 Meeting #86	R4-1802209

Athens, Greece, 26th Febuary – 2nd March, 2018

Source: 	Verizon, [Ericsson]

Title: 					NR CA BW Class for mmWave

Agenda Item: 			7.4.4

Document for:	Approval

# 1	Introduction

In previous RAN4 meeting a proposal on NR CA Bandwidth Class definition was discussed and possible FR2 CA BW classes were listed in [1].

In this contribution we present our requirement on FR2 for mmWave system.

# 2	FR2 Class Discussion

For FR2, there is no agreement that UE has to support which NR CA Bandwidth Class from previous RAN4 meeting. Although we have refereed the approach from LTE, all channel bandwidths that are defined for an mmWave band have created difficult to put classes in a CA BW class table.

First, companies are not confident on if it is a realistic approach to limit the class up to 8CC. And, vendors want to verify the restriction of the aggregated CH BW range to 400 MHz carrier from their deployments. For us, we are not sure if the assumption of 100 MHz as mandatory for mmWave has reflected practical requirement in implementation.

To ensure success for development of mmWave from beginning, we propose 50MHz channel bandwidth to FR2 CA bandwidth class in Table 1. We would like RAN4 to consider this requirements in Release 15 specification.

Table 2: FR2 CA BW Class table

| NR CA bandwidth class |  | Aggregated channel bandwidth |  | number of contiguous CC |  |
| --- | --- | --- | --- | --- | --- |
| A |  | CBW ≤ 400 MHz |  | 1 |  |
| B |  | 400 MHz < CBW ≤ 800 MHz |  | 2 |  |
| C |  | 800 MHz < CBW ≤ 1200 MHz |  | 3 |  |
| D |  | 100 MHz < CBW ≤ 200 MHz |  | 2 |  |
| E |  | 200 MHz < CBW ≤ 300 MHz |  | 3 |  |
| F |  | 300 MHz < CBW ≤ 400 MHz |  | 4 |  |
| G |  | 400 MHz < CBW ≤ 500 MHz |  | 5 |  |
| H |  | 500 MHz < CBW ≤ 600 MHz |  | 6 |  |
| I |  | 600 MHz < CBW ≤ 700 MHz |  | 7 |  |
| J |  | 700 MHz < CBW ≤ 800 MHz |  | 8 |  |
| K |  | 200 MHz < CBW ≤ 400 MHz |  | 2 |  |
| L |  | 400 MHz < CBW ≤ 600 MHz |  | 3 |  |
| M |  | 600 MHz < CBW ≤ 800 MHz |  | 4 |  |
| N |  | CBW ≤ 50 MHz |  | 1 |  |
| O |  | 50 MHz < CBW ≤ 100 MHz |  | 2 |  |
| P |  | 100 MHz < CBW ≤ 150 MHz |  | 3 |  |
| Q |  | 150 MHz < CBW ≤ 200 MHz |  | 4 |  |
| R |  | 200 MHz < CBW ≤ 250 MHz |  | 5 |  |
| S |  | 250 MHz < CBW ≤ 300 MHz |  | 6 |  |
| T |  | 300 MHz < CBW ≤ 350 MHz |  | 7 |  |
| U |  | 350 MHz < CBW ≤ 400 MHz |  | 8 |  |

# 3	Conclusion

Out proposal in this contribution is to add 50MHz carrier to FR2 CA BW class definition to ensure success for development of mmWave.

Table 1: FR1 CA BW Class table

| NR CA bandwidth class |  | Aggregated channel bandwidth |  | number of contiguous CC |  |
| --- | --- | --- | --- | --- | --- |
| A |  | CBW ≤ 400 MHz |  | 1 |  |
| B |  | 400 MHz < CBW ≤ 800 MHz |  | 2 |  |
| C |  | 800 MHz < CBW ≤ 1200 MHz |  | 3 |  |
| D |  | 100 MHz < CBW ≤ 200 MHz |  | 2 |  |
| E |  | 200 MHz < CBW ≤ 300 MHz |  | 3 |  |
| F |  | 300 MHz < CBW ≤ 400 MHz |  | 4 |  |
| G |  | 400 MHz < CBW ≤ 500 MHz |  | 5 |  |
| H |  | 500 MHz < CBW ≤ 600 MHz |  | 6 |  |
| I |  | 600 MHz < CBW ≤ 700 MHz |  | 7 |  |
| J |  | 700 MHz < CBW ≤ 800 MHz |  | 8 |  |
| K |  | 200 MHz < CBW ≤ 400 MHz |  | 2 |  |
| L |  | 400 MHz < CBW ≤ 600 MHz |  | 3 |  |
| M |  | 600 MHz < CBW ≤ 800 MHz |  | 4 |  |
| N |  | CBW ≤ 50 MHz |  | 1 |  |
| O |  | 50 MHz < CBW ≤ 100 MHz |  | 2 |  |
| P |  | 100 MHz < CBW ≤ 150 MHz |  | 3 |  |
| Q |  | 150 MHz < CBW ≤ 200 MHz |  | 4 |  |
| R |  | 200 MHz < CBW ≤ 250 MHz |  | 5 |  |
| S |  | 250 MHz < CBW ≤ 300 MHz |  | 6 |  |
| T |  | 300 MHz < CBW ≤ 350 MHz |  | 7 |  |
| U |  | 350 MHz < CBW ≤ 400 MHz |  | 8 |  |

# 4				References

[1] R4-1800065, NR CA BW Class, Nokia, Nokia Shanghai Bell, RAN4#AH1801