3GPP TSG-RAN WG4 Meeting #87                                                                R4-1806321

Busan, KR, 21th – 25th May, 2018

Agenda item:	6.21.4.4

Source:	Intel Corporation

Title:	CQI Reporting requirements with 1024QAM

Document for:	Discussion

- Introduction

In RAN4#86bis demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM were discussed and a way forward [1] was approved. In this contribution we share our views on CQI reporting tests with 1024QAM.

- Discussion

In RAN4#86, the options for CQI reporting tests in static channel with 1024QAM were agreed as [2]:

Option 1: Minimum requirement PUCCH 1-0 (Cell-Specific Reference Symbols): CRS based TM single codeword with TM1 under static channel in Annex B.1

Bandwidth:

FDD: 10MHz

TDD: 20MHz

Option 2: Minimum requirement PUCCH 1-1 (Cell-Specific Reference Symbols): CRS based TM dual codeword with TM4 under static channel in Annex B.1

Bandwidth

FDD: 10MHz

TDD: 20MHz

UE supporting 1024QAM are more likely to support higher number of Rx antennas and higher number of MIMO layers in order to achieve high data rates. In order to test introduce requirements with the new CQI table introduced with 1024QAM and to have sufficient test coverage for UE supporting 1024QAM, we recommend introducing CQI reporting tests with dual codeword in TM4. The CQI reporting test for PUCCH 1-1 in static channel could have the following test parameters. The SNR ranges for Test1 and Test2 need to be selected. The SNR range for Test1 shall be in the low SNR range corresponding to CQI values for 64QAM and the SNR for Test2 shall be selected to correspond to CQI values in 1024QAM range in order to test the new CQI table.

Table 9.2.2.1-1: PUCCH 1-1 static test (FDD)

| Parameter |  | Unit | Test 1 |  | Test 2 |  |
| --- | --- | --- | --- | --- | --- | --- |
| Bandwidth |  | MHz | 10 |  |  |  |
| PDSCH transmission mode |  |  | 4 |  |  |  |
| Downlink power allocation |  | dB | -3 |  |  |  |
|  |  | dB | -3 |  |  |  |
|  |  | dB | 0 |  |  |  |
| Propagation condition and antenna configuration |  |  | Clause B.1 (2 x 2) |  |  |  |
| CodeBookSubsetRestriction bitmap |  |  | 010000 |  |  |  |
| SNR (Note 2) |  | dB | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB[mW/15kHz] | [TBD] | [TBD] | [TBD] | [TBD] |
|  |  | dB[mW/15kHz] | -98 |  | -98 |  |
| Max number of HARQ transmissions |  |  | 1 |  |  |  |
| Physical channel for CQI/PMI reporting |  |  | PUCCH Format 2 |  |  |  |
| PUCCH Report Type for CQI/PMI |  |  | 2 |  |  |  |
| PUCCH Report Type for RI |  |  | 3 |  |  |  |
| Reporting periodicity |  | ms | Npd = 5 |  |  |  |
| cqi-pmi-ConfigurationIndex |  |  | 6 |  |  |  |
| ri-ConfigIndex |  |  | 1 |  |  |  |

Proposal #1: Introduce CQI reporting test for dual codeword in TM4 with 1024QAM

Proposal#2: Select suitable SNR range for CQI test with 1024QAM

- Conclusions

In this paper we present our views on CQI reporting tests with 1024QAM to test the newly introduced CQI tables and propose the following:

Proposal #1: Introduce CQI reporting test for dual codeword in TM4 with 1024QAM

Proposal#2: Select suitable SNR range for CQI test with 1024QAM

- References

- R4-1805959, “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon, Anritsu, Rohde & Schwarz

- R4-1803106 “Way forward on demodulation and CSI requirements for high capacity stationary wireless link and 1024QAM”, Huawei, HiSilicon