3GPP TSG-RAN WG4 Meeting #87	R4-1807744

## Busan, Korea, 21 - 25 May, 2018

Source: 	Huawei

Title:	TP to TS 38.141-1: NR channel numbering correction

Agenda Item:	7.8.3 (Conducted conformance testing (38.141-1))

Document for:	Approval

# Introduction

In this TP to the latest TS 38.141-1 shared on the RAN4 reflector before the meeting, correction of the NR channels numbering is introduced.

The following modifications were introduced in the attached TP:

- 4.5.1.3: application of the NR-EARFCNs numbering. Please note that this abbreviation was already introduced into this specification.

# Conclusions

Based on the above introduction, the following proposals is formulated:

Proposal 1:  approve the attached TP to TS 38.141-1 correcting the NR channel numbering.

# References

[1] 3GPP TS 38.141-1, shared on RAN4 reflector after RAN4#86bis

# TP to TS 38.141-1

------------------------------ Modified section ------------------------------

#### 4.5.1.3	Duplexers

The requirements of the present document shall be met with a duplexer fitted, if a duplexer is supplied as part of the BS. If the duplexer is supplied as an option by the manufacturer, sufficient tests should be repeated with and without the duplexer fitted to verify that the BS meets the requirements of the present document in both cases.

The following tests shall be performed with the duplexer fitted, and without it fitted if this is an option:

1)	subclause 6.2, base station output power, for the highest static power step only, if this is measured at the antenna connector;

2)	subclause 6.6, unwanted emissions; outside the BS transmit band;

3)	subclause 6.6.4.5.3, protection of the BS receiver;

4)	subclause 6.7, transmit intermodulation; for the testing of conformance, the carrier frequencies should be selected to minimize intermodulation products from the transmitters falling in receive channels.

The remaining tests may be performed with or without the duplexer fitted.

NOTE 1:	When performing receiver tests with a duplexer fitted, it is important to ensure that the output from the transmitters does not affect the test apparatus. This can be achieved using a combination of attenuators, isolators and filters.

NOTE 2:	When duplexers are used, intermodulation products will be generated, not only in the duplexer but also in the antenna system. The intermodulation products generated in the antenna system are not controlled by 3GPP specifications, and may degrade during operation (e.g. due to moisture ingress). Therefore, to ensure continued satisfactory operation of a BS, an operator will normally select NR-EARFCNs to minimize intermodulation products falling on receive channels. For testing of complete conformance, an operator may specify the NR-EARFCNs to be used.

------------------------------ End of modified section ------------------------------