3GPP TSG-RAN WG4 Meeting #87                 	R4-1807757

Busan, Korea (Republic of), 21-25 May 2018

Source:	Nokia, Nokia Shanghai Bell

Title:	Simulation results for SPUCCH

Agenda item:	6.22.6.2

Document for:	Discussion

Introduction

In this contribution we provide simulation results for SPUCCH demodulation based on the agreed simulation assumptions in [1].

Simulation assumptions

Simulation assumptions as agreed for SPUCCH in [1] are shown below.

Table 1: Simulation assumption for sPUCCH

| Parameter | Value |
| --- | --- |
| Number of Tx antennas | 1 |
| Number of Rx antennas | 2, 4 |
| Propagation condition | EPA5 |
| Cyclic prefix | Normal |
| Reference receiver | MRC |
| Channel bandwidth | [5 MHz], 10 MHz, 15 MHz, 20 MHz |
| TTI length | Subslot TTI |
| Number of OFDM-symbols | Subslot TTI: 2 and 3 |
| SPUCCH formats | Format 1a, format 4 |
| Number of bits per PRB for format 4 | 3 bits per 2 PRB |

Simulation results

Simulation results for format 1a using the parameters from Table 1 are shown below in Table 2 for 2 OS TTI and in Table 3 for 3 OS TTI.

Table 2: Simulation results for SPUCCH with 2 OS TTI (format 1a).

| Number of Tx Antennas | Number of Rx Antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [dB] with DTXtoACK probability 1 % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 Mhz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | Normal | EPA5 Low | 1.871 | 2.006 | 1.747 | 1.766 |
|  | 4 | Normal | EPA5 Low | -2.033 | -2.005 | -2.056 | -2.128 |

Table 3: Simulation results for SPUCCH with 3 OS TTI (format 1a).

| Number of Tx Antennas | Number of Rx Antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [dB] with DTXtoACK probability 1 % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 Mhz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | Normal | EPA5 Low | 0.966 | 0.987 | 0.729 | 0.733 |
|  | 4 | Normal | EPA5 Low | -2.851 | -2.753 | -2.864 | -2.926 |

Simulation results for format 4 using the parameters from Table 1 are shown below in Table 4 for 2 OS TTI and in Table 5 for 3 OS TTI.

Table 4: Simulation results for SPUCCH with 2 OS TTI (format 1a).

| Number of Tx Antennas | Number of Rx Antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [dB] with DTXtoACK probability 1 % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 Mhz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | Normal | EPA5 Low | 5.396 | 5.613 | 5.533 | 5.52 |
|  | 4 | Normal | EPA5 Low | -0.418 | -0.448 | -0.405 | -0.295 |

Table 5: Simulation results for SPUCCH with 3 OS TTI (format 1a).

| Number of Tx Antennas | Number of Rx Antennas | Cyclic Prefix | Propagation conditions and correlation matrix (Annex B) | Channel Bandwidth / SNR [dB] with DTXtoACK probability 1 % |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  | 5 Mhz | 10 MHz | 15 MHz | 20 MHz |
| 1 | 2 | Normal | EPA5 Low | 3.26 | 3.526 | 3.398 | 3.56 |
|  | 4 | Normal | EPA5 Low | -2.394 | -2.435 | -2.472 | -2.392 |

Conclusion

In this contribution we have presented simulation results for SPUCCH with format 1a and format 4.

- References

- R4-1805495, Simulation assumptions for sPUCCH and sPUSCH, Ericsson, 3GPP TSG-RAN WG4 #86bis, Melbourne, April 2018.