3GPPRAN4#87	R4-1807787

Busan, Korea May 21st- 25th, 2018

Title: 	SCell activation in NR

Source: 	Qualcomm Incorporated

Agenda item:	7.10.10.5 - SCell (de)activation [NR_newRAT-Core]

Document for:	Discussion

# Introduction

In RAN4#86-Bis, a WF for Scell activation timeline [1] was agreed on.  The activation time is split into two parts, 1) time from MAC-CE command to when UE is ready to receive, 2) from UE being ready to receive to CQI reporting back to gNodeB. As part of activation the UE may need to take interruptions on all other active cells, E-UTRA and NR.

# Discussion

## Scell Activation Timeline in E-UTRA

Figure 1 shows the LTE timeline for SCell activation.  From the time UE receives Scell activation command, it has 8 ms to be ready to receive on Scell. After the HARQ ACK for the message has been sent, the UE can take a 1ms interruption on any of the already active cells. Depending on whether the activation is based on the previous SCell measurement or blind, UE has different amount of time to report back CQI. In case of measurement based activation, UE can take up to 16 ms from when it ready to receive. In case of blind activation time to report CQI is 26 ms. Thus, the total time to activation from the time when MAC-CE command is received is 1) 24 ms in case of measurement based activation, 2) 34 ms in case of blind activation.

Figure 1: LTE Scell Activation Timeline

## SCell Activation in NR

The activation timeline starts when the UE receives the command via MAC-CE and is deemed to be complete when UE reports back CQI to gNodeB.

In WF in RAN4#85 we agreed on the following, upon receiving SCG SCell activation command in slot n, the UE shall be capable to transmit valid CSI report and apply actions related to the activation command for the SCell being activated no later than in slot n+ [THARQ + Tactivation_time + TCSI_Reporting]

In WF [1], the following agreement was reached

- Tactivation_time is consisted of

  - RF warm up excluding AGC settling: TBD and independent of SCS

  - AGC settling: N1 SMTC periods for known and N2 SMTC periods for unknown cells

  - PSS/SSS and SSB index acquiring:

    - X1 SMTC periods for known cell

    - X2 SMTC period for unknown cell

    - X1 and X2 will be decided by RAN4#86

- In addition, companies to investigate the time needed to MAC-CE decode.

In this paper we provide our view on the above times.

The HARQ feedback time, THARQ, would depend on configured K1 for the slot in which the MAC-CE command is received.  In addition to providing HARQ feedback, the UE would need to decode the MAC-CE message and SW would then need to program both baseband and RF. For MAC-CE message decode, RAN1 has the following agreement.

“Agreement The latency for MAC CE activation is 12 slots for 60kHz SCS and 24 slots for 120kHz SCS. The latency is calculated from the ending symbol of UE’s ACK transmission for PDSCH carrying MAC-CE activation message to when the MAC CE message is applied.”

The above agreement implies that the UE would need 3ms in FR2 for MAC-CE message decode and application. We propose to use the same number for FR1 also.

Proposal 1: The time needed by UE for MAC-CE message decode and application is 3ms for both FR1 and FR2.

Proposal 2: The UE can take interruptions on active carriers between the time it provides HARQ feedback to network and before the UE is ready to receive on the to be activated SCell.

Proposal 3: RF warmup time for UE after application of MAC-CE command can be up to 0.5 ms.

Once the UE is ready to receive, it will then need time to set AGC, acquire timing and report back CQI. For these purposes it would it either need to wait for SS blocks or the network would need to provide TRS. In the following discussion we concentrate on the number of samples needed for SCell activation. Except for the first sample in case of unknown cell which needs to be an SSB to acquire timing, the rest of these could be either SSB or TRS. Also, in all types of scenarios the UE would need 1 slot CSI-RS to report back CQI

Proposal 4: The number of samples (SSB or TRS) needed for SCell activation is given as follows

| Scenario | Number of samples | Notes |
| --- | --- | --- |
| Known cell (measured within 160 ms) | 1 |  |
| Known cell (measured longer than 160ms) | 2 |  |
| Unknown cell | 3 | The first sample needs to SSB |

Note that the UE would need at least 2 slots between reception of consecutive samples for processing.

For CQI feedback, RAN1 is still discussing CSI feedback time. For Scell activation purposes we need the aperiodic CSI feedback time. The agreements will be captured in Section 5.4 of 38.214

Proposal 5: The UE would need 1 slot CSI-RS to report back CQI. The time to report back CQI is being discussed in RAN1

# Conclusions

In this paper we provide a proposal for NR SCell activation. The activation timeline in NR is longer than E-UTRA if we base it solely on SSB. For this purpose, we request the network to provide aperiodic TRS which will significantly shorten the timeline.

Proposal 1: The time needed by UE for MAC-CE message decode and application is 3ms for both FR1 and FR2.

Proposal 2: The UE can take interruptions on active carriers between the time it provides HARQ feedback to network and before the UE is ready to receive on the to be activated SCell.

Proposal 3: RF warmup time for UE after application of MAC-CE command can be up to 0.5 ms.

Proposal 4: The number of samples (SSB or TRS) needed for SCell activation is given as follows

| Scenario | Number of samples | Notes |
| --- | --- | --- |
| Known cell (measured within 160 ms) | 1 |  |
| Known cell (measured longer than 160ms) | 2 |  |
| Unknown cell | 3 | The first sample needs to SSB |

Proposal 5: The UE would need 1 slot CSI-RS to report back CQI. The time to report back CQI is being discussed in RAN1

# References

[1] R4-1805968, “Way forward on Scell activation delay”, RAN4#86-Bis, Melbourne, Australia.