3GPP TSG-RAN WG4 RAN4-88	R4-1809926

Gothenburg, Sweden, 20 - 24 August, 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.113 | CR |  | rev | - | Current version: | 15.2.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR to TS 38.113: BS Test Configurations |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Ericsson |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-08-20 |
| Category: | B |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | In TS 38.113, section 4.5 requires the definition of BS Test Configurations. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Definition of BS test configurations for EMC testing. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | EMC testing can be affected due to the lack of definition in the BS test configurations. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 4.5 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

- ------------------------------ Start of Text ------------------------------

## 4.5	BS test configurations

The present subclause defines the BS test configurations that shall be used for demonstrating conformance. A single NR carrier shall be used for testing of single-carrier capable BS. For other NR base stations, the test configurations in table 4.5-1 and table 4.5-2 shall be used. The NR test configurations (NRTCx) are defined for BS type 1-C and BS type 1-H in TS 38.141-1 [3], subclause 4.7 and for BS type 1-O and BS type 2-O  in TS 38.141-2 [4], subclause 4.8.

Table 4.5-1: Test configurations for BS type 1-C and BS type 1-H (According to TS 38.141-1 [3])

| BS test case | BS capable of multi-carrier and/or CA operation in contiguous spectrum in single band only | BS capable of multi-carrier and/or CA operation in both contiguous and non-contiguous spectrum in single band | BS capable of multi-band operation |
| --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 | NRTC1/3 (Note 1), NRTC5 |
| Immunity tests | NRTC1 | NRTC3 | NRTC1/3 (Note 1), NRTC5 |
| - NOTE 1: NRTC1 or NRTC3 shall be applied in each supported operating band according to the respective capability in each band, as defined in the 2nd and 3rd column of the table. |  |  |  |

Table 4.5-2: Test configurations for BS type 1-O and BS type 2-O (According to TS 38.141-2 [4])

| BS test case | BS capable of multi-carrier and/or CA operation in contiguous spectrum in single band only | BS capable of multi-carrier and/or CA operation in both contiguous and non-contiguous spectrum in single band | BS capable of multi-band operation |
| --- | --- | --- | --- |
| Emission tests | NRTC1 | NRTC3 | NRTC1/3 (Note 2), NRTC5 |
| Immunity tests | NRTC1 | NRTC3 | NRTC1/3 (Note 2), NRTC5 |
| - NOTE 2: NRTC1 or NRTC3 shall be applied in each supported operating band according to the respective capability in each band, as defined in the 2nd and 3rd column of the table. |  |  |  |

------------------------------ End of proposed text ------------------------------