3GPP TSG-WG RAN4 Meeting #88	R4-1810804

# Gothenburg, Sweden, 20th – 24th August, 2018

Source: 	Nokia, Nokia Shanghai Bell, [T-Mobile USA]

Title: 					TP for TR 37.716-21-11 Introduction of DC_2A-66A_n261A, DC_2A-66A_n261(2A), DC_2A-66A_n257A and DC_2A-66A_n257(2A)

Agenda Item: 			9.4.3 DC_R16_2BLTE_1BNR_3DL2UL-Core

Document for:	Approval

# Introduction

This TP introduces the following three-band EN-DC configurations to TR 37.716-21-11.

- DL_2A-66A_n257(2A)_UL_2A_n257A

- DL_2A-66A_n257A_UL_2A_n257A

- DL_2A-66A_n257(2A)_UL_66A_n257A

- DL_2A-66A_n257A_UL_66A_n257A

- DL_2A-66A_n261(2A)_UL_2A_n261A

- DL_2A-66A_n261A_UL_2A_n261A

- DL_2A-66A_n261(2A)_UL_66A_n261A

- DL_2A-66A_n261A_UL_66A_n261A

**************************** Start of changes ********************************************

## 6.X	DC_2-66_n257

### 6.X.1	Operating bands for DC

Table 6.X.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_n257 | CA_2-66 | n257 | No |

6.X.2	Configurations for DC

Table 6.X.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n257A DC_2A-66A_n257(2A) | DC_2A_n257A DC_66A_n257A | CA_2A-66A | n257A CA_n257(2A) |

Table 6.X.2-2: Supported bandwidths per DC band combination of 2 LTE bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_2A-66A_n257A | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 |  |  |  |  |  | Yes |  |  | Yes | Yes |  | 240 |  |  |
|  |  | 120 |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes | 440 |  |  |
| DC_2A-66A_n257(2A) | 2 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes | Yes |  |  |  |  |  |  |  |  |  |  |
|  | n257 | 60 | See CA_n257(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  | 440 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |

### 6.X.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 2 and n257 has has been already made in clause 6.86.3 of TS 37.863-01-01.

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n257 has has been already made in clause 6.62.3 of TS 37.863-01-01.

Therefore, we skip these analysis in this TR.

Table 6.X.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.X.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2-66_n257 | Band 2, 4, 5, 7, 10, 12, 13, 14, 17, 24, 25, 26, 27, 28, 29, 30, 38, 41, 43, 50, 51, 66, 70, 71, 74 | FDL_low | - | FDL_high | -50 | 1 |  |
|  | Band 42, 48 | FDL_low | - | FDL_high | -50 | 1 | 1 (Note) |
| Note: Table 6.5.3.2-1 of TS38.101 |  |  |  |  |  |  |  |

### 6.X.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.X.5	∆TIB and ∆RIB values

For DC_2-66-n257, the TIB,c and RIB values are given in the tables below, reusing LTE CA configuration CA_2-66 for band 2 and 66 while no relaxation is introduced to band n257.

Table 6.X.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n257 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n257 | 0 |

Table 6.X.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n257 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n257 | 0 |

## 6.Y	DC_2-66_n261

### 6.Y.1	Operating bands for DC

Table 6.Y.1-1: Band combinations EN-DC (three bands)

| EN-DC Band | E-UTRA Band | NR Band | Single UL allowed |
| --- | --- | --- | --- |
| DC_2-66_ n261 | CA_2-66 | n261 | No |

### 6.Y.2	Configuration for DC

Table 6.Y.2-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration | E-UTRA configuration | NR configuration |
| --- | --- | --- | --- |
| DC_2A-66A_n261A DC_2A-66A_n261(2A) | DC_2A_n261A DC_66A_n261A | CA_2A-66A | n261A CA_n261(2A) |

Table 6.Y.2-2: Supported bandwidths per DC band combination of 2 LTE bands and 1 NR band

| DC operating band / channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| E-UTRA and NR DC Configuration | E-UTRA and NR Band | Subcarrier spacing [kHz] | 5 MHz | 10 MHz |  | 15 MHz | 20 MHz | 40 MHz | 50 MHz | 60 MHz | 80 MHz | 100 MHz | 200 MHz |  |  | 400 MHz | Maximum aggregated bandwidth [MHz] |  |  |
| DC_2A-66A_n261A | 2 | 15 | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes |  | Yes | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 |  |  |  |  |  |  | Yes |  |  | Yes | Yes |  |  |  | 240 |  |  |
|  |  | 120 |  |  |  |  |  |  | Yes |  |  | Yes | Yes | Yes |  |  | 440 |  |  |
| DC_2A-66A_n261(2A) | 2 | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | 66 | 15 | Yes | Yes | Yes |  | Yes |  |  |  |  |  |  |  |  |  |  |  |  |
|  | n261 | 60 | See CA_n261(2A) CA bandwidth combinations in TS 38.101-2. |  |  |  |  |  |  |  |  |  |  |  |  |  | 440 |  |  |
|  |  | 120 |  |  |  |  |  |  |  |  |  |  |  |  |  |  | 840 |  |  |

### 6.Y.3	Spurious emission band UE co-existence for DC

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 2 and n261 has has been already made in TR 37.716-01-01.

The co-existence analysis of harmonics and 2UL intermodulations for DC bands 66 and n261 has has been already made in clause 6.93.3 of TS 37.863-01-01.

Therefore, we skip these analysis in this TR.

Table 6.Y.3-3 lists the protected bands required for the dual connectivity configuration.

Table 6.Y.3-3: Protected bands for the dual connectivity configuration

| E-UTRA and NR DC Configuration | Spurious emission |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Protected band | Frequency range (MHz) |  |  | Maximum Level (dBm) | MBW (MHz) | NOTE |
| DC_2A-66A_n261A DC_2A-66A_n261(2A) | Bands 4, 5, 12, 13, 14, 17, 24, 26, 29, 30, 41, 48, 66, 70, 71, n260 | FDL_low | - | FDL_high | -50 | 1 |  |

### 6.Y.4	MSD analysis for DC

No IMD issues are expected for DC configuration between the LTE bands below 6GHz and FR2 NR band. Furthermore, up to 7th order harmonics have no impact to Rx frequency of both LTE and NR bands.

### 6.Y.5	∆TIB and ∆RIB values

For DC_2-66-n261, the TIB,c and RIB values are given in the tables below, reusing LTE CA configuration CA_2-66 for band 2 and 66 while no relaxation is introduced to band n261.

Table 6.Y.5-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.5 |
|  | 66 | 0.5 |
|  | n261 | 0 |

Table 6.Y.5-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_2-66_n261 | 2 | 0.3 |
|  | 66 | 0.3 |
|  | n261 | 0 |

**************************** End of changes ********************************************