3GPP TSG-RAN WG4 Meeting #88	R4-1811362

Gothenburg, Sweden, 20th- 24th August, 2018

Agenda item:		7.12.3.4

Source:	MediaTek Inc.

Title:	Discussion on RRM measurement accuracy performance

Document for:		Discussion

1	Introduction

RAN4 is going to define the NR RRM testing on UE measurement accuracy performance. In this paper, we discuss the values of key parameters that can be leveraged from LTE and the differences between LTE and NR testing.

2	Discussion

In TS 36.133, the key parameters in test case “FDD Intra frequency case” are:

- Noc

- Es/Noc

- Io

Among them, AWGN level Noc is the first value we need to decide. Based on the CR provided by Anritsu [1], this value is set considering the PRB number NRB used in this channel BW, the largest positive signal to noise ratio value, and the most negative signal to noise ratio value. The values of Noc in LTE test cases are shown as follows:

Table A.9.1.1.2-1: RSRP FDD Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Note2 | Bands FDD_A Note 8 | dBm/15 kHz | -106 | -106 | -88 | -88 | -116 |  |
|  | Bands FDD_B |  |  |  |  |  | -115.5 |  |
|  | Bands FDD_C |  |  |  |  |  | -115 |  |
|  | Bands FDD_D |  |  |  |  |  | -114.5 |  |
|  | Bands FDD_E, FDD_F Note 5 |  |  |  |  |  | -114 |  |
|  | Bands FDD_G Note 7 |  |  |  |  |  | -113 |  |
|  | Bands FDD_H |  |  |  |  |  | -112.5 |  |

Here, the Noc in Test 3 is set as the 5dB margin plus the minimum Io (Unit dBm/15kHz) values in Table 9.1.2.1-1: RSRP Intra frequency absolute accuracy. In [2], the NR operating band groups has been captured in the measurement accuracy table. We fill in the minimum Io values based on Table 7.3.2-1 in TS 38.101-1 [3]:

Table 10.1.2.1.1-1: SS RSRP Intra frequency absolute accuracy in FR1

| Accuracy |  | Conditions |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Normal condition | Extreme condition | Ês/Iot | Io Note 1 range |  |  |  |  |
|  |  |  | NR operating band groups | Minimum Io |  |  | Maximum Io |
| dB | dB | dB |  | dBm / SCSSSB |  | dBm/BWChannel | dBm/BWChannel |
|  |  |  |  | SCSSSB = 15 kHz | SCSSSB = 30 kHz |  |  |
| [4.5] | [9] | [-6] dB | NRFDD_FR1_A, NRTDD_FR1_A | [-122] | [-119] | N/A | -70 |
|  |  |  | NRFDD_FR1_B | [-121.5] | [-118.5] | N/A | -70 |
|  |  |  | NRTDD_FR1_C | [-121] | [-118] | N/A | -70 |
|  |  |  | NRFDD_FR1_E, NRTDD_FR1_E | [-120] | [-117] | N/A | -70 |
|  |  |  | NRFDD_FR1_G | [-119.5] | [-116.5] | N/A | -70 |
|  |  |  | NRFDD_FR1_H | [-119] | [-116] | N/A | -70 |
| [8] | [11] | [-6] dB | All | N/A | N/A | -70 | -50 |
| NOTE 1: Io is assumed to have constant EPRE across the bandwidth. |  |  |  |  |  |  |  |

Besides, the maximum transmission bandwidth configuration NRB for each UE channel bandwidth and subcarrier spacing is specified in Table 5.3.2-1 [3].

Table 5.3.2-1: Maximum transmission bandwidth configuration NRB

| SCS (kHz) | 5MHz | 10MHz | 15MHz | 20 MHz | 25 MHz | 30 MHz | 40 MHz | 50MHz | 60 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 25 | 52 | 79 | 106 | 133 | 160 | 216 | 270 | N/A | N/A | N/A | N/A |
| 30 | 11 | 24 | 38 | 51 | 65 | 78 | 106 | 133 | 162 | 217 | 245 | 273 |
| 60 | N/A | 11 | 18 | 24 | 31 | 38 | 51 | 65 | 79 | 107 | 121 | 135 |

We then calculate the Noc and corresponding Io values in SCS 15kHz and 30kHz based on the assumption that  uses the same values as LTE:

Table 1: SS-RSRP Intra frequency test parameters in SCS 15kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| BWchannel |  | MHz | 10 |  | 10 |  | 10 |  |
| Note2 | NRFDD_FR1_A | dBm/15 kHz | [-106 ] | [-106] | [-88] | [-88] | [-117] |  |
|  | NRFDD_FR1_B |  |  |  |  |  | [-116.5] |  |
|  | NRTDD_FR1_C |  |  |  |  |  | [-116] |  |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-115] |  |
|  | NRFDD_FR1_G |  |  |  |  |  | [-114.5] |  |
|  | NRFDD_FR1_H |  |  |  |  |  | [-114] |  |
|  |  | dB | [2.5] | [-6] | [2.5] | [-6] | [0.46] | [-5.76] |
| RSRPNote3 | NRFDD_FR1_A | dBm/15 kHz | [-100] | [-105] | [-82] | [-87] | [-114] | [-118] |
|  | NRFDD_FR1_B |  |  |  |  |  | [-113.5] | [-117.5] |
|  | NRTDD_FR1_C |  |  |  |  |  | [-113] | [-117] |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-112] | [-116] |
|  | NRFDD_FR1_G |  |  |  |  |  | [-111.5] | [-115.5] |
|  | NRFDD_FR1_H |  |  |  |  |  | [-111] | [-115] |
| IoNote3 | NRFDD_FR1_A | dBm/9.36 MHz | [-70.09] | [-70.09] | [-52.10] | [-52.10] | [-83.26] |  |
|  | NRFDD_FR1_B |  |  |  |  |  | [-82.76] |  |
|  | NRTDD_FR1_C |  |  |  |  |  | [-82.26] |  |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-81.26] |  |
|  | NRFDD_FR1_G |  |  |  |  |  | [-80.76] |  |
|  | NRFDD_FR1_H |  |  |  |  |  | [-80.26] |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |

We then further evaluate the range of Io values in SCS 15kHz:

- For the maximum scaling at full RB allocation:  
10log10(270RBs x 12 subcarriers) = +35.1dB.

- For minimum channel bandwidth:
10log10(25RBs x 12 subcarriers) = +24.77dB.

Assume the SNR range [-3.4dB, 25.3dB], the maximum level and minimum level would be

- The maximum level:  
Noc (max)+ 25.3dB + 35.1dB = -88 + 60.4 dBm = -27.6 dBm/ Ch BW 
(LTE maximum input level -25dBm)

- The minimum level:
Noc (min)– 3.4dB + 24.77dB = -122 + 24.37 dBm = -97.63 dBm/ Ch BW

(LTE REFSENS level of -103.2dBm for 1.4MHz)

Table 2: SS-RSRP Intra frequency test parameters in SCS 30kHz

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| BWchannel |  | MHz | 40 |  | 40 |  | 40 |  |
| Note2 | NRFDD_FR1_A | dBm/30 kHz | [-103 ] | [-103] | [-85] | [-85] | [-114] |  |
|  | NRFDD_FR1_B |  |  |  |  |  | [-113.5] |  |
|  | NRTDD_FR1_C |  |  |  |  |  | [-113] |  |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-112] |  |
|  | NRFDD_FR1_G |  |  |  |  |  | [-111.5] |  |
|  | NRFDD_FR1_H |  |  |  |  |  | [-111] |  |
|  |  | dB | [2.5] | [-6] | [2.5] | [-6] | [0.46] | [-5.76] |
| RSRPNote3 | NRFDD_FR1_A | dBm/30 kHz | [-97] | [-102] | [-79] | [-84] | [-111] | [-115] |
|  | NRFDD_FR1_B |  |  |  |  |  | [-110.5] | [-114.5] |
|  | NRTDD_FR1_C |  |  |  |  |  | [-110] | [-114] |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-109] | [-113] |
|  | NRFDD_FR1_G |  |  |  |  |  | [-108.5] | [-112.5] |
|  | NRFDD_FR1_H |  |  |  |  |  | [-108] | [-112] |
| IoNote3 | NRFDD_FR1_A | dBm/38.16 MHz | [-64] | [-64] | [-46] | [-46] | [-77.17] |  |
|  | NRFDD_FR1_B |  |  |  |  |  | [-76.67] |  |
|  | NRTDD_FR1_C |  |  |  |  |  | [-76.17] |  |
|  | NRFDD_FR1_E, |  |  |  |  |  | [-75.17] |  |
|  | NRFDD_FR1_G |  |  |  |  |  | [-74.67] |  |
|  | NRFDD_FR1_H |  |  |  |  |  | [-74.17] |  |
|  |  | dB | 6 | 1 | 6 | 1 | 3 | -1 |

We then further evaluate the range of Io values in SCS 30kHz:

- For the maximum scaling at full RB allocation:  
10log10(273RBs x 12 subcarriers) = +35.15dB.

- For minimum channel bandwidth:
10log10(24RBs x 12 subcarriers) = +24.6dB.

Assume the SNR range [-3.4dB, 25.3dB], the maximum level and minimum level would be

- The maximum level:  
Noc (max)+ 25.3dB + 35.15dB = -85 + 60.45 dBm = -24.55 dBm/ Ch BW

- The minimum level:
Noc (min)– 3.4dB + 24.6dB = -119 + 21.2 dBm = -97.8 dBm/ Ch BW

Based on above evaluation, the Noc value needs to be carefully set, otherwise it can’t have enough headroom for the largest positive signal to noise ratio value or left enough margin for the most negative signal to noise ratio value.

Proposal 1: RAN4 to check if AWGN level Noc should be revised, when difference of PRB number and SCS are taken into account .

Finally, in NR there is no CRS for UE to refer the EPRE value. We can only use EPRE of SSS as the power reference. The RS parameters in NR are also quite different with that in LTE. This changes should also be updated in the Tables of test case.

Proposal 2: Use EPRE of SSS as the reference power and specify the power offset of PBCH data, PBCH DMRS, and PSS in test cases of UE measurement accuracy performance.

4	Summary

In this contribution, we propose

Proposal 1: RAN4 to check if AWGN level Noc should be revised, when difference of PRB number and SCS are taken into account .

Proposal 2: Use EPRE of SSS as the reference power and specify the power offset of PBCH data, PBCH DMRS, and PSS in test cases of UE measurement accuracy performance.

5	Reference

[1] R4-090445, AWGN level for UE DL demodulation performance tests, Anritsu, Feb. 2009.

[2] R4-180XXXX, CR to TS38.133, Intel, Jul. 2018.

[3] TS 38.101-1