3GPP TSG RAN WG4 Meeting #88bis	R4-1812006

Chengdu, China, 8th – 12th October, 2018

3GPP TSG RAN WG1 Meeting #94			R1-1809992

Gothenburg, Sweden, August 20th – 24th, 2018

Title:	LS on NR-LTE Co-existence

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG1

Response to:	-

To:	RAN WG2, RAN WG4

Cc:

Contact Person:

#### Name:	Hong He

Tel. Number:

E-mail Address:	hong.he (at) intel.com

#### Name:	Young Tae Kim

Tel. Number:

E-mail Address:	youngtae99.kim (at) lge.com

Attachments:	None

1. Overall Description:

RAN WG1 discussed the phase discontinuity issue for a single PA structure and other issues for NR-LTE coexistence and made the following agreements:

Agreement:

UE capability signalling that indicates per band per band combination whether the UE can handle (motivated by impacts of PA phase discontinuity) cases with overlapping transmissions with non-aligned starting or ending times or hop boundaries across carriers is beneficial and is introduced.

- Only applicable to intra-band EN-DC, intra-band CA, FDM-based ULSUP (UL Sharing from UE Perspective)

  - Note: Normal UE behaviour is assumed when the UE does not indicate the lack of this capability through the capability signalling or when the capability is not applicable

- RAN4 to determine whether a UE that cannot handle such cases needs different performance requirements or whether these cases are treated as error cases.

- For intra-band EN-DC and FDM-based ULSUP, whether the capability is applicable also when NR is operated with different numerology than LTE should be determined in RAN4

- RAN4 to determine in which bands, in which band-combinations (including in which bands in the band-combination) the capability is applicable

  - Note: This is not intended to change other UE capabilities and minimum performance requirements related to the number of antennas and PAs as per the RAN4 requirements.

Agreement:

A UE is not expected to be configured with sTTI and/or shortened processing time operation on a carrier that is part of an EN-DC configuration in Rel-15.

2. Actions:

To RAN WG2, RAN WG4:

ACTION: 	RAN WG1 respectfully asks RAN WG2 and RAN WG4 to take the above RAN WG1 agreements into account.

3. Date of Next TSG-RAN WG1 Meetings:

# TSG RAN1 #94bis		8 - 12 Oct 2018			Chengdu, China

# TSG RAN1 #95			12 Nov - 16 Nov 2018		Spokane, WA, US