3GPP TSG-RAN WG4 Meeting RAN4#88bis                                               R4-18Xxxxxx

Chengdu ,China, 8 -12 Oct, 2018

Agenda Item:   7.13.1.4

Source:        Samsung

Title:          Simulation results for RI test cases

Document for:  Discussion

# Introduction

In last RAN4 meeting, list of test cases has been captured in WF [1]:

- Static CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband fading CQI test cases - Test 1: FR1 FDD 10MHz &15kHz , 2*2, 2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 2*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - Wideband PMI test cases - Test 1: FR1 FDD 10MHz &15kHz , 8*2, 8*4 - Test 2: FR1 TDD 40MHz&30kHz, 8*2, 8*4 - Test 3: FR2 100MHz& 120kHz, 2*2 - 4Tx PMI test cases (Wideband) - Test 1: FR1 FDD 10MHz &15kHz , 4*2, 4*4 - Test 2: FR1 TDD 40MHz&30kHz, 4*2, 4*4 - Sub-band CQI test cases (2nd priority) - Test 1: FR1 FDD 10MHz&15kHz, 2*2,2*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2,2*4 - RI test - Test 1: FR1 FDD 10MHz&15kHz, 2*2,4*4 - Test 2: FR1 TDD 40MHz&30kHz, 2*2, 4*4 - Test 3: FR2 100MHz&120kHz, 2*2

In this contribution, initial simulation results for RI test cases provided.

# Discussion

## Simulation assumption

| Parameter | Unit | Test 1 (FR1 FDD) | Test 2 (FR1 FDD) | Test 3 FR2 |
| --- | --- | --- | --- | --- |
| System parameter |  |  |  |  |
| Bandwidth/SCS | MHz/kHz | 10MHz /15kHz | 40MHz/30kHz | 100MHz/120kHz |
| Transmission Scheme |  | 1 | 1 | 1 |
| Duplex Mode |  | FDD | TDD | TDD |
| DLUL configuration |  | N.A | 7D1S2U | DDDSU |
| Propagation channel |  | TDL_A 30ns, 5Hz | TDL_A 30ns, 10Hz | TDL_A 30ns, 75Hz |
| Correlation and antenna configuration |  | 2Rx 2*2 4Rx: 4*4 [ULA Low ULA High] | 2Rx 2*2 4Rx: 4*4 [ULA Low ULA High] | 2Rx 2*2 [ULA Low, ULA High] |
| CSI-RS resource setting (periodic) |  |  |  |  |
| resourceConfigType |  | Periodic | Periodic | Periodic |
| CSI-RS/CSI-IM periodicity and slot offset |  | 5 slots, 1 slot offset | 5slots, 1 slot offset | 5slots, 1 slot offset |
| NZP CSI-RS ports |  | 2/4 | 2/4 | 2 |
| NZP CSI-RS configuration CDM type/CSIRS resource mapping/CSI-RS-Density |  | 2 ports: [FD-CDM2/density 1] 4 ports: [FD-CDM2 density 1 (row 4)] | 2 ports: [FD-CDM2/density 1] 4 ports: [FD-CDM2 density 1 (row 4)] | [FD-CDM2/density 1] |
| CSI-IM configuration |  | FFS | FFS | FFS |
| CSI report setting (periodic) |  |  |  |  |
| reportConfigType |  | periodic | periodic | periodic |
| Reporting periodicity and slot offset |  | FFS | FFS | FFS |
| Codebook Restriction |  | FFS | FFS | FFS |
| Reporting granularity |  | Wide-band | Wide-band | Wide-band |
| Physical channel for reporting |  | PUSCH | PUSCH | PUSCH |
| PUCCH reporting format |  | FFS | FFS | FFS |
| CQI table |  | Table 2 with 256QAM | Table 2 with 256QAM | Table 1 without 256QAM |

## Simulation results

Simulation results for FR1 FDD

Table 1 RI test with 2*2

| SNR | 2*2 Low |  | 2*2 High |  |
| --- | --- | --- | --- | --- |
|  | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 | Follow RI/Fixed RI=1 | Follow RI/Fixed RI=2 |
| 0 | 1 | 1.13 | 1 | 1.91 |
| 1 | 1.00 | 1.21 | 1.00 | 2.04 |
| 2 | 1.03 | 1.24 | 1.00 | 2.20 |
| 3 | 1.02 | 1.15 | 1.00 | 2.32 |
| 4 | 1.04 | 1.12 | 1.00 | 2.45 |
| 5 | 1.04 | 1.10 | 1.00 | 2.62 |
| 6 | 1.02 | 1.11 | 1.00 | 2.77 |
| 7 | 1.03 | 1.13 | 1.00 | 2.91 |
| 8 | 1.01 | 1.06 | 1.00 | 2.90 |
| 9 | 1.04 | 1.06 | 1.00 | 2.79 |
| 10 | 1.05 | 1.03 | 1.00 | 2.65 |
| 11 | 1.02 | 0.97 | 1.00 | 2.58 |
| 12 | 1.05 | 1.03 | 1.00 | 2.53 |
| 13 | 1.05 | 1.03 | 1.00 | 2.39 |
| 14 | 1.06 | 1.06 | 1.00 | 2.23 |
| 15 | 1.03 | 0.96 | 1.00 | 2.14 |
| 16 | 1.08 | 1.05 | 1.00 | 2.08 |
| 17 | 1.12 | 1.02 | 1.00 | 2.02 |
| 18 | 1 | 1.02 | 1 | 1.97 |
| 19 | 1 | 1.01 | 1 | 1.92 |
| 20 | 1.16 | 0.99 | 1.00 | 1.84 |
| 21 | 1.20 | 1.02 | 1.00 | 1.87 |
| 22 | 1.22 | 1.01 | 1.00 | 1.80 |
| 23 | 1.26 | 1.01 | 1.00 | 1.69 |
| 24 | 1.24 | 1.07 | 1.00 | 1.60 |

Table 2 RI test with 4*4

| SNR [dB] | 4*4 Low |  |  |  | 4*4 High |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | RI=1 | RI=2 | RI=3 | RI=4 | RI=1 | RI=2 | RI=3 | RI=4 |
| 0 | 1 | 0.96 | 1.35 | 1.05 | 1 | 3.80 | 3.79 | 27.14 |
| 1 | 0.81 | 0.93 | 1.18 | 1.07 | 0.93 | 3.24 | 3.81 | 28.90 |
| 2 | 0.85 | 0.95 | 1.01 | 1.22 | 0.90 | 2.85 | 3.74 | 28.90 |
| 3 | 0.87 | 0.90 | 0.89 | 1.28 | 0.91 | 2.63 | 3.80 | 28.01 |
| 4 | 0.94 | 0.85 | 0.94 | 1.08 | 0.87 | 2.32 | 3.61 | 23.42 |
| 5 | 1.03 | 0.93 | 1.14 | 0.99 | 0.88 | 2.23 | 3.57 | 20.30 |
| 6 | 1.06 | 0.90 | 1.19 | 0.97 | 0.84 | 2.12 | 3.41 | 16.13 |
| 7 | 1.04 | 0.89 | 1.02 | 1.02 | 0.84 | 2.11 | 3.35 | 13.84 |
| 8 | 1.07 | 0.89 | 0.94 | 1.14 | 0.83 | 2.04 | 3.35 | 12.18 |
| 9 | 1.05 | 0.91 | 0.91 | 1.23 | 0.83 | 2.04 | 3.37 | 10.57 |
| 10 | 1.10 | 0.91 | 0.93 | 1.14 | 0.81 | 1.98 | 3.35 | 8.63 |
| 11 | 1.10 | 0.91 | 0.94 | 1.01 | 0.84 | 2.04 | 3.59 | 7.05 |
| 12 | 1.14 | 0.89 | 1.00 | 0.97 | 0.83 | 2.07 | 3.81 | 5.80 |
| 13 | 1.10 | 0.82 | 1.04 | 0.91 | 0.88 | 2.21 | 4.21 | 5.06 |
| 14 | 1.19 | 0.88 | 1.01 | 1.00 | 0.86 | 2.25 | 4.53 | 4.36 |
| 15 | 1.27 | 0.90 | 0.98 | 1.17 | 0.89 | 2.17 | 4.96 | 4.10 |
| 16 | 1.36 | 0.95 | 0.98 | 1.37 | 0.91 | 2.12 | 5.25 | 3.94 |
| 17 | 1.39 | 0.96 | 0.98 | 1.56 | 0.90 | 2.05 | 5.42 | 3.94 |
| 18 | 1 | 0.98 | 0.95 | 1.65 | 1 | 1.90 | 5.52 | 4.10 |
| 19 | 1 | 1.06 | 1.00 | 1.75 | 1 | 1.75 | 5.50 | 4.22 |
| 20 | 1.46 | 1.05 | 0.95 | 1.73 | 0.93 | 1.63 | 5.38 | 4.29 |
| 21 | 1.35 | 0.97 | 0.92 | 1.60 | 0.92 | 1.55 | 5.07 | 4.37 |
| 22 | 1.20 | 0.86 | 0.89 | 1.38 | 0.91 | 1.47 | 4.91 | 4.52 |
| 23 | 1.11 | 0.76 | 0.83 | 1.21 | 0.90 | 1.38 | 4.55 | 4.73 |
| 24 | 1.32 | 0.91 | 0.93 | 1.38 | 0.88 | 1.31 | 4.31 | 5.37 |

# Conclusion

In this contribution, we provided initial simulation results for RI test cases.

# Reference

[1] R4-1811693, “Way forward on NR UE CSI requirements”, Samsung