3GPP TSG-RAN4 Meeting #89 	R4-1815684

# Spokane, US, 12-16 November 2018

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.817-02 | CR | 0026 | rev | - | Current version: | 15.1.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network | X | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

raft

| Title: | CR to TR 38.817-02: Classification of radiated Tx requirements (9.1) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | NEC |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  |  | Date: |  | 2018-11-02 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | FFS still remains for the text on the co-location requirements. OTA transient period for FR1 is clasified as “directional requirement”. However, it is not true. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Added text for co-location requirements classification. Corrected the classification of OTA transient period for FR1 from “directional” to “co-location” and modified the description for the requirement. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | FFS still remains for the text on the co-location requirements OTA transient period for FR1 is wrongly classified. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | 9.1.1 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | x | Other core specifications |  |  | TS/TR ... CR ... |  |  |  |
| affected: |  |  | X | Test specifications |  |  | TS/TR ... CR ... |  |  |  |
| (show related CRs) |  |  | x | O&M Specifications |  |  | TS/TR ... CR ... |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

--------------Start of change-------------

9	Radiated BS transmitter characteristics

9.1	General

9.1.1 	Spatial definitions (FR1)

FR1 NR BS will use the same spatial definitions as AAS BS, these are further described in 3GPP TR 37.842 [8] and 3GPP TR 37.843 [9].

OTA transmitter requirements can be split into either:

1.	Directional requirements

-	The manufacturer to declare beam(s) and coverage ranges over which the beam can be steered.

-	Directional requirement type does not imply the requirement is only in one direction as many requirements have a number of compliance directions. It implies the requirement applies to a single direction at a time.

2.	TRP requirements

-	TRP is defined as:

,where EIRP is the total EIRP of two orthogonal polarizations.

3.	Co-location requirements

-	Co-location requirements are specified as power levels into or out of the conducted interface of the co-location reference antenna. For conformance testing the requirements are translated to the output of either a co-location test antenna (CLTA) or a standard test antenna (such as a dipole).

In table 9.1.1-1 classification of the radiated Tx requirements is provided with brief justification.

Table 9.1.1-1: Classification of radiated Tx requirements

| Tx requirement | Description | Classification |
| --- | --- | --- |
| Radiated transmit power | The minimum requirements for radiated transmit power, are placed on one or more manufacturer declared beams over a declared OTA peak direction set. OTA requirements for NR BS output power are defined for directional EIRP requirements as radiated transmit power requirements. This requirement is based on the Rel-13 AAS BS requirement for the EIRP accuracy. | Directional |
| OTA BS output power | TRP metric is used for NR BS output power limit requirement. | TRP |
| OTA output power dynamics | OTA output power dynamics consists of the Total power dynamic range, as well as the RE power control dynamic range requirements. For E-UTRA specification, the RE power control dynamic range requirement has no specific test and it is tested together with the EVM. Furthermore verification of the output power dynamics is not impacted by the spatial aspects around the DUT. Therefore the OTA output power dynamics requirements are considered as directional requirements. | Directional |
| OTA transmit ON/OFF power | For AAS BS, the OTA Transmit OFF power was agreed to be a co-location requirement, defined at the co-location reference antenna conductive output side, subject to scaling. For NR BS, the same approach is reused. | Co-location |
| OTA transient period | Same as OTA transmit OFF power, for AAS BS, the OTA transient period was agreed to be a co-location requirement defined at the co-location reference antenna conductive output side, subject to scaling. For NR BS, the same approach is reused. | Co-location |
| OTA transmitted signal quality | EVM: The range of directions where the EVM requirement must be met is declared by the manufacturer as OTA coverage range, while the requirement itself is considered directional. Frequency error: The frequency error is coherent and will have a ‘flat’ response in the spatial domain, i.e. OTA frequency error will not depend on the selection of the measurement point within beam’s compliance directions set. Therefore single directional requirement can be applied. TAE: In terms of testing effort it is beneficial, to coordinate testing of OTA TAE with testing of other transmitter parameters such as OTA frequency error and radiated transmit power. | Directional |
| OTA occupied bandwidth | For occupied bandwidth, the beam characteristics are not important. The requirement should however cover the fact that all transmitter is active and the system is operating at the maximum declared rated total radiated power. Occupied bandwidth is specified as a directional requirement valid over the OTA coverage range. | Directional |
| OTA ACLR | ACLR requirement is the ratio of two TRP measures: the total radiated filtered mean power centred on the assigned channel frequency to the total radiated filtered mean power centred on an adjacent channel frequency. | TRP |
| OTA operating band unwanted emission | The OBUE unwanted emissions requirement in the OTA domain must capture all emissions around the DUT by application of the TRP metric. | TRP |
| OTA transmitter spurious emission | Similar to other Unwanted emissions requirements, the metric used to capture transmitter spurious emissions OTA is TRP. | TRP |
| OTA transmitter intermodulation | OTA transmitter intermodulation requirement relies on Unwanted emission requirements (i.e. operating band unwanted emission, transmitter spurious emission, and ACLR; all defined as TRP) in the presence of a wanted signal and an interfering signal. No requirement for BS type O-2 is defined. | Co-location |

Directional requirements are to be met over one of two defined directions sets, with each direction set being declared:

-	the OTA coverage range: range of directions over which directional requirements associated with BS-UE communication are intended such as modulation quality, TAE and frequency error. It can be regarded as the range of directions which define the cell coverage. There is only one OTA coverage range per BS.

-	the OTA peak directions set: intended for directional requirements which are intended for the centre of the beam for example EIRP accuracy. The OTA peak directions set must always be within the OTA coverage range. There may be more than one OTA peak directions set declared, the declarations cover the range of directions which a beam may be steered. As the BS may generate more than one type of beam with different beam widths and different steering capabilities the declaration allows for multiple OTA peak direction sets to be declared. The minimum set of declarations covers the beams with the narrowest and the widest beam widths.

--------------End of change-------------