3GPP TSG-RAN4 Meeting #90 	R4-1900716

# Athens, Greece, 25 Feb. – 1 March, 2019

| CR-Form-v11.2 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.4.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME | X | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | Draft CR for Intra-frequency SS-RSRQ Accuarcy Test Cases for EN-DC FR2 (section A.5.7.2) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | LG Electronics |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT |  |  |  |  |  |  | Date: |  | 2019-02-15 |
| Category: | F |  |  |  |  |  |  | Release: |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | BWP configuration and AoA setup were defined in RRM test configuration section. There are some typo for control channel RMC. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Update BWP configuration and AoA setup, and revise control channel RMC |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | Wrong test configuration could be used. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | A.5.7.2 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  |  |  |  |
| affected: |  | X |  | Test specifications |  |  |  |  |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  |  |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

----- << Start of Change 1>> -----

A.5.7.2	SS-RSRQ

A.5.7.2.1	Intra-frequency case

A.5.7.2.1.1	Test Purpose and Environment

The purpose of this test is to verify that the SS-RSRQ measurement accuracy is within the specified limits. This test will verify the requirements in Clause 10.1.8.1.1.

A.5.7.2.1.2	Test Parameters

In this test case all cells are on the same carrier frequency. Supported test configurations are shown in Table A.5.7.2.1.2-1. The absolute accuracy of SS-RSRQ intra-frequency measurement is test by using the parameters in Table A.5.7.2.1.2-2 and Table A.5.7.2.1.2-3. The configuration of cell 1 (E-UTRA PCell) is specified in section A.3.7.2.1. In all test cases, Cell 2 is the PSCell and Cell 3 is the target cell.

Table A.5.7.2.1.2-1: SS-RSRQ Intra frequency SS-RSRQ supported test configurations

| Configuration | Description |
| --- | --- |
| 1 | FDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| 2 | TDD LTE PCell, Cell 2&3 120 kHz SSB SCS, 100MHz bandwidth, TDD duplex mode |
| Note: The UE is only required to pass in one of the supported test configurations |  |

Table A.5.7.2.1.2-2: SS-RSRQ Intra frequency test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 2 | Cell 3 | Cell 2 | Cell 3 | Cell 2 | Cell 3 |
| SSB ARFCN |  |  | Freq1 |  | Freq1 |  | Freq1 |  |
| Duplex mode |  |  | TDD |  | TDD |  | TDD |  |
| TDD configuration |  |  | TDDConf.3.1 |  | TDDConf.3.1 |  | TDDConf.3.1 |  |
| BWchannel |  | MHz | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  | 100: NRB,c = 66 |  |
| BWP configuration | Initial DL BWP |  | DLBWP.0.1 |  |  |  |  |  |
|  | Dedicated DL BWP |  | DLBWP.1.1 |  |  |  |  |  |
|  | Initial UL BWP |  | ULBWP.0.1 |  |  |  |  |  |
|  | Dedicated UL BWP |  | ULBWP.1.1 |  |  |  |  |  |
| PDSCH Reference measurement channel |  |  | SR.3.1 TDD |  | SR.3.1 TDD |  | SR.3.1 TDD |  |
| RMSI CORESET Reference Channel |  |  | CR.3.1 TDD | - | CR.3.1 TDD | - | CR.3.1 TDD | - |
| Control channel RMC |  |  | CCR.3.1 TDD | - | CCR.3.1 TDD | - | CCR.3.1 TDD | - |
| OCNG Patterns |  |  | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 | OP.1 |
| SMTC configuration |  |  | SMTC.1 |  |  |  |  |  |
| SSB configuration |  |  | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 | SSB.1 FR2 |
| PDSCH/PDCCH subcarrier spacing |  | kHz | 120 | 120 | 120 | 120 | 120 | 120 |
| SS-RSSI-Measurement |  |  | Not Applicable |  |  |  |  |  |
| EPRE ratio of PSS to SSS |  | dB | 0 | 0 | 0 | 0 | 0 | 0 |
| EPRE ratio of PBCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PBCH to PBCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDCCH to PDCCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH_DMRS to SSS |  |  |  |  |  |  |  |  |
| EPRE ratio of PDSCH to PDSCH_DMRS |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG DMRS to SSSNote 1 |  |  |  |  |  |  |  |  |
| EPRE ratio of OCNG to OCNG DMRS Note 1 |  |  |  |  |  |  |  |  |
|  |  | dB | 3 | 3 | -2.9 | -2.9 | -4 | -4 |
| Note 1: OCNG shall be used such that both cells are fully allocated and a constant total transmitted power spectral density is achieved for all OFDM symbols. Note 2: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 3: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 4: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. |  |  |  |  |  |  |  |  |

Table A.5.7.2.1.2-3: SS-RSRQ Intra frequency OTA related test parameters

| Parameter |  | Unit | Test 1 |  | Test 2 |  | Test 3 |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  | Cell 1 | Cell 2 | Cell 1 | Cell 2 | Cell 1 | Cell 2 |
| Angle of arrival configuration |  |  | According to section A.3.15.1 |  | According to section A.3.15.1 |  | According to section A.3.15.1 |  |
| Note1 | NR_TDD_FR2_A | dBm/15kHzNote4 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| Note1 | NR_TDD_FR2_A | dBm/SCSNote3 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| SS-RSRPNote2 | NR_TDD_FR2_A | dBm/SCS Note4 | TBD | TBD | TBD | TBD | TBD | TBD |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD | TBD |
| SS-RSRQ Note2 | NR_TDD_FR2_A | dB | TBD | TBD | TBD | TBD | TBD | TBD |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_C |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_D |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_E |  |  |  |  |  | TBD | TBD |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD | TBD |
|  |  | dB | TBD | TBD | TBD | TBD | TBD | TBD |
| IoNote2 | NR_TDD_FR2_A | dBm/95.04 MHz Note4 | TBD |  | TBD |  | TBD |  |
|  | NR_TDD_FR2_B |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_F |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_G |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_T |  |  |  |  |  | TBD |  |
|  | NR_TDD_FR2_Y |  |  |  |  |  | TBD |  |
| Note 1: Interference from other cells and noise sources not specified in the test is assumed to be constant over subcarriers and time and shall be modelled as AWGN of appropriate power for to be fulfilled. Note 2: SS-RSRQ, SS-RSRP, and Io levels have been derived from other parameters for information purposes. They are not settable parameters themselves. Note 3: SS-RSRQ and SS-RSRP minimum requirements are specified assuming independent interference and noise at each receiver antenna port. Note 4: Equivalent power received by an antenna with 0dBi gain at the centre of the quiet zone Note 5: As observed with 0dBi gain antenna at the centre of the quiet zone Note 6: NR operating band groups are as defined in Section 3.5.2. |  |  |  |  |  |  |  |  |

A.5.7.2.1.3	Test Requirements

The SS-RSRQ measurement accuracy shall fulfil the requirements in section 10.1.8.1.1.

----- << End of Change 1>> -----