3GPP TSG-RAN WG4 Meeting #90bis	R4-1902982

Xian, China, 8th– 12th April, 2019

| CR-Form-v11.4 |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
| CHANGE REQUEST |  |  |  |  |  |  |  |  |
|  | 38.133 | CR |  | rev | - | Current version: | 15.5.0 |  |
| For HELP on using this form: comprehensive instructions can be found at http://www.3gpp.org/Change-Requests. |  |  |  |  |  |  |  |  |

| Proposed change affects: | UICC apps |  | ME |  | Radio Access Network |  | Core Network |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |

| Title: | CR on BWP switching requirement in TS38.133 (Section 8.6) |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Source to WG: | Intel Corporation |  |  |  |  |  |  |  |  |  |
| Source to TSG: | R4 |  |  |  |  |  |  |  |  |  |
| Work item code: | NR_newRAT-Core |  |  |  |  |  | Date: |  |  | 2019-04-01 |
| Category: | F |  |  |  |  |  | Release: |  |  | Rel-15 |
|  | Use one of the following categories: F (correction) A (mirror corresponding to a change in an earlier release) B (addition of feature), C (functional modification of feature) D (editorial modification) Detailed explanations of the above categories can be found in 3GPP TR 21.900. |  |  |  |  |  |  |  | Use one of the following releases: Rel-8 (Release 8) Rel-9 (Release 9) Rel-10 (Release 10) Rel-11 (Release 11) Rel-12 (Release 12) Rel-13 (Release 13) Rel-14 (Release 14) Rel-15 (Release 15) Rel-16 (Release 16) |  |
| Reason for change: |  | The RRC based BWP switching delay requirement is not specified in TS 38.133. |  |  |  |  |  |  |  |  |
| Summary of change: |  | Modifications are made in section 8.6.3 to specify the delay requirement for RRC based BWP switching and MAC based BWP switching. |  |  |  |  |  |  |  |  |
| Consequences if not approved: |  | The open issues decript in Editor’s note in section 8.6.3 are unsloved. |  |  |  |  |  |  |  |  |
| Clauses affected: |  | Section 8.6.3 |  |  |  |  |  |  |  |  |
|  |  | Y | N |  |  |  |  |  |  |  |
| Other specs |  |  | X | Other core specifications |  |  |  | TS/TR ... CR ... |  |  |
| affected: |  |  | X | Test specifications |  |  |  | TS/TR ... CR ... |  |  |
| (show related CRs) |  |  | X | O&M Specifications |  |  |  | TS/TR ... CR ... |  |  |
| Other comments: |  |  |  |  |  |  |  |  |  |  |

## << Start of Change >>

### 8.6.3	RRC based BWP switch delay

For RRC-based BWP switch, after the UE receives BWP switching request, UE shall be able to receive PDSCH (for DL active BWP switch) or transmit PUSCH (for UL active BWP switch) on the new BWP on the serving cell on which BWP switch on the first DL or UL slot right after the beginning of DLslot n+ TRRCprocessingDelay + TBWPswitchDelayRRC, where DL slot n is the last slot containing the RRC command, and TRRCprocessingDelay is the length of the RRC procedure delay in slots defined in clause 12 in TS 38.331 [2], and TBWPswitchDelayRRC is the BWP switching delay for RRC based BWP switch.

TBWPswitchDelayRRC is defined as the values for

- Scenario 1 in Table 8.6.3-1, if UE switches between the BWPs in disjoint channel bandwidths or in partially overlapping channel bandwidths;

- Scenario 2 in Table 8.6.3-1, if UE switches between the BWPs in fully overlapping channel bandwidths or in the two channel bandwidths where one is fully contained in the other.

Table 8.6.3-1: RRC based BWP swich delay

|  | NR Slot length (ms) | RRC based BWP switch delay TBWPswitchDelayRRC (slots) |  |
| --- | --- | --- | --- |
|  |  | Scenario 1 | Scenario 2 |
| 0 | 1 | 4+2*SMTC | 4 |
| 1 | 0.5 | 8+2*SMTC | 8 |
| 2 | 0.25 | 16+2*SMTC | 16 |
| 3 | 0.125 | 32+2*SMTC | 32 |
| Note: If the BWP switch involves changing of SCS, the BWP switch delay is determined by the larger one between the SCS before BWP switch and the SCS after BWP switch. |  |  |  |

## << End of Change >>