3GPP TSG-RAN WG4 Meeting #90-bis	R4-1904443

Xi’an, China, 8th to 12th April 2019

Source:	Ericsson, Telstra

Title:	TP for TR 37.716-21-11: 1A-3A_n5A, 1A-3C_n5A.

Agenda item:			9.4.2

Document for:	Approval

# 1. Introduction

Rel-16 LTE-NR DC combinations LTE 2 band + NR 1 band combination are defined in the WID [1]. This contribution is a text proposal for TR 37.716-21-11 to add 1A-3A_n5A and 1A-3C_n5A.

# 2. Text Proposal

# ---Start of changes---

## 5.1.x	DC_1A-3A_n5A
DC_1A-3C_n5A

### 5.1.x.1	Operating bands for EN-DC

Table 5.2B.5.3-1: Band combinations EN-DC (three bands)

| EN-DC band | E-UTRA CA band | NR band | Single UL allowed |
| --- | --- | --- | --- |
| DC_1-3_n5 | CA_1-3 | n5 | DC_3_n5 |

### 5.1.x.2	Configuration for DC

Table 5.5B.5.3-1: Inter-band EN-DC configurations (three bands)

| EN-DC configuration | Uplink EN-DC configuration (NOTE 1) | E-UTRA CA configuration | NR band |
| --- | --- | --- | --- |
| DC_1A-3A_n5A DC_1A-3C_n5A | DC_1A_n5A DC_3A_n5A DC_3C_n5A | CA_1A-3A CA_1A-3C | n5 |

#### 5.1.x.3	Co-existence Studies

Co-existence analysis from DC_3_n5 shows that there is no impact from DC_3_n5 UL to Band 1 DL.

Co-existence analysis from DC_1_n5 shows that there is no impact from DC_1_n5 UL to Band 3 DL.

### 5.1.x.4		∆TIB and ∆RIB values

For DC_1-3_n5, the TIB,c and RIB,c values are reused from the LTE combination CA_1-3-5, and are given in the tables below.

Table 5.1.x.4-1: ΔTIB,c

| Inter-band DC Configuration | E-UTRA and NR Band | ΔTIB,c [dB] |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0.3 |
|  | 3 | 0.3 |
|  | n5 | 0.3 |

Table 5.1.x.4-2: ΔRIB

| Inter-band DC Configuration | E-UTRA and NR Band | ΔRIB [dB] |
| --- | --- | --- |
| DC_1-3_n5 | 1 | 0 |
|  | 3 | 0 |
|  | n5 | 0 |

### 5.1.x.5	REFSENS requirements

MSD does not need to be defined.

---End of changes---

# Reference

[1]	RP-190379, “Revised WID on Dual Connectivity (EN-DC) of 2 bands LTE inter-band CA (2DL/1UL) and 1 NR band (1DL/1UL)”, Huawei