3GPP TSG-RAN4 Meeting #90BIS 	R4-1904684

# Xian, China, 8 – 12 April, 2019

Title:	Reply LS on UE behavior on reception of channels or RS in the same OFDM symbol

Response to:	R1-1809890

Release:	Rel-15

Work Item:	NR_newRAT-Core

Source:	RAN WG4

To:	RAN WG1

Cc:

Contact Person:

Name:	Jin-Yup Hwang

E-mail Address:	jinyup.hwang@lge.com

1. Overall Description:

RAN4 has discussed RAN1 LS on UE behavior on reception of channels or RS in the same OFDM symbol for FR2. Since RAN4 has not defined core requirements for CSI-RS based L3 measurement, this LS is not applicable to CSI-RS based L3 measurement in Rel-15.

RAN4 would like to inform RAN1 the conclusion in RAN4 as follows:

RAN4 kindly remind RAN1 that additional to the following answers regarding restrictions due to FR2 beam sweeping, when performing reception of channels/RS in group 1 with different subcarrier spacing than channels/RS group 2, the UE which does not support simultaneousRxDataSSB-DiffNumerology cannot receive both channels/RS group 1 and group 2 simultaneously.

- Answer to Q1

Case 1 & 4

  - If the RS in group 2 is QCL-Type D with RS in group 1 and the QCL association is known to UE, UE can receive both RS in group 1 and group 2 in the same OFDM symbol when UE is not expected to sweep its Rx beam for both two RSs in FR2. Otherwise, UE may not receive both RS in group 1 and group 2 in the same OFDM symbol.

    - For L3 SSB based measurement, UE is expected to perform Rx beam sweeping on all the predefined SSB symbols within the SMTC window duration.

    - For RLM-RS/BFD-RS/L1-RSRP measurement, UE is expected to perform Rx beam sweeping on the configured RS under the condition of N>1, which is defined in section 8.1, 8.5, and 9.5 of TS38.133.

Case 2 & 3

  - If the TCI of the PDCCH/PDSCH includes at least one of the CSI-RS resource IDs of the CSI-RS resource set and QCL-Type D of the CSI-RS with repetition “OFF” is configured and UE is not expected to sweep its Rx beam in FR2, UE can receive a PDCCH/PDSCH that is overlapping with one or more symbols configured with the CSI-RS resource(s) with repetition set to “OFF”. Otherwise, UE is not required to receive both PDCCH/PDSCH and the CSI-RS resource with repetition set to “OFF” in the same OFDM symbol.

    - For CSI-RS based L1-RSRP measurement, UE is expected to perform Rx beam sweeping on the configured RS under the condition of N>1, which is defined in section 9.5 of TS38.133.

- Answer to Q2

For rate matching issue by scheduling restriction, RAN4 has already agreed in RAN4 NR-AH#4 meeting as follows:

  - It is up to RAN1 whether or not to define specific behavior for rate matching around scheduling restricted symbols.

2. Actions:

To RAN1:

ACTION: RAN4 respectfully asks RAN1 to take the above RAN4 conclusion into consideration for their further work.

3. Date of Next TSG-RAN4 Meetings:

3GPP RAN4 #91 		13 - 17 May 2019, 		, US

3GPP RAN4 #92 		26 - 30 August 2019, 		, Slovenia